3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206486r06

Elbonia, August 17th – 26th, 2022	(revision of S2-220xxxx)

Source:	vivo

Title:	Interim conclusions on KI#6

Document for:	Approval

Agenda Item:	9.16

Work Item / Release:	FS_PIN / Rel-18

Abstract: this paper proposes evaluation update and interim conclusions on KI#6.

1. Introduction

This paper proposes evaluation update and interim conclusions on KI#6.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-88.

# * * * * First change * * * *

## 8.6	Conclusion on Key Issue #6

The following principles are concluded for Key Issue #6 "Policy and parameters provisioning for PIN":

2)	The policy and parameters provisioned to PEGC for PIN management may include following potential information:

a)	IP address allocation information for allocating IP address to PINE, in order to support communication within PIN.

b)	PIN connection parameters for a PIN, e.g., SSID, BT ID, password, etc.

c)	PIN RAT parameters for a PIN, e.g. whether the SSID is hidden or not when device is announcer, activated RAT types (e.g. WiFi, BT), etc.

d)	PIN discovery parameters for a PIN, e.g., open or restrict (e.g. WiFi for open – authorization by PEMC when accept a connection, BT for restrict - accept connection from the PINE in allowed list), etc.

e)	PIN access control parameters for a PIN, e.g., allowed/block list of MAC addresses, allowed/block SSIDs, allowed/block BT IDs, etc.

f)	PIN activation parameters for a PIN, to instruct the PEGC activating/deactivating D2D Routing Rule and D2N Routing Rule of the PIN, as well as optionally opening/closing non-3GPP access radio.

3)	The policy and parameters provisioned to PEGC for PIN communication may include following potential information:

a)	PDU Session mapping table for PINE’s traffic relay (URSP like information).

b)	non-3GPP delay budget between PEGC and PINE.

c)	D2D Routing Rule for a PIN to enable/disable traffic routing among PINEs and PEMCs via a PEGC without 5GS involved.

d)	D2N Routing Rule for a PIN to enable/disable traffic routing between PINE/PEMC and 5GS via a PEGC.

e)	The QoS flow mapping for PINE’s traffic relay is received via existing procedure from PCF,.

f) Non-3GPP QoS assistance information.

4)	The policy and parameters provisioned to UPF for PIN communication may include following potential information. The existing procedure used by the SMF to provide the UPF with, PDR, FAR, etc are applicable without modification.

a)	UL and DL packet filters for a PIN to enable/disable traffic routing between PINE/PEMC and external DN via 5GC.

b)	N2N Routing Rule for a PIN to enable/disable traffic routing within the PIN via 5GC.

c)	Framed Route Info for the PDU Session of a PEGC.

5)	PIN related policy and parameter(s) is able to be updated by an AF for PIN, if used, invoking an NEF API.

6)	The AF for PIN service, if used, may provide the necessary PIN related parameters to 5GC for differentiation and policy/QoS control of specific PINE traffic via 5GC.

7) 	The AF for PIN service, if used, may provide necessary PIN related parameter to PEGC (e.g. URSP like information) via 5GC.

Editor’s note: It is FFS whether the parameters will originate in the UDM/UDR, PCF, or a new network/application function.

Editor’s note: What procedure (i.e., Registration, UE Configuration Update, or a new procedure) is used to send the parameters to the UE is FFS.

Editor’s note: whether needs AF or 5GC NF for PIN is dependent on final conclusion of KI#1

Editor’s note: Whether the above parameters are needed and whether they includes some new IEs is FFS.

# * * * * End of changes * * * *