3GPP TSG-SA WG2 Meeting #152e	S2-2206898r04

# Aug 17 – 26, 2022

Title:	[draft]LS on XR and Media Services

Response to:	LS on UE Power Saving for XR and Media Services (R1-2205531/S2-2205421)

Release:	Rel-18

Work Item:	FS_XRM, FS_NR_XR_enh

Source:	3GPP TSG SA WG2

To:	RAN WG1, RAN WG2

Cc:

Contact Person:

Name:	Xiaowan Ke

Tel. Number:

E-mail Address:	xiaowan(dot)ke(at)vivo(dot)com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA2 would like to thank RAN1’s LS reply (R1-2205531/S2-2205421) on UE power saving for XR and media services. So far, SA2 starts to evaluate and conclude solutions for key issues for the study of XR and Media Services and would like to coordinate with RAN WG about the following aspects

- KI#3 studies what information is useful and can be exposed by 5GS to the server. Wherein, SA2 is considering RAN node congestion and per QoS flow congestion which are exposed by RAN. SA2 understands the granularity of RAN scheduling is DRB/LCH based and a DRB can be used for multiple QoS flows, hence SA2 would like to request RAN2 to feedback the following questions:

  - Q1: Whether it is feasible for RAN to detect the per QoS flow congestion and per RAN node congestion.

  - Q2: If the answer to Q1 is yes, please clarify how to detect those congestions; otherwise, please also feedback the reason.

  - Q3: Please RAN also shares the understanding of the definition per QoS flow congestion/per RAN node congestion, and the other consideration, if any.

- KI#4&5 is about the PDU Set handling, mainly including PDU Set identification and PDU Set level QoS. SA2 will focus on the how to support the PDU Set handling for downlink XR traffic. The uplink is closely related to the interaction between the UE and RAN, hence SA2 would like to request RAN to decide whether and how to support the PDU Set handling for uplink XR traffic. SA2 will align with RAN’s progress and decision for uplink, if any.

- KI#8, i.e. power saving and CDRX enhancement, SA2 is working the conclusion. Before that,

- SA2 would like to clarifies he main usage of following information raised in (R1-2205531/S2-2205421) is about PDU Set QoS and PDU Set integrated handing: “PDU set level QoS parameters including priority and delay budget of a PDU set, PDU set identity and relationship information among PDUs within the same PDU set, PDU set end indication or indication of the last PDU in a PDU set, PDU set size (number of bits) or number of PDUs in a PDU set.” Whether the above information is concluded is left to PDU Set QoS and PDU Set integrated handing. If they were concluded, whether and how to use them into power saving and CDRX enhancement will be left to RAN WG’s decision.

2. Actions:

To RAN WG1, RAN WG2:

ACTION: 	SA2 kindly asks RAN1, RAN2 to take the above information into account and reply.

3. Date of Next TSG SA WG2 Meetings:

SA WG2 Meeting #153     	10 – 14 October August 2022			E-Meeting

SA WG2 Meeting #154      	14 – 18 November 2022			Canada, CA