SA WG2 Meeting #S2-152E	S2-2206906r06

17 – 26 August , 2022, E-meeting

Source:	Xiaomi

Title:	conclusion for Key Issue#3

Document for:	Approval

Agenda Item:	9.16

Work Item / Release:	5G_PIN /Rel-18

Abstract of the contribution: This paper proposes the conclusion for key Issue #3.  This paper merged S2-2206015, S2-2206483, S2-2206906 and S2-2205743

*** Start of changes ***

8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.3	Conclusion on Key Issue #3

The following principles are concluded for Key Issue #3 "Management of PIN and PIN Elements":

1)	PEMC and  are responsible for PIN Element management (e.g., add/remove the PINE/PEGCs, etc.) and PIN management (e.g. create/modify/delete/activate/deactivate a PIN, etc.).

Editor’s note: whether AF will be involved in PIN/PINE management depends on the conclusion of KI#1 (whether the architecture of PIN contains the PIN AF)

3)	 PEMC, UDR store information for PIN management and PIN Element management (e.g., PIN ID/name, PIN Element list, profile of PIN Element, PIN configuration). UDR stores the service specific information that supports PIN communication in 5GS.

4)	UE’s subscription data should contain PIN related information when it needs to act as PEMC, or PEGC, including:

- PIN ID, which is provided by UDM

- PINE type, which provided when PEMC subscribes to UDM.

Editor’s note: The other possible PIN related information will be decided based on the final conclusion.

Editor’s NOTE: The other possible  PIN related information will be decided on the final conclusion..

5)	AF may provide necessary service specific information to 5GC to support the PIN communication.

NOTE 2: The service specific information will be specified in normative work.

9)	A PEMC or a PEGC shall be a member of a trust member group, which includes at least one PEMC and one or more PEGCs.

10)	To support management of multiple PEMCs for a PIN, a trust member group may include multiple PEMCs.

11)	In order to make sure PEMCs and PEGCs using same AF, or a PEMC using same AF after recovery, the AF registers itself into UDR via UDM or NEF for a trust member group.

Editor’s note: more details of justification of 9-11 are needed.

*** End of changes ***