SA WG2 Meeting #S2-152E	S2-2206906r11

17 – 26 August , 2022, E-meeting

Source:	Xiaomi

Title:	conclusion for Key Issue#3

Document for:	Approval

Agenda Item:	9.16

Work Item / Release:	5G_PIN /Rel-18

Abstract of the contribution: This paper proposes the conclusion for key Issue #3.  This paper merged S2-2206015, S2-2206483, S2-2206906 and S2-2205743

*** Start of changes ***

# 8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.3	Conclusion on Key Issue #3

The following principles are concluded for Key Issue #3 "Management of PIN and PIN Elements":

1)	PEMC is responsible for PIN Element management (e.g., add/remove the PINE/PEGCs, etc.) and PIN management (e.g. create/modify/delete/activate/deactivate a PIN, etc.).

Editor’s note: whether AF will be involved in PIN/PINE management depends on the conclusion of KI#1 (whether the architecture of PIN contains the PIN AF)

4)	UE’s subscription data should contain  authorization information when it needs to act as PEGC

.

Editor’s note: If there is other possible  PIN related information needed will be decided on the final conclusion..

5)	AF may provide necessary service specific information to 5GC to support the PIN communication via 5GC.

NOTE 2: The service specific information will be specified in normative work.

9)	When PEMC is different from PEGC and communication via 5GC is needed, the existing functionalities in 5GS can be applied if available.

Editor’s note: whether AF information needs to be stored in UDR for the case, e.g., when UE change PEMC UE, or user’s PEMC UE is recovered, etc., is FFS.

*** End of changes ***