SA WG2 Meeting #S2-152E	S2-2207489

17 – 26 August, 2022, Electronic meeting	revision of S2-2205943r10

Source:	Nokia, Nokia Shanghai Bell, Ericsson, LG Electronics, Qualcomm, ZTE

Title:	evaluation and interim conclusions for key issue 2

Document for:	Approval

Agenda Item:	9.18

Work Item / Release:	FS_MBS2 / Rel-18

Abstract of the contribution: This paper proposes evaluation and interim conclusions for key issue 2

# 2		Proposal

It is proposed to include the following changes in TR 23.700-47.

*** 1st Change (all new)***

# 8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.X	Key issue 2

### 8.X.1	Interim Conclusions

Editor's note:	RAN WGs will determine the feasibility of radio resource utilization optimization.

For conclusions, the following aspects will be considered:

-	For solutions where the broadcast MBS sessions for different PLMNs are established towards a NG-RAN node, the NG-RAN node shall be able to identify the same MBS service and avoid multiple deliveries over radio.

-	A solution compatible with Rel-17 UEs is preferred.

The following interim conclusions will be taken into account:

- It should be possible not to establish all the shared delivery tunnels to the same NG RAN from different PLMNs for the same MBS service.

- The solution should support scenario where all RAN nodes are shared by PLMNs and the scenario where only part of the RAN nodes are shared by PLMNs.