SA WG2 Meeting #153E eMeeting	S2-2208777

Elbonia, Oct. 10 – 14, 2022 	(revision of S2-20xxxxx)

Source:	NTT DOCOMO, Oppo, Huawei, Oracle, Toyota

Title:	TR 23.700-80: KI#5 – Conclusion

Document for:	Approval

Agenda Item:	9.21

Work Item / Release:	FS_AIMLsys / Rel-18

Abstract of the contribution: This contribution proposes a KI#5 conclusion.

# Background

Based on the evaluation in clause 7.5, the following conclusions are proposed for KI#5.

# Proposals

***** Start of Change *****

## 8.5	Key Issue #5: 5GC Enhancements to enable Application AI/ML Traffic Transport

AI/ML data transfer window negotiation mechanism: It is agreed to consider solution #10 as a basis for KI#5, with the following principles:

- Principle 1: A new Application AI/ML Data Transfer (AAMDT) policy is adopted to support the planned and event driven application AI/ML traffic transport. The AAMDT policy includes a list of recommended time window(s) with predicted QoS parameters (e.g., PDB, PER, bit rate) per UE per time window.

- Principle 2: The AF includes the latency and reliability requirements for the planned and event driven Application AI/ML traffic transport, in the AAMDT policy negotiation request.

- Principle 3: The PCF uses the extended Network Performance Analytics of NWDAF to derive the candidate time window(s) that may fulfil the latency and reliability requirements requested from AF. The PCF generates the list of AAMDT policies and feedback it to the AF. The QoS parameters in the AAMDT policy can be determined by PCF based on the QoS requirements from the AF.

- Principle 4: The Network Performance Analytics is enhanced to support the additional analytics outputs for the following parameters:

- 	Overall Average and Maximum Packet Delay for UL/DL per UE per time window;

-	Overall Average and Maximum Packet Loss Rate for UL/DL per UE per time window;

-	Overall Average, Minimum and Maximum Traffic rate for UL/DL per UE per time window.

- Principle 5: The AF selects one of the AAMDT policies provided by PCF which may most fit for its requirement, and indicate it to the PCF.

- Principle 6: It is agreed to support data transfer time window negotiation between the AF and the 5GS for the Application AI/ML traffic.

  - The Application AI/ML traffic could be non GBR type or GBR type.

- Principle 7: The network should guarantee the minimum required QoS (i.e., Flock QoS) for the selected UEs to ensure the adequate performance e.g., for the FL operation

Application AI/ML Traffic Transport:

- AF requests to influence the traffic routing as defined by clause 4.3.6 of TS 23.502 [4].

  - Enhance the existing AF Influence Traffic Routing mechanism to leverage and extend the User Experience Analytics and DN Performance Analytic to support the traffic routing decision.

- When the Application AI/ML begins e.g., to transport intermediate data, local training data, inference results, or model performance data, through 5GS, the AF can leverage the existing service Nnef_AFsessionWithQoS_Create request message as described in clause 4.15.6.6 of TS 23.502 [4], including the specific QoS parameters for the AF session.

NOTE 1: The details scenarios where “AI/ML data transfer window negotiation mechanism” and “Application AI/ML Traffic Transport mechanism” are applicable are described in the normative. For instance, the AI/ML data transfer window negotiation mechanism can be applicable for bulk data transfer, and the Application AI/ML Traffic Transport mechanism can be applicable when a small amount of UEs are involved.

Charging for Application AIML traffic transport:

- Based on operator policy, charging of different Application AI/ML traffic needs to be perform, however, do not expect any normative work on this aspect in the SA2 specifications.

NOTE 2: An alignment can be required (if any) in the normative based on the outcome of SA WG5.

***** End of Change *****