3GPP TSG-SA WG2 Meeting #154AH-E		S2-2300589r04

Elbonia, Jan 16th – 20th, 2023	(revison of S2-221xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0164 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On Clarifying the Scenario Considering the Power Saving Mechanism |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Nokia, Nokia Shanghai-Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | KI#5 had been concluded in SA2#154 meeting. It is concluded that Sol#14 is used as the basis for normative work with some clarifications as mentioned in TR 23.700-47 clause 8.5. Therefore, it is possible to specify the conclusion of KI#5 in TS 23.247. To be specific, a clause with including the general description can be added, it lists the general principles for 5GS providing MBS data to the UEs with requiring the power saving. Other potential enhancement shall be triggered by the RAN WGs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the descriptions for coexistence with existing power saving mechanisms for capability-limited UEs in TS 23.247. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing the functionalities for KI#5, which has been concluded in TR 23.700-47. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Start of the change * * * *

## 6.X	Support of MBS reception for UEs using power saving mechanisms

MBS provide means to deliver data, triggering payload etc. over MBS session to multiple UEs at the same time. However, for UEs using power saving functions (e.g., MICO (Mobile Initiated Connection Only) mode with Active Time, DRX (Discontinuous Reception) or extended idle mode eDRX (Extended Discontinuous Reception) as defined in clause 5.31.7 of TS 23.501 [5], the UEs are usually unreachable for long periods of time. Moreover, different UEs are likely to be reachable at different times.

If a UE becomes unreachable for unicast data transfer due to either MICO mode with Active Time or extended idle mode DRX, the UE may still perform MBS specific procedures, e.g. activation/deactivation of the MBS service, MBS data transfer reception, reception of service announcement (if needed).

To receive MBS data, those UEs need to wake up at coordinated times when the MBS data is being transmitted.

The UE is informed via the service announcement about a start time and/or a sequence of scheduled activation times (e.g. a first time and a periodicity) of the MBS session when the AF may activate the MBS session and transmit MBS data, as described in clause 6.11.

The AF may send data starting either at the start time or at any scheduled activation times. If the AF sends data using an MBS multicast session at a scheduled activation time, it shall previously activate the MBS session at that scheduled activation time.

NOTE 1:	If the UE has not previously joined the MBS multicast session, at the possible scheduled activation time it performs MBS join procedure as currently defined in clause 7.2.1.3. Whether the UE performs MBS join procedure in advance and stays "joined" or every time at activation time is left up to UE implementation.

Editor's Note: The details for multicast MBS when the UE is in CM-CONNECTED/ RRC_INACTIVE state with eDRX > 10.24s will be confirmed with RAN WGs.

# * * * * End of changes * * * *