3GPP TSG-WG SA2 Meeting #154 	S2-2210305

Toulouse, France, November 14 – 18, 2022	(revision of S2-221xxxx)

Source:	Nokia, Nokia Shanghai Bell

Title:	KI#1: Update of the Conclusion

Document for:	Approval

Agenda Item:	9.19.1

Work Item / Release:	FS_XRM / Rel-18

Abstract: This document updates KI#1 conclusion agreed in the previous meeting.

# 1. Introduction/Discussion

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-60.

# * * * * First change * * * *

## 8.X	Conclusions for Key issue#1

The following aspects are concluded as principles for the normative work:

For Key issue#1, single UE case.

The following aspects are concluded as principles for the normative work:

-	Those data streams that are closely related and require strong application coordination are transmitted in a single PDU session by a single UE. However, those data streams that contribute to the immersive experience, but may still be valid stand-alone, may be transmitted over separate PDU sessions from multiple UEs. In order to ensure that the UE selects the correct DNN/S-NSSAI combination for the XRM traffic, the existing URSP Rule evaluation framework can be reused. A traffic descriptor (e.g. an FQDN) for the XRM session will be used during URSP rule.

-	The procedure for AF session setup with required QoS, is reused for XRM applications (untrusted AFs) interacting with NEF. However, current Nnef_AFsessionWithQoS service shall be extended to allow the AF to provide information for multiple medias.

-	Normative impact to AF and NEF/PCF: extend the existing Nnef_AFsessionWithQoS service to allow the AF to provide, at the same time, service requirements, alternative service requirements, a common ID and additional common requirements for multiple IP data flows associated to a multi-modal (XRM) application.

-	The common requirements include the following:

-	Admission control requirements:

-	Admit the flows only if all of the flows can be admitted or admit the flows only if at least all the flows labelled as critical can be admitted

-	QoS fulfilment and retention requirements:

-	QoS is considered fulfilled only if the service requirements of all the flows can be fulfilled or QoS is considered fulfilled only if the service requirements of at least all the flows labelled as critical can be fulfilled.

-	Synchronization requirements:

-	An indication that the flows are part of a common synchronization group and optionally the maximum tolerated 5GS delay difference across the flows

-	PCF is responsible for the following tasks:

-	PCF performs the flow authorization.

-	PCF provisions QoS information considering the requirements provided by the AF for all data flows associated to a multi-modal (XRM) application. For this the PCF takes into account both the service requirements of the individual flows and the additional common requirements related to:

-	Admission control

-	QoS fulfilment and retention

-	Synchronization

-	PCF enforces the policy for the use of Alternative QoS parameters.

-	These policies above are enforced only according to the AF provided explicit requirements.

PCF provides the policy rules, common ID and additional common requirements to SMF. SMF provides the common ID and additional common requirements to NG-RAN as part of the QoS flow establishment or modification procedure.

NG-RAN is able to take the additional common requirements it receives into account for its admission control, QoS fulfilment and retention decisions (e.g., related to handovers) within the boundaries of the existing procedures.

# * * * * End of changes * * * *