3GPP TSG-SA2 Meeting #154	S2-2210318

# Toulouse, FR, Nov 14 – Nov 18, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0562 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New analytic on PFD determination |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Proposal to update TS 23.288 to add a new analytic as concluded in TR 23.700-81 KI #2 NWDAF-assisted application detection, where the conclusions state the following: It is concluded to focus on the use case of PFD updates for known applications in Solution#9, which is selected as baseline for the normative work. The known application means an application for which the application ID is already known by the 5GC and can be referenced within PCC rules and for which PFD information (may not be the latest) is already available. It is concluded that the NEF(PFDF) as a consumer can request NWDAF to get a new Analytics ID on PFD determination (as the output is a list of new/updated PFDs), following the detail described in Solution#9. Details of UPF data Collection by NWDAF are being specified in TS 23.502 as in CRs approved in SA2 #153e based on TR 23.700-62 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added Clause 6.15 to define a new analytic on PFD determination |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conclusions agreed in TR 23.700-81 (eNAPh3) KI #2 cannot be fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1x (all new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is part of outcome of SI FS_eNA_Ph3 and the WI code for the work is not assigned yet & will replace WI code DUMMY when available |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** Start of Changes all new ***************

### 6.1x PFD Determination Analytics

#### 6.1x.1 General

To assist on PFD determination for known application identifiers, an NWDAF may perform data analytics on existing PFD information and user plane traffic and provide analytics results in the form of new/updated PFDs to analytic consumer in the 5GC.

An NWDAF supporting PFD determination Analytics collects current PFD information in use (including Application ID and the corresponding IP 3-tuple, URL and/or Domain name information) from the NEF (PFDF), and user plane traffic information from UPF.

The consumer of these analytics may indicate in the request:

-	Analytics ID = "PFD determination".

-	Target of Analytics Reporting: any UE.

-	Application ID list;

-	Analytics Filter Information optionally including:

-	S-NSSAI;

-	DNN;

-	An Analytics target period indicates the time period over which the statistics or predictions are requested.

-	Optionally, maximum number of objects. This refers to the analytic output (i.e. maximum number of new/updated PFDs).

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

#### 6.1x.2 Input Data

The NWDAF supporting analytics on PFD determination shall be able to collect information from the NEF (PFDF) and user plane data from UPF. The detailed information collected by the NWDAF is defined in the Table 6.1x.2-1.

Table 6.1x.2-1: Input data from 5GC related to PFD determination

| Information | Source | Description |
| --- | --- | --- |
| SUPI | UPF | UE ID for the UE that uses the application |
| S-NSSAI | UPF | Identifies the Network Slice for which analytics information is provided. |
| DNN | UPF | Identifies the data network name (e.g. internet) for which analytics information is provided. |
| Application ID | UPF | Identifies the application |
| IP Flow Information (1..max) | UPF | Per IP flow related information for the application |
| > IP 5-tuple | UPF | Identifies a service flow of the UE that uses the application. |
| > Start time | UPF | Start time of traffic detection for the flow. |
| > End time | UPF | End time of traffic detection for the flow. |
| > UL Data volume | UPF | Measured UL data traffic volume for the flow. |
| > DL Data volume | UPF | Measured DL data traffic volume for the flow. |
| > UL Data Rate | UPF | Measured UL data rate for the flow. |
| > DL Data Rate | UPF | Measured DL data rate for the flow. |
| > URL list | UPF | List of URLs extracted from the inspected user plane packets in the flow. |
| > Domain Name list | UPF | List of domain names extracted from the inspected user plane packets in the flow. |
| PFD Information | NEF(PFDF) | PFD Information stored in the UDR (as Application Data) and retrieved by NEF, as defined in clause 6.1.2.3.2 of TS 23.503 [4]. |
| > Application ID | NEF(PFDF) | Identification of the application that refers to one or more application detection filters. |
| > IP 3-tuple list | NEF(PFDF) | Including protocol, server side IP address and port number. |
| > URL list | NEF(PFDF) | The significant parts of the URL to be matched, e.g. host name. |
| > Domain Name list | NEF(PFDF) | A Domain name matching criteria and information about applicable protocol(s). |

NOTE 1:	Extensive reporting of all traffic flows may conflict with the requirement to avoid extra UPF load. An NWDAF may subscribe only for reporting for some UEs to limit the load.

#### 6.1x.3 Output Analytics

The NWDAF supporting PFD determination Analytics provides the analytics results to consumer NFs, e.g. NEF (PFDF).

Table 6.1x.3-1: PFD determination statistics

| Information | Description |
| --- | --- |
| Application ID | Application ID of the application that refers to the obtained application detection filters (IP 3-tuple, URL and/or Domain name list). |
| S-NSSAI | Identifies the Network Slice for which analytics information is provided. |
| DNN | Identifies the data network name (e.g. internet) for which analytics information is provided. |
| IP 3-tuple list | Flow descriptor containing 3-tuple, server side (destination address, port, and protocol). |
| URL list | The significant parts of the URL to be matched, e.g. host name. |
| Domain name list | A Domain name matching criteria and information about applicable protocol(s). |

Table 6.1x.3-2: PFD determination predictions

| Information | Description |
| --- | --- |
| Application ID | Application ID of the application that refers to the obtained application detection filters (IP 3-tuple, URL and/or Domain name list). |
| S-NSSAI | Identifies the Network Slice for which analytics information is provided. |
| DNN | Identifies the data network name (e.g. internet) for which analytics information is provided. |
| IP 3-tuple list | Flow descriptor containing 3-tuple, server side (destination address, port, and protocol). |
| URL list | The significant parts of the URL to be matched, e.g. host name. |
| Domain name list | A Domain name matching criteria and information about applicable protocol(s). |
| Confidence | Confidence of this prediction. |

#### 6.1x.4 Procedures

The NWDAF can provide PFD determination analytics to a 5GC NF (e.g. NEF (PFDF)).

Figure 6.1x.4-1: Procedure for PFD determination analytics

1.	The Consumer NF, e.g. NEF(PFDF), requests or subscribes to the NWDAF to request analytics for PFD determination. The Target of Analytics Reporting is set to AnyUE. A single App-ID or a list of App-IDs are also provided as input. The Analytics Filter Information may optionally include the S-NSSAI and DNN.

2.	The NWDAF fetches currently stored PFD information from NEF(PFDF) by including as input the App-ID. In case the consumer provided an App-ID list, the fetch procedure is repeated for each App-ID in the list.

3.	The NWDAF collects user plane traffic information from the UPF using Nupf event exposure service. Exposure of events from UPF to NWDAF is specified in TS 23.502 [3] clause 4.15.

Editor’s Note: UPEAS WI is specifying UPF event exposure for NWDAF cata collection. It is FFS whether TS 23.502 clause 4.15.and the other UPEAS normative work address the needs of this new analytic.

4.	The NWDAF derives the requested analytics by obtaining new/updated PFDs for the application/s of interest.

5.	The NWDAF notifies the analytics consumer NF, e.g. NEF (PFDF), with the analytic result, which includes new/updated PFDs for the application/s of interest.

********** End of Changes ***************