SA WG2 Meeting #S2-155	S2-2302183

20 - 24 February, 2023, Athens, Greece

SA WG2 Meeting ##154AH-e	S2-2300029

16 - 20 January, 2023, Electronic Meeting

3GPP TSG-CT WG1 Meeting #139	C1-227197

# Toulouse, France, 14th – 18th November 2022

# Title:	LS on Handling of the Allowed PDU session status IE in Non-allowed service area

# Response to:	---

# Release:	Rel-18

# Work Item:	5GProtoc18

Source:	CT1

To:	SA2

Cc:

Contact Person:

Name:	Robert Zaus

E-mail Address:	rzaus at apple dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	none

1. Overall Description:

At CT1#139e, CT1 discussed certain requirements related to the UE behaviour in a non-allowed service area.

CT1 would like to ask SA2 for guidance on an issue. The scenario is as follows:

- The UE is registered to an AMF both via 3GPP and non-3GPP access. The UE is currently in a non-allowed area. The registration area (TAI list) for 3GPP access includes both TAs from the UE's non-allowed area and TAs from the allowed area. The UE is currently in a non-allowed area.

- The UE has at least 1 PDU session activated via non-3GPP access which cannot be transferred to 3GPP access.

- The UE is in IDLE mode both for 3GPP and non-3GPP access, when it receives a paging via 3GPP access indicating non-3GPP access and responds with a Service Request.

CT1 understands that if the UE responds to the paging when it is in a TA belonging to the allowed area, then the UE needs to include the Allowed PDU session status IE (in 23.502 terminology: "List Of Allowed PDU Sessions") indicating which of the PDU sessions associated with non-3GPP access can be transferred to 3GPP access and may include the Uplink data status IE if it has uplink data pending.

Question 1: Does the UE need to include the Allowed PDU session status IE also if it responds to the paging when it is in a non-allowed area?

Question 2: Is the UE allowed to include the Uplink data status IE if it has uplink data pending if it responds to the paging when it is in a non-allowed area?

2. Actions:

To SA2 group.

ACTION: 	CT1 kindly ask SA2 to answer CT1's question.

3. Date of Next CT1 Meetings:

CT1#140	27th February - 03rd March 2023		Athens, Greece

CT1#141e	17th - 21st April 2023 		E-Meeting