3GPP TSG-WG SA2 Meeting #155	S2-2302376

# Athens, Greece, Feb 20th – 24th, 2023                                       (revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3884 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarify Preferred UE RAT type and/or Access type criteria for UE member selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-02-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Describe the procedure when the AF indicates a “Preferred UE Access type and/or RAT type” filtering criteria for UE member selection |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added procedures for indicating the Preferred UE Access type and/or RAT type filtering criteria for UE member selection |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not able to provide the Preferred UE Access type and/or RAT type filtering criteria assistance to UE member selection. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.X.2 (new clause), 4.15.X.3.x(new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 23.502 CR 3674 TS/TR … CR… |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

4.15.X.2	UE Member Filtering Criteria for 5GS assistance to UE member selection

| UE Member filtering criteria | Description of filtering criterion | UE filtering information | Detailed description clause |
| --- | --- | --- | --- |
| Preferred UE Access type and/or RAT type | indicates the expected access type and/or RAT type, where the candidate UE should stay, e.g. Access type =3GPP or Non-3GPP. RAT type =5G NR, WLAN, |  | 4.15.X.3.x |
| Number of UEs fed back | indicates the minimum number of candidate UEs that need to be fed back |  | 4.15.X.3.x |

* * * Next Change (all new text) * * *

4.15.X.3.x	Specific procedure for the filtering criteria Preferred UE Access type and/or RAT type

This clause describes an example of the procedure that 5GS assistance for FL member selection based on Preferred UE Access type and/or RAT type.

- AF requests NEF assistance to support the UE selection by considering the Preferred UE Access type and/or RAT type, AF includes the UE lists or AOI and the following criteria as part of the FL UE selection request:

- Preferred UE Access type and/or RAT type: indicates the expected Access type and/or RAT type, where the candidate UE should stay, e.g. Access type =3GPP or Non-3GPP, RAT type =5G NR, WLAN, etc.

- Number of UE fed back: indicates the minimum number of candidate UEs that need to be fed back.

2a-2b. If a list of GPSI is received from AF, then the NEF translates the GPSIs to SUPIs, or if an AOI is received from AF, then the NEF retrieves SUPI list of the UEs in the AOI from the AMF.

  - According to the criteria “Preferred UE Access type and/or RAT type”, the NEF invokes Nsmf_EventExposure_Subscribe service operation with the SUPI list, event ID =RAT type, and/or, event ID=Access type. SMF will provide the RAT type and/or Access type for each UE in the SUPI list to the NEF.

5.  Based on the RAT type and/or Access type of UE(s) retrieved from SMF in steps 3-4, the NEF selects the candidate UEs, which are in the Preferred RAT or Access type.

6. [Conditional] If the Preferred UE access type and/or RAT type is Non-3GPP, for the candidate UEs retrieved in step 5, the NEF may invoke Nnwdaf_AnalyticsSubscription_Subscribe with Analytics ID =WLAN performance, Target of Analytics Reporting=SUPI(s), Analytics target period, Analytics Filters = AOI and optionally with a WLAN performance Threshold vaule.

7. [Conditional] The NWDAF provides WLAN performance analytics per UE by invoking Nnwdaf_AnalyticsSubscription_Notify service operation, the output analytics is as defined in clause 6.11.3, TS 23.288.

8. [Conditional] Based on the WLAN performance analytics per UE received from the NWDAF in step 7, the NEF can further abbreviate the range of candidate UEs.

9. According to the criteria “Number of UE” from the AF indicating the minimum number of candidate UEs that need to be fed back, the NEF sends a final candidate UEs list to the AF by invoking Nnef_UESelectionAssistance_Notify (Candidate UEs list).

* * * End of Changes * * *