3GPP TSG-WG SA2 Meeting #155 	S2-2302573

February 20 – 24, 2023, Athens, Greece	(revision of S2-23xxxxx)

Source:	CATT

Title:	QoS handling general description

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract: This paper proposes to add the QoS handling general description into TS 23.586.

1. Introduction

This paper proposes to add the QoS handling general description into TS 23.586 based on the following KI#1 conclusions in TR 23.700-86.

-	The specific Ranging/SL positioning parameters to UE include:

-	The mapping between Ranging/SL positioning services (e.g. ProSe identifiers, V2X service types) and Ranging/SL positioning QoS parameters. Ranging/SL Positioning QoS parameters include RSPP transport QoS, i.e. PQI if RSPP is transported over PC5-U, and positioning QoS.

NOTE 1:	Details of the RSPP transport QoS and the positioning QoS will be coordinated with RAN WGs during the normative phase. Whether RSPP is transported over PC5-U depends on the outcome of KI#4.

NOTE 2:	Positioning QoS parameters can also be dynamically provided in the Ranging/SL Positioning service request by the service application.

-	The specific Ranging/SL positioning service authorization to RAN include:

-	QoS parameters used by NG-RAN.

NOTE 4:	What QoS parameters of Ranging/SL positioning used by NG-RAN will be specified during normative phase.

2. Text Proposal

It is proposed to capture the following changes into TS 23.586.

# * * * * First change* * * *

5.7	QoS handling

5.7.1	QoS handling for RSPP transport

In order to support QoS handling for RSPP transport, the mechanism defined in clause 5.4 of TS 23.287 [6] is reused with the following differences:

-	Only NR PC5 QoS model is used.

-	PC5 Packet Filter Set supports the Ranging/SL Positioning Packet Filter Set. Each PC5 QoS Rule additionally contains the Ranging/SL Positioning service identifier when the Ranging/SL Positioning service identifier is not included in the PC5 Packet Filter Set.

-	V2X Packet Filter Set is replaced by Ranging/SL Positioning Packet Filter Set. Ranging/SL Positioning Packet Filter Set shall support Packet Filters based on at least any combination of:

-	Ranging/SL Positioning service identifier;

-	Source/Destination Layer-2 ID;

-	Application Layer ID.

-	V2X application layer is replaced by Ranging/SL Positioning application layer.

-	V2X layer is replaced by Ranging/SL Positioning layer.

-	V2X service type is replaced by Ranging/SL Positioning service identifier.

-	UE-PC5-AMBR is only applied for NR PC5.

Editor's note:	PQI values for RSPP transport is FFS.

5.7.2	QoS handling for LCS QoS

The LCS QoS as defined in TS 23.273 [8] is applied to Ranging/SL Positioning.

Editor's note:	Any adaptation of LCS QoS is FFS.

# * * * * End of changes * * * *