3GPP TSG-SA WG2 Meeting #155	S2-2302605

# Athen, Greece Feburary 20 – 24, 2023	(revision of S2-2301399)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0088 | rev | 3 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | The EAS discovery procedure with V-EASDF using IP replacement mechanism for supporting HR-SBO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [China Unicom,] LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the conclusion in the clause 8.1.2 of TR 23.700-48, the EAS discovery part for the option D is concluded as follows: - For Option D, the H-SMF sends H-DNS server address included in PCO to UE via V-SMF during PDU Session Establishment/Modification procedure. The DNS query related to the edge computing (corresponding to FQDNs) can be routed to V-EASDF/Local DNS server in the VPLMN reusing IP replacement mechanism. In case of routing to V-EASDF, V-UPF replaces the destination IP address of DNS query (corresponding to FQDNs) to V-EASDF and routes this DNS query to V-EASDF, the EAS discovery procedure with V-EASDF which implements the functions of EASDF refers to TS 23.548. For the DNS query requiring DNS resolution in the HPLMN, the DNS resolution path is same as the normal path in the HR PDU Session. The procedure and restriction of Option D defined in clause 6.2.3.2.3 of TS 23.548 still apply in the HR roaming case. For revision 3, the following Editor's note remains to be resolved regarding when the V-SMF to select the V-EASDF and for overlapping step to be replaced by the reference. Editor’s Note: it is FFS When the V-SMF selects the V-EASDF in this scenario. Editor’s Note: If some steps of this procedure overlap with the description of clause 6.7.2.2, the necessary reference to clause 6.7.2.2 will be done. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to describe the EAS discovery procedure with V-EASDF using IP replacement mechanism for supporting HR-SBO. Revision 3 resolves the remaining editor's notes to be aligned with the procedure for PDU Session supporting HR-SBO in VPLMN in clause 6.7.2.2 and the procedure for EAS Discovery with V-EASDF for HR-SBO roaming scenario 6.7.2.3 for consistency. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conclusion of the study item is not reflected in the normative specification. Unresolved Editor's note in the procedure remains without revision 3.. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change (all new text) * * * *

6.7.2.X	EAS discovery procedure with V-EASDF using IP replacement mechanism for supporting HR-SBO

Based on the operator's configuration and local regulations, the IP replacement mechanism may be used for EAS discovery supporting HR-SBO:

-	For supporting HR-SBO, the H-SMF sends DNS server address of HPLMN included in PCO to UE via V-SMF during PDU Session Establishment/Modification procedure. The DNS query related to the edge computing (corresponding to FQDNs) can be routed to V-EASDF/Local DNS server in the VPLMN using IP replacement mechanism.

NOTE X: This EAS discovery procedure requests modification of IP address of DNS messages. Whether this is allowed or not is subject to local regulations. As this procedure for EAS discovery requires an UPF to detect target FQDN in DNS message, it cannot apply when DNS security applies e.g. it does not apply to usage of DoH or DoT.

Figure 6.7.2.X-1: EAS discovery procedure with V-EASDF using IP replacement mechanism for supporting HR-SBO

0.	The registration procedure is described in step 1 of clause 6.7.2.2.

1.	The HR-SBO PDU Session establishment is described in the step 2 Figure 6.7.2.2-1 with the following differences:

The V-SMF selects a UPF in VPLMN supporting UL-CL and PSA functionalities based on UE location information and the HR-SBO allowed indication after the step 3 of Figure 4.3.2.2.2-1 of TS 23.502[X].

NOTE Y: Based on the deployment of VPLMN, The selected UPF in VPLMN can support both UL-CL and PSA functionalities and/or can be the V-UPF selected in the step 4 of Figure 4.3.2.2.2-1 of TS 23.502[X].

The V-SMF selects the V-EASDF and sends the request for establishment of the PDU session supporting HR-SBO in VPLMN without the V-EASDF IP address in the step 6 of Figure 4.3.2.2.2-1 of TS 23.502[X].

If this Nsmf_PDUSession_Create Request received from the V-SMF does not include the V-EASDF IP address, the H-SMF constructs PCO with DNS server address field set to DNS server address of HPLMN and sends the PCO to UE via V-SMF.

2.	The V-SMF configures the V-EASDF and the V-UPF as described in step 3 of clause 6.7.2.2.

Based on the FQDN range received from the optional VPLMN specific offloading policy, the V-SMF indicates the UPF in VPLMN to route DNS queries for the FQDN (range) query to V-EASDF. The V-SMF configures the UPF in VPLMN with IP replacement information (i.e. DNS server IP address and port number of HPLMN, V-EASDF IP address and port number). In uplink direction, UPF in VPLMN replaces the destination address of the DNS query for DNS requests targeting an FQDN eligible for HR-SBO related offload from DNS server IP address of HPLMN to V-EASDF IP address; In downlink direction, UPF in VPLMN replaces the source address of the DNS response for DNS requests targeting an FQDN eligible for HR-SBO related offload from V-EASDF IP address to DNS server IP address of HPLMN.

NOTE Z: For the DNS query requiring DNS resolution in the HPLMN, the DNS resolution path is same as the normal path in the HR PDU Session.

3.	UE sends DNS query to DNS server of HPLMN.

4a.	If the DNS query does not match the FQDN range eligible for HR-SBO related offload, UPF in VPLMN delivers the DNS query via H-PSA via N9 and H-PSA delivers the DNS query to the DNS server of HPLMN.

4b.	If the DNS query matches the FQDN range eligible for HR-SBO related offload, the UPF in VPLMN delivers the DNS query to V-EASDF using IP replacement mechanism. The following EAS discovery procedure is based on step 4b.

5.	The EAS discovery procedure with V-EASDF for HR-SBO described in step 2-4 of clause 6.7.2.3 applies.

6.	The V-EASDF sends the DNS response including FQDN to the UPF in VPLMN. The UPF in VPLMN replaces the source address from V-EASDF to DNS server of HPLMN in the DNS response based on the V-SMF instructions and sends this DNS response to the UE directly or via UL-CL of VPLMN if existing.

* * * * End of Changes * * * *