3GPP TSG-WG SA2 Meeting #155 	S2-2302746

February 20 – 24, 2023, Athens, Greece	(revision of S2-220xxxx)

Source:	vivo

Title:	TS 23.586: Procedures for Target UE requested positioning with Ranging involved

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract:  This paper implements to the Procedures for Target UE requested positioning with Ranging involved.

1. Introduction/Discussion

This paper implements the following procedure based on the conclusions in KI#5 Network assisted SL Positioning.

-	If Target UE cannot establish NAS connection with AMF, for MO-LR or pending MT-LR (e.g. deferred MT-LR), the following procedures can be used:

Option a):

-	The Target UE performs the Located UE's discovery and selection.

-	The Target UE may request for the Location of the Located UE(s).

-	If the Located UE(s) needs network assistance to retrieve its location, the Located UE(s) may trigger a MO-LR. This assumes that the Located UE(s) has a NAS connection to an AMF.

-	The Target UE uses the location returned by Located UE(s) together with the ranging/SL positioning measurement data or result to estimate its own location. The Ranging/Sidelink positioning and the positioning of the Located UE(s) can be scheduled with the same time (according to TS 23.273 [11]) to improve the Target UE positioning accuracy.

Option b):

-	The Target UE performs the Located UE's discovery and selection.

-	The Target UE may transmit its ranging measurements/results to the Located UE(s).

-	Located UE(s) may report the ranging/SL positioning measurement result to the LMF. This may include ranging measurements/results received from the Target UE. The endpoints for LPP messages can be the LMF and the Located UE(s) in this case.

-	The LMF may use the received information to calculate the location of the Target UE, and provide the resulting location via the Located UE to the Target UE or via 5G NF to the LCS client or the Application Server.

NOTE 6:	Security aspects require confirmation from SA WG3.

NOTE 7:	Supporting of location report via the Located UE to the LMF needs to be confirmed by RAN.

NOTE 8:		It will be determined during Normative phase that when AMF does not maintain target UE's context or GMLC cannot get target UE AMF address because of deregistration, whether and how location report from Located UE on behalf of Target UE is supported.

Above conclusion is initiated by Target UE itself and there isn’t NAS connection between Target UE and AMF, these above related processes are not suitable to be classified to the Network assisted Ranging. Option A and B can have no network involved in, at most the network is involved in to calculate the Ranging result and at most it can be called Network based. So for above conclusions, it is proposed to use a sperate clause to describe the procedure, which may be more suitable.

2. Text Proposal

It is proposed to capture the following changes to TS 23.586.

# * * * * First change * * * *

## 6.X	Procedures for Target UE requested positioning with Ranging involved

### 6.X.1	General

The procedures for Target UE requested positioning with Ranging involved occurs when the Target UE cannot establish NAS connection with AMF, which includes the following options:

-	Target UE derived positioning with Ranging involved as described in clause 5.X.2

-	Located UE derived positioning with Ranging involved for Target UE as described in clause 5.X.3

### 6.X.2	Procedures for Target UE derived positioning with Ranging involved

Figure 6.X.2-1 illustrates the general procedures for Target UE derived positioning with Ranging involved.

Figure 6.X.2-1: Target UE derived positioning with Ranging involved Procedure

1)	When Target UE cannot establish NAS connection with AMF and to obtain its own location, the Target UE determines to use the Located UE’s assistant, then the Target UE performs the Located UE discovery and selection.

2)	Target UE sends a Ranging service request to the selected Located UE(s) to request for the location of Located UE(s).

3)	After receiving the Ranging service request, if the Located UE doesn’t have the available location of itself, the Located UE perform the MO-LR procedure as described in clause 6.2 of TS23.273[8].

4)	The Located UE(s) return its location to the Target UE.

5)	The Target UE performs the Ranging and sidelink procedure with the Located UE(s), this step can be initiated after the step1.

6)	The Target UE derives its own location based on Ranging and Sidelink positioning measurement data/result and the location of the Located UE(s)

### 6.X.3	Located UE derived positioning with Ranging involved for Target UE

Figure 6.X.3.1-1 illustrates general Located UE derived positioning with Ranging involved for Target UE.

Figure 6.X.2-1: Located UE derived positioning with Ranging involved for Target UE Procedure

1)	When Target UE cannot establish NAS connection with AMF and to obtain its own location, the Target UE determines to use the Located UE’s assistant, then the Target UE performs the Located UE discovery and selection.

2)	The Target UE performs the Ranging/Sidelink Positioning procedure with the selected Located UE(s).

3)	The Target UE sends a Ranging service request to the Located UE for requesting its own location.

4)	After receiving the Ranging service request:

-	If the Located UE doesn’t have the available location of itself, the Located UE perform the MO-LR procedure as described in clause 6.2 of TS23.273[8] to obtain its own location. Then the Located UE derives the location of the Target UE based on the Ranging and Sidelink positioning measurement data/result with the Target UE and the location of the Located UE. In this case, after the derivation, the following process can jump to the step 11 directly.

-	If the Located UE can not derives the Target UE’s location, the Located UE may initiate a enhanced MO-LR procedure by sending the MO-LR request to the AMF, the MO-LR request additionally includes the Ranging/Sidelink Positioning measurement data/result to indicate the AMF that it is for Network calculated positioning with Ranging and Sidelink positioning.

5)	The AMF performs the LMF selection taking the Ranging/Sidelink Positioning capability of LMF into account.

6)	The AMF sends a Nlmf_Location_ProvideLocation reuqest including Ranging/Sidelink Positioning measurement data/result to the LMF.

7)	The LMF performs the Located UE positioning procedure and obtains the Located UE’s location.

9)	The LMF uses the location of Located UE together with the ranging/SL positioning measurement data or result reported by Located UE to estimate the location of the Target UE.

10)	The LMF responds to the AMF the Target UE’s Location in the Nlmf_Location_DetermineLocation response.

11)	The AMF sends the MO-LR response to Located UE including the Target UE’s location.

12) The Located UE responds to the Target UE with the Target UE’s location included in the Ranging service response.

# * * * * End of changes * * * *