3GPP TSG SA WG2 Meeting #155	S2-2302847

# February 20 - 24, 2023, Athens, Greece	(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1274 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Providing subscriber specific graphical user interface to the UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Sol#20 in TR 23.700-87 is used as baseline for the DC solution description in TS 23.228. TR 23.700-87 contains a procedure in clause 6.20.2.6 on how the network can provide a subscriber specific application list (HTML based menu of applications from which the user can select one or more) to the UE via the bootstrap data channel. This menu can be adapted to user and device capabilities and is build based on user’s subscription data. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new procedure in clause XX.x.x is explaining how the network provides the subscriber specific application menu to the UE via the bootstrap DC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete DC solution. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex XX.x.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** BEGIN of Changes (all new text) ****

Annex XX (normative):
Support of Data Channel services in IMS

# XX.x	Procedures

### XX.x.y	Providing subscriber specific graphical user interface to the UE

According to clause 6.2.10.1 of TS 26.114 [76], the UE can send a HTTP GET Request for the root ("/") URL through the bootstrap data channel which is replied with a data channel application describing the graphical user interface and the logic needed to handle any further data channel usage beyond the bootstrap data channel itself. The graphical user interface can e.g., contain a menu of applications, from which the user can choose one or several. The graphical user interface should be subscriber specific and contain only applications for which the user has subscribed to and is authorized to use. To provide the graphical user interface containing subscriber specific data channel applications via the bootstrap data channel to the UE, the DCSF, when receiving a call event notification including subscriber specific information from the IMS AS, creates the URL identifying the graphical user interface for this subscriber.

NOTE X:	How the DCSF creates the URL based on subscription and potentially other information is not specified in this release of the specification.

The DCSF provides the URL via the IMS AS to the DCMF or enhanced MRF during data channel media resource reservation for the bootstrap data channel. The DCMF or enhanced MRF associate this URL with the bootstrap data channel. Once the DCMF or enhanced MRF, acting as HTTP proxy, receive a HTTP GET Request containing the root ("/") URL through the bootstrap data channel, the DCMF or enhanced MRF replace the root ("/") URL in the HTTP GET Request with the subscriber specific URL and forward the request to the DCSF for downloading the subscriber specific graphical user interface to the UE.

**** End of Changes ****