3GPP TSG-SA WG2 Meeting #155	S2-2302850

Athens, 20-24 February 2023

Title:	[draft] LS on network slices with Area of Service not matching a Tracking area

Response to:	None

Release:	Rel-18

Work Item:	eNS_Ph3

Source:	SA2

To:	SA5

Cc:	RAN3

Contact Person:

Name:	alessio casati

E-mail Address:	alessio.casati@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	none

1. Overall Description:

SA2 is defining the support of network slices with Area of Service not matching a Tracking Area boundary (i.e. the AoS can be mapped to a list of TAs and a list of cells, rather than just to a list of TAs).

It is SA2 understanding that SA5 represents the NG.116 Area of Service attribute by the CoverageArea attribute in the Service Profile defined in TS 28.541. This then is translated into a list of Tracking Areas in the CoverageAreaTAList  (of the RANSliceSubnetProfile , CNSliceSubnetProfile, TopSliceSubnetProfile.

SA2 is defining a network slice availability information to convey the cells of a tracking area where the network slice is available (i.e. cells of a TA belonging to the AoS where a UE can access the network slice because more than zero resources are allocated to the network slice) if the network slice is not available in the whole TA. The Network slice support is still indicted to CN at TA level by the RAN, but the availability is configured in the RAN (by allocating more than zero resources, see next paragraph) and in the CN with cell level granularity for the TAs where a network slice is not available in the whole TA.

The network slice availability information is expected to be linked with the configuration of the RAN (i.e. a slice is configured with greater than zero resources in the cells that are identified in the network slice availability information). Also, the network slice availability information may be provided by OAM to the AMF so the AMF can configure supporting UEs with this information.

SA2 believes that when deployments enable the above feature, the Coverage Area would have to be translated in the RANSliceSubnetProfile , CNSliceSubnetProfile, TopSliceSubnetProfile in additional information that enables also the identification of cell level granularity availability of a network slice in some TAs of the list of TAs in the CoverageAreaTAList  where the network slice is indicated to be supported.

2. Actions:

To RAN2 and RAN3 group.

ACTION: 	SA2 kindly asks SA5 to take into account the above information and update their specifications as necessary.

3. Date of Next SA2 Meetings:

Next SA2 Meetings dates