SA WG2 Meeting #S2-155	S2-2303037

20 – 24 February 2022, Electronic

Source: 	MediaTek Inc.

Title: 	pCR 23.586: Authorization and Provisioning

Document for: 	Approval

Agenda item: 	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract of the contribution: This contribution proposes some changes to the authorization and provisioning.

# pCR 23.586 v0.1.0

**** FIRST CHANGE ****

## 5.2	Authorization and Provisioning for Ranging/SL positioning service

In 5GS, the parameters for Ranging/SL positioning service may be made available to the UE in the following ways:

-

-	provisioned in the ME and/or configured in the UICC; or

-	provided or updated by the Ranging/SL positioning Application Server via PCF and/or PC1 reference point; or

-	provided or updated by the PCF to the UE.

The UE shall apply any given parameterin the following priority order:

-	provided or updated by the PCF (regardless whether the parameter is determined by the PCF itself or provided by the Ranging/SL positioning Application Server to the PCF), if available, otherwise;

-	provided or updated by the Ranging/SL positioning Application Server via SR1 reference point, if available, otherwise;

-	configured in the UICC, if available, otherwise;

-	provisioned in the ME, if available.

The parameters provided or updated by the Ranging/SL positioning Application Server via SR1 reference point may need to be complemented with configuration data from other sources listed above.

NOTE:	The Ranging/SL positioning Application Server can provision the same Ranging/SL positioning parameters via 5GC or directly to the UE via SR1 reference point, and can revoke (e.g. delete) the Ranging/SL positioning parameters via 5GC in order for the provisioning via SR1 reference point to take effect.

The basic principles of service authorization and provisioning for Ranging/SL positioning service are as follows:

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorized to use Ranging/SL positioning.

-	The PCF in the HPLMN merges authorization information from home and other PLMNs and provides the UE with the final authorization information.

-	The UE is authorized to use Ranging/SL positioning only in the PLMN(s) indicated by the PCF, and the equivalent PLMNs thereof.

-	The PCF in the VPLMN or HPLMN may revoke the authorization (via H-PCF when roaming) at any time by using the UE Configuration Update procedure for transparent UE Policy delivery procedure defined in clause 4.2.4.3 of TS 23.502 [3].

-	The Ranging/SL positioning Policy/parameters provisioning to UE is controlled by the PCF and may be triggered by UE. The PCF provisions one or more of the following Policy/parameters:

-	When the UE is served by NG-RAN, the PLMNs in which the UE is authorized to perform Ranging/SL positioning service over PC5 reference point for acquiring relative distance, relative direction, high accuracy relative distance and high accuracy relative direction.

-	When the UE is not served by NG-RAN, indicates whether the UE is authorized to perform Ranging/SL positioning service over PC5 reference point for acquiring relative distance, relative direction, high accuracy relative distance and high accuracy relative direction.

-	The mapping between Ranging/SL positioning services (e.g. ProSe identifiers, V2X service types) and Ranging/SL positioning QoS parameters.

-	The authorized Ranging/SL positioning role per PLMN, e.g. target UE, reference UE, assistant UE, and located UE.

-	When the UE is not served by NG-RAN, indicates the authorized Ranging/SL positioning role, e.g. target UE, reference UE, assistant UE, and located UE.

**** END OF CHANGES ****