3GPP TSG SA WG2 Meeting #155 	S2-2333161

Athens, 20-24 February 2023	(revision of S2-220xxxx)

Source:	InterDigital Inc.

Title:	On when to enable registration to an S-NSSAIs rejected partially in a Registration Area

Document for:	Discussion and Agreement

Agenda Item:	9.14

Work Item / Release:	eNS_Ph3/Rel-18

Abstract: This paper addresses handling the case where an S-NSSAI is rejected partially in the Registration Area (KI#5, but the UE’s current TA is not one of the TA’s where the S-NSSAI is considered “not-supported”.

# 1. Introduction/Discussion

The concept of an “S-NSSAI rejected partially in the RA” was introduced as part of the conclusion reached in 23.700-41. This conclusion reads that:

“AMF can provide information to the UE enabling the UE to be able to register an S-NSSAI that is rejected S-NSSAI for the RA, when the UE moves to a TA supporting the S-NSSAI. The AMF can provide additional IE to the UE e.g., Partially rejected S-NSSAI in the registration procedure and the UE configuration update procedure if the UE indicates that it supports this feature. The UE shall be able to request a rejected S-NSSAI, by initiating a registration update procedure, in a supported TA based on the supported/not supported TA information associated with this S-NSSAI. The current concept of Allowed NSSAI and uniform support of it in UE’s RA is not impacted by this feature based on indication of where in the RA certain rejected S-NSSAIs are supported/not supported. In this approach, the Allowed NSSAI is still uniformly supported in the RA.”

With this approach it is possible that the TA where the UE is receiving the information regarding “S-NSSAIs rejected partially in the RA” (i.e., the current TA) is not included in the list of TAs where some S-NSSAI are rejected partially, Later, the condition that caused the S-NSSAI to be rejected partially may cease to exist (e.g., a congestion scenario) and therefore the UE should be able to attempt to register to S-NSSAI that were previously not available, without explicit network signalling (e.g., UCU).

Observation 1: In some scenarios, rejecting an S-NSSAI partially in a RA may be based on having reached a threshold, due to congestion, or due to local AMF policies. As such the UE might have a preference regarding S-NSSAI(s) that are rejected partially in the current RA over S-NSSAIs part of the Allowed NSSAI. Note that if an S-NSSAI is not in the Allowed NSSAI and this S-NSSAI is rejected partially in other TA of the RA, but not the current, the UE might attempt a Mobility Registration Update.

Proposal 1: Enable the UE to signal its preference regarding S-NSSAI(s) partially rejected in the current RA over existing S-NSSAI in the current Allowed NSSAI, when attempting a Periodic Registration Update or Mobility Registration Update.

Action 1: Update clause 5.15.X to reflect this capability

# 2. Proposal

It is proposed that the CRs in S2-2301483 be updated to reflect the proposals described above.