3GPP TSG-SA WG2 Meeting #154AHE	S2-2303398

# E-meeting, 16th – 20th January 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0179 | rev | 5 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Mobility procedures for UEs receiving multicast MBS session data in RRC Inactive state |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Huawei, HiSilicon, Ericsson, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As described in the conclusion for Key Issue #1 "MBS session reception in RRC Inactive" in TR 23.700-47: - When an RRC_INACTIVE UE is in the process of receiving ongoing MBS session data and moves to a new cell within the RNA, the UE shall be able to receive MBS session data. - When the UE moves outside the RNA, the UE performs UE Triggered Connection Resume in RRC Inactive procedure to the target RAN node as per existing procedures in TS 23.502 [3]. - When the UE receives the MBS data in RRC Inactive state and move out of the RNA area but Connection resume fails, it follows existing procedures and transition to CM-IDLE. When the UE transition to CM-IDLE since is not able to receive the MBS multicast data at the new cell, the UE initiates the mobility registration update or Service Request procedure and activates the associated PDU session, so that the shared tunnel (if not already established) or the individual delivery can be established towards the NG-RAN node for multicast data delivery to the UE. - When the UE moves outside the RA, the UE performs mobility registration procedure and as per the existing procedure in clause 5.3.3.2.5 of TS 23.501 [2] for "Mobile initiated NAS signalling procedure" for UEs in CM-CONNECTED with RRC Inactive state, the UE will resume the RRC Connection. - During the handover procedure, the SMF includes the "MBS assistance information for RRC Inactive" if any in N2 SM Info and sent to NG-RAN via AMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add mobility procedures for UEs receiving multicast MBS session data in RRC Inactive state. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mobility procedures for UEs receiving multicast MBS session data in RRC Inactive state is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3.Y (new), 7.2.3.Y.1 (new), 7.2.3.Y.2 (new), 7.2.3.Y.3 (new), 7.2.3.Y.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

7.2.3.Y	Mobility procedures to enable delivery of multicast MBS session data to UEs in RRC_INACTIVE state

7.2.3.Y.1	General

The procedures in clause 7.2.3.2, 7.2.3.3, 7.2.3.4, 7.2.3.6 and 7.2.3.7 apply with the following additions:

-	If MBS assistance information is available at the SMF and the target RAN node supports MBS, depending on configuration the SMF may always provide the MBS session information comprising the MBS assistance information towards the target RAN node. This includes the following procedures:

-	For the Xn based handover procedure, if MBS assistance information is available at SMF and the target RAN node supports MBS, after the successful handover SMF triggers the modification of the associated PDU session to provide the MBS assistance information within the MBS session information to target NG-RAN.

-	During the N2 based handover preparation phase, if MBS assistance information is available at SMF and the target RAN node supports MBS, SMF provides the MBS assistance information in the MBS session information to target NG-RAN.

- The MBS session information transferred from source NG-RAN towards target NG-RAN also include MBS assistance information if such information is available at the source RAN node.

Editor's note: 	It is up to RAN WG to confirm that the MBS session information transferred from source NG-RAN towards target NG-RAN includes MBS assistance information.

NOTE 1:	In deployments where not all the MBS supporting NG RAN nodes support delivery of multicast MBS session data to UEs in RRC_INACTIVE state, a source NG-RAN node not supporting delivery of multicast MBS session data to UEs in RRC_INACTIVE state will not provide MBS assistance information to the target NG-RAN node.

NOTE 2:	It is assumed that a RAN node not supporting delivery of multicast MBS session data to UEs in RRC_INACTIVE state ignores the MBS assistance information.

For a UE that is receiving multicast MBS data in CM-CONNECTED with RRC_INACTIVE state, the scenarios for UE mobility are as follows:

-	UE moves to new cell within the RAN Notification Area (RNA).

-	UE moves outside the current RAN Notification Area but within the current Registration Area (RA).

-	UE moves out of the current Registration Area.

7.2.3.Y.2	Mobility of UE in RRC_INACTIVE state receiving MBS data within RNA

When a UE in RRC_INACTIVE state is receiving ongoing MBS session data and moves to a new cell within the RNA, the UE shall be able to continue to receive the MBS session data.

Editor's note:	The procedure of UE mobility within RNA is to be specified in RAN WGs and alignment with RAN will be done later.

7.2.3.Y.3	Mobility of UE in RRC_INACTIVE state receiving MBS data out of RNA and within RA

When a UE in RRC_INACTIVE state is receiving multicast MBS session data, if the UE moves out of the RNA and within the RA, the UE resumes the connection in the same way as specified in clause 7.2.3.7 with the following enhancements:

-	If the UE context cannot be retrieved successfully, the NG-RAN triggers the UE to move to RRC_IDLE state as specified in TS 38.300 [9]. The UE initiates the Mobility Registration Update procedure or Service Request procedure to activate the associated PDU session(s). Hence the shared delivery (if not already established) or the individual delivery can be established towards the NG-RAN node to enable delivery of multicast MBS session data to the UEs.

Editor's note:	Whether the handling of UE context retrieval failure should be captured in clause 7.2.3.7 is FFS.

7.2.3.Y.4	Mobility of UE in RRC_INACTIVE state receiving MBS data out of RA

When the UE in RRC_INACTIVE receiving multicast MBS session data, if the UE moves out of the RA, the UE initiates the Mobility Registration Update procedure to activate the associated PDU session(s) as specified in clause 4.2.2.2.2 of TS 23.502 [6]. Hence the shared delivery (if not already established) or the individual delivery can be established towards the NG-RAN node to enable delivery of multicast MBS session data to the UEs.

NOTE:	The Connection resume procedure of Clause 7.2.3.7 also applies as part of the registration procedure for RRC_INACTIVE UEs.

Editor's note:	It is FFS whether and how MBS session information and MBS assistance information is provided when the UE in RRC_IDLE state moves out of the RA and performs Mobility Registration Update procedure after RRC connection setup.

* * * End of Changes * * * *