3GPP TSG-SA WG2 Meeting #155		S2-2303485

# Athens, Greece Februrary 20 – 24, 2023									   (revison of S2-2302848)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0714 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to FL procedure related to the maximum response time |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, SK Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It was agreed in S2-2301460 that the Server NWDAF may provide the Client NWDAF with the maximum response time for the Client NWDAF to provide the interim local ML model information during the Federated Learning procedure based on the TR conclusion. In this regard, it needs to be clarified how the Server NWDAF can decide such a time value. Accordingly, it should be clarified how the Server and Client NWDAFs handle the case, respectively, where some of the Client NWDAF cannot complete the local ML model training and provide the interim local ML model within the given maximum response time. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates to FL procedure regarding the maximum response time for Client NWDAFs - How the Server NWDAF determines the maximum response time for FL Client based on the time when the analytics is needed - How the Client and Server NWDAF operates when the Client NWDAF cannot provide the interim local ML model within the given maximum response time |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The operations of FL Server and Client NWDAFs remain unclarified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2C.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

6.2C	Federated Learning among Multiple NWDAFs

6.2C.1	Description

This clause specifies how NWDAF containing MTLF can leverage Federated Learning technique to train an ML model, in which there is no need for input data transfer (e.g. centralized into one NWDAF) but only need for cooperation among multiple NWDAFs (MTLF) distributed in different areas i.e. sharing of ML model(s) and of the learning results among multiple NWDAFs (MTLF).

6.2C.2	Procedures

6.2C.2.1	General procedure for Federated Learning among Multiple NWDAF Instances

Figure 6.2C.2.1-1: General procedure for Federated Learning among Multiple NWDAF

0.	The consumer (NWDAF containing AnLF) sends a subscription request to NWDAF containing MTLF to retrieve a ML model, including Analytic ID and ML model filter information as described in TS 23.288 [5], the NWDAF containing MTLF can be a FL server (Server NWDAF) with FL server capability or a MTLF without FL server capability.

Editor's note:	Procedure of MTLF registration and discovery with respect to FL is FFS.

Editor's note:	Definition of FL capability is FFS.

If the consumer (i.e., the NWDAF containing AnLF or the consumer of the NWDAF containing AnLF) provides the time when the analytics information is needed, the Server NWDAF can take this information into account to decide the maximum response time for its Client NWDAFs.

1.	Server NWDAF sends a request to the selected NWDAF containing MTLF(Client NWDAF) that participates in the Federated learning to perform the local model training for Federated Learning. The request also includes the maximum response time before which the Client NWDAF has to report the interim local ML model information to the Server NWDAF.

2.	Each Client NWDAF collects its local data by using the current mechanism in clause 6.2 of TS 23.288 [5].

3.	During Federated Learning training procedure, each Client NWDAF further trains the retrieved ML model from the server NWDAF based on its own data, and reports interim local ML model information to the Server NWDAF.

The ML model information are exchanged between the Client NWDAF(s) and the Server NWDAF during the FL training process.

Editor's note:	The ML model information exchanged between the Client NWDAF(s) and the Server NWDAF is FFS.

Editor's note:	The services in step 1, 3, 5a and 6 to enable FL based ML model training is for FFS, and the Figure 6.2C.2.1-1 will be updated accordingly.

If the Client NWDAF is not able to complete the training of the interim local ML model within the maximum response time provided by the Server NWDAF, the Client NWDAF notifies the Server NWDAF on the delay event with proper cause information (e.g., local ML model training failure, more time necessary for local ML model training) before the maximum response time elapses.

4.	The Server NWDAF aggregates all the local ML model information retrieved at step 3, to update the global ML model.

If the Server NWDAF provides the maximum response time for the Client NWDAFs to provide the interim local ML model information in step 1, the Server NWDAF decides either to wait for the Client NWDAFs which have not yet provided their interim local ML model within the maximum response time or aggregates only the retrieved local ML model information instances to update global ML model. The Server NWDAF makes this decision, considering the notification from the Client NWDAF or, if the notification is not received, based on local configuration.

Editor's note:	it is FFS whether Server NWDAF solicits a response from an FL clients which has not reported the FL status within the maximum response time.

5a.	Based on the consumer request, the Server NWDAF updates the training status (an accuracy level) to the consumer periodically (one or multiple rounds of training or every 10 min, etc.) or dynamically when some pre-determined status (e.g. some accuracy level) is achieved.

5b.	[Optional] Consumer decides whether the current model can fulfil the requirement e.g. accuracy and time. The consumer modifies subscription if the current model can fulfil the requirement.

Editor's note:	Further details on providing the accuracy of FL model process is for FFS.

5c.	According to the request from the consumer, Server NWDAF updates or terminates the current FL training process.

6.	If the FL procedure continues, Server NWDAF sends the aggregated ML model information to each Client NWDAF for next round model training.

7.	Each Client NWDAF updates its own ML model based on the aggregated ML model information distributed by the Server NWDAF at step 6.

NOTE:	The steps 2-7 should be repeated until the training termination condition (e.g. maximum number of iterations, or the result of loss function is lower than a threshold) is reached.

After the training procedure is complete, the Server NWDAF may send the globally optimal ML model information to the consumer.

* * * * End of Changes * * * *