3GPP TSG-SA2 Meeting #155 	S2-2303534

# Athens, Greece, Feb 20th – 24th, 2023	                                       (revision of S2-2303486)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0604 | rev | 6 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support the Maintenance of Federated Learning Process in 5GC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-02-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Principle 5 and NOTE 2 listed in the conclusion in clause 8.8 of TR 23.700-81 as follows: “During the FL procedure, the NWDAF containing MTLF as FL server may trigger reselection, addition or removal of FL clients and may issue a new FL client discovery via NRF, based on local policy or status of FL clients, e.g. load, availability, capability, latency, accuracy, etc. FL clients can join or leave FL operation dynamically in the execution phase.” “NWDAF containing MTLF as FL Server can monitor the accuracy level of the trained model and exclude a Client NWDAF from the FL group …” Besed on these conclusions, the procedures to support maintenance of Federated Learning in FL execution phase are added to TS 23.288, including FL server triggers reselection, addition or removal of FL client(s), discovers FL client(s) via NRF, and FL client joins or leaves FL operation dynamically. SA2#155: There are two ENs left in SA2#154AH Editor’s Note: The details of the judgement process is FFS. Editor’s Note: The further actions by Client NWDAF is FFS. The clarification is needed to explain how FL server NWDAF determines the reselection of FL client NWDAF and what actions the client NWDAF can do if it is not involved in the FL procedure anymore. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add procedure to TS 23.288 to support FL server to reselect FL client(s), discover FL client(s) via NRF, and to support FL client to join or leave Federated Learning operation dynamically. SA2#155: Resolve two ENs Editor’s Note: The details of the judgement process is FFS. Editor’s Note: The further actions by Client NWDAF is FFS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedures for support maintenance of Federated Learning process in FL execution phase will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2C.2.Y(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

#### 6.2C.2.Y Procedures for Maintaining  Federated Learning Procedures

This clause specifies how to maintain a Federation Learning process in FL execution phase, including Server NWDAF triggers reselection, addition or removal of Client NWDAF(s), discovers new Client NWDAF(s) via NRF, and Client NWDAF(s) joins or leaves Federated Learning operation dynamically.

In Federated Learning execution phase, Server NWDAF monitors the status changes of Client NWDAF(s), and may reselects Client NWDAF(s) based on the updated status, availability, and/or capability, etc.

NOTE 1: Server NWDAF checks if there is a need to carry on the FL execution phase and then reselects FL members for the next iteration if needed.

Figure 6.2C.2.Y-1: Procedure of Server NWDAF reselects Client NWDAF(s), Client NWDAF(s) Join or Leave Federated Learning Operation Dynamically in Federated Learning execution phase

The procedure for Server NWDAF reselecting Client NWDAF(s), Client NWDAF(s) joining or leaving Federated Learning operation dynamically is as follows:

0.	Server NWDAF registers into NRF about the Federated Learning procedure with the following parameters:

-	Analytics ID.

1a.	Server NWDAF may get the updated status of current Client NWDAF(s) via NRF by using Nnrf_NFManagement service (as in clause 5.2.7.2 of TS 23.502) in the Federated Learning execution phase.

Server NWDAF may subscribe to NRF for notifications of status changes of the current NWDAF(s) (Client NWDAFs 1…N) by invoking an Nnrf_NFManagement_NFStatusSubscribe service operation. NRF notifies the Server NWDAF the status changes of the current Client NWDAF(s) by invoking Nnrf_NFManagement_NFStatusNotify service operation(s).

The status of a current Client NWDAF could be availability changes, capability changes (e.g., it will not support FL anymore, etc.).

1b. The current Client NWDAF(s) may inform Server NWDAF to leave the Federated Learning operation with cause code (reason for leaving, e.g., high NF load, time availability changes).

1c. Server NWDAF may get the information of the new Client NWDAF(s) dynamically via NRF by subscribing to the event that a new Client NWDAF registers (Nnrf_NFManagement _NFStatusSubscribe service as in clause 5.2.7.2 of TS 23.502).

1d. NWDAF may subscribe for NF load analytics of the Client NWDAF(s).

1e. Client NWDAF(s) may report status of FL training including accuracy of local model and Training Input Data Information (e.g., areas covered by the data set, sampling ratio, maximum/minnum of value of each dimention, etc).

Editor’s note:	It is FFS whether and what and how (i.e. using which service) Training Input Data Information is provided from the Client NWDAF to the Server NWDAF.

2.	Server NWDAF checks Client NWDAF(s) status based on the received information, and may determinewhether reselection of Client NWDAF(s) for the next round(s) of Federated Learning is needed based on the received information from step 1.

NOTE 1:	 Several examples of the factors that the Server NWDAF can consider to reselect the Client NWDAF(s) are:

updated status of Client NWDAF reported by NRF is different than the criteria were initially used for selecting the client;

characteristics of local training dataset is different than global validation dataset and accuracy of the local model is lower than the accuracy of the global model;

the load of the Client NWDAF (from the NF load Analytics or from the client NWDAF directly) is high;

the Server NWDAF receives leave request from the Client NWDAF;

the Client NWDAF reports the status of FL Training within the maximum response time.

3.	[If re-selection is needed as judged in step 2.] If step 1c is not performed, Server NWDAF may discover new candidate Client NWDAF(s) via NRF by using Nnrf_NFDiscovery services as in clause 5.2.7.3 of TS 23.502. Server NWDAF reselects Client NWDAF(s) from the current Client NWDAF(s) and the new candidate Client NWDAF(s) (found in steps 1c or 3). For the new candidate Client NWDAF(s), the interaction between Server NWDAF and Client NWDAF(s) is same as the selection procedure described in clause 6.2C.2.1. The adding / deleting Client NWDAF(s) may happen at the end of each iteration.

4.	Server NWDAF sends termination request to the Client NWDAF(s) which will be removed from the Federated Learning process optionally indicating the reason. Client NWDAF(s) terminates Federated Learning operations if receive termination request from the Server NWDAF and may performe further action to be qualitifed in participation of FL training in the next cycles.

NOTE 2:	In the case of high load, the Client NWDAF can e.g., decline new service request. In the case of low accuracy, the Client NWDAF can gather new local training data.

Editor’s Note: What service will be used by Server and Client NWDAFs to terminate Federated Learning operation is FFS.

* * * End of Changes * * *