SA WG2 Meeting #155	S2-2302947

Athens, Greece, Feb 20 – 24, 2023	(revision of S2-2301767)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3956 | rev | 6 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Transfer of Satellite Coverage Data to a UE and AMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Qualcomm Incorporated], Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The conclusions for FS_5GSAT_Ph2 in TR 23.700-28, include supporting the the transfer of satellite coverage area information to a UE and AMF. This CR introduces the support as proposed in a comparison of alternative solutions in S2-2300668. The external server providing the satellite coverage area information to the UE computes the satellite coverage area information in the target area based on satellite orbit data. Similarily, the AF or the O&M provisions to the AMF the satellite coverage area information that is computed based on satellite orbit data. UE may use the provisioned satellite coverage area to determine the power saving parameters that it includes in NAS signalling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add support for transfer of satellite coverage area data to a UE and to an AMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Transfer of enhanced satellite coverage data to a UE compared to support for EPS in Release 17 and to an AMF would not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.X.Y (new), 5.4.X.Z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change (all new) ****

5.4.X Support of discontinuous network coverage

#### 5.4.X.Y UE coverage information provisioning to the UE

A UE may use satellite coverage area information for satellite access to support discontinuous coverage operations. Coverage information can be provided to a UE by an external server by user plane or control plane in case NIDD is used.

Editor’s Note: 	It is FFS whether only UP approach is sufficient or NIDD is also to be supported.

#### 5.4.X.Z UE Coverage information provisioning to the AMF

The AMF may use satellite coverage area information to support satellite access by UEs with discontinuous coverage operation. O&M or AF may determine satellite coverage area based on satellite orbit information and provision the satellite coverage area to the AMF.

Editor’s Note: 	It needs to be determined whether both sources (i.e. O&M and AF) of satellite coverage area will be supported or just one of them.

Editor's Note:	What NEF exposure procedure can be re-used for provisioning satellite coverage area from the AF is FFS.

**** End of Changes ****