3GPP TSG-WG SA2 Meeting #155 	S2-2303006

February 20 – 24, 2023, Athens, Greece	(revision of S2-220xxxx)

Source:	Philips International B.V.

Title:	High Level Description for Located UE Discovery and Selection

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract: This paper proposes a high level description for Located UE Discovery & Selection for TS 23.586.

# 1. Introduction/Discussion

This paper proposes a high level description for Located UE Discovery & Selection for TS 23.586.

# 2. Text Proposal

It is proposed to capture the following new clause 5.X.Y (e.g. 5.2.2 as in draft table of contents) for TS 23.586.

# * * * * Start of changes * * * *

## 5.X.Y	Located UE Discovery & Selection

The discovery of Located UEs follows the same principles as specified in clause 5.2.1. The UE can indicate its role “Located UE” in its list of supported roles during discovery, if it is authorized to be a Located UE in a given PLMN as per the Authorization and Provisioning for Ranging/SL positioning service as specified in clause 5.1.

Depending on the privacy profile of a Located UE, a Located UE may be willing to share its location publicly, or only to authorized UEs (e.g. Target UE, other Reference UEs, SL positioning server UE) or only to the LMF. The UE can indicate that in the location privacy field during discovery.

A Target UE can select one or more Located UEs (and other Reference UEs) to be used in a Ranging/SL positioning procedure (as specified in clauses 5.3 through 5.5), based on the discovered information, the coverage situation, a received Ranging/SL positioning Request and/or pre-configured Ranging/SL positioning policy/parameters.

Editor’s Note: It is FFS if the Target UE can be provisioned with information (e.g. User Info IDs) about a set of known Located UEs in an area to perform targeted discovery, e.g. through pre-configuration or receiving this information through a Ranging/SL positioning Request.

Editor’s Note: It is FSS whether an indication of whether a Located UE is static/mobile needs to be provided during discovery, similar to the discussion on PRUs in TS 23.273.

NOTE: 	The role of being “Located UE” is dynamic and may change over time, in particular if the Located UE is moving. Hence, the discovery results need to be refreshed if there is a (significant) delay between discovery and initiating of a ranging procedure with a discovered Located UE.

NOTE 2: 	The criteria to determine if the location of a UE is sufficiently stable and accurate in order for the UE to announce itself as a “Located UE” can be determined by RAN WGs.

# * * * * End of changes * * * *