3GPP TSG-WG SA2 Meeting # 156E	S2-2304362

# Elbonia, April 17-21, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0141 | rev | 3 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Public Warning Notification Relaying by 5G ProSe UE-to-Network Relay. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sony, LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introducing the Support of the Relaying of Public Warning Notification and warning message according to the requirement specified in TS 22.268. Aligning SA2 specification with RAN Rel-17 solution for forwarding PWS SIBs over unicast link to Remote UE(s) as specified in TS 38.300. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Describe when a UE that operate as a 5G ProSe UE-to-Network Relay receives a warning message, the UE shall broadcast the warning message to the remote UE(s). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Relay of Public Warning Notification is not supported for L3 Relays and unconnected Remote UEs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.1.4.1, New 5.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** FIRST CHANGE *****

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[3]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

[4]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[5]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[6]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[7]	3GPP TS 22.278: "Service requirements for the Evolved Packet System (EPS)".

[8]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[9]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[10]	Void.

[11]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[12]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[13]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in idle mode".

[14]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station in idle mode".

[15]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[16]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

[17]	IETF RFC 4862: "IPv6 Stateless Address Autoconfiguration".

[18]	IETF RFC 3927: "Dynamic Configuration of IPv4 Link-Local Addresses".

[19]	IETF RFC 826: "An Ethernet Address Resolution Protocol".

[20]	Void.

[21]	3GPP TR 23.752: "Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS)".

[22]	3GPP TS 32.277: "Proximity-based Services (ProSe) charging".

[23]	3GPP TS 24.554: "Proximity-services (ProSe) in 5G System (5GS) protocol aspects; Stage 3".

[24]	IETF RFC 2131: "Dynamic Host Configuration Protocol".

[25]	IETF RFC 4039: "Rapid Commit Option for the Dynamic Host Configuration Protocol version 4 (DHCPv4)".

[26]	Void.

[27]	Void.

[28]	3GPP TS 38.351: "NR; Sidelink Adaptation Layer Protocol".

[29]	3GPP TS 33.503: "Security Aspects of Proximity based Services (ProSe) in the 5G System (5GS)".

[30]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[31]	3GPP TS 23.167: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; IP Multimedia Subsystem (IMS) emergency sessions".

[xx]	3GPP TS 23.041: "Public Warning System".

***** NEXT CHANGE *****

5.1.4.1	Policy/Parameter provisioning for 5G ProSe UE-to-Network Relay

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe UE-to-Network Relay:

1)	Authorisation policy for acting as a 5G ProSe Layer-3 and/or Layer-2 UE-to-Network Relay when "served by NG-RAN":

-	PLMNs in which the UE is authorized to relay traffic for 5G ProSe Layer-3 and/or Layer-2 Remote UEs.

The authorisation for a UE to act as a 5G ProSe UE-to-Network Relay also authorizes the use of 5G ProSe UE-to-Network Relay Discovery with Model A and Model B.

NOTE 1:	It is up to UE and application implementation to select a discovery model or whether to perform both models simultaneously.

2)	ProSe Relay Discovery policy/parameters for 5G ProSe UE-to-Network Relay:

-	Includes the parameters that enable the UE to perform 5G ProSe UE-to-Network Relay Discovery when provided by PCF or provisioned in the ME or configured in the UICC:

-	5G ProSe UE-to-Network Relay Discovery parameters (User Info ID, Relay Service Code(s), UE-to-Network Relay Layer Indicator per RSC, optional Control Plane Security Indicator per RSC). The UE-to-Network Relay Layer Indicator indicates whether the associated RSC is offering 5G ProSe Layer-2 or Layer-3 UE-to-Network Relay service. If the Control Plane Security Indicator is provided for a RSC, then the Control Plane based security procedure as described in clause 5.1.4.3.2 is performed for UE-to-Network Relay Communication for that RSC, otherwise if it is not provided for a RSC, then the User Plane based security procedure as described in clause 5.1.4.3.3 is performed for that RSC. RSC dedicated for emergency service may also be provisioned.

-	Default Destination Layer-2 ID(s) for sending Relay Discovery Announcement and Relay Discovery Additional Information messages and receiving Relay Discovery Solicitation messages;

-	For 5G ProSe Layer-3 UE-to-Network Relay, the PDU Session parameters (PDU Session type, DNN, SSC Mode, S-NSSAI, Access Type Preference) to be used for the relayed traffic for each ProSe Relay Service Code;

-	Includes security related content for 5G ProSe UE-to-Network Relay, see TS 33.503 [29].

NOTE 2:	5G ProSe Relay Discovery policy/parameters can be provided from ProSe Application Server to the 5G ProSe UE-to-Network Relay, except for the Security parameters in bullet 2).

3)	For 5G ProSe Layer-3 UE-to-Network Relay, QoS mapping(s):

-	Each QoS mapping entry includes:

-	a mapping between a 5QI value and a PQI value;

-	a PQI PDB adjustment factor, for the PC5 communication for the 5G ProSe Layer-3 UE-to-Network Relay operation;

-	optional the Relay Service Code(s) associates with the QoS mapping entry.

4)	For 5G ProSe Layer-3 UE-to-Network Relay to relay Ethernet or Unstructured traffic from 5G ProSe Layer-3 Remote UE by using IP type PDU Session:

-	Mapping of ProSe Service(s) to ProSe Application Server address information (consisting of IP address/FQDN and transport layer port number).

5)	Destination Layer-2 ID to broadcast warning messages.

6)	PC5 QoS parameters defined in clause 5.6.1 to broadcast warning messages.

7)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe UE-to-Network Relay discovery and communication.

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe Remote UE and thereby enabling the use of a 5G ProSe UE-to-Network Relay:

1)	Authorisation policy for using a 5G ProSe Layer-3 and/or Layer-2 UE-to-Network Relay:

-	For 5G ProSe Layer-3 Remote UE, indicates whether the UE is authorised to use a 5G ProSe Layer-3 UE-to-Network Relay.

-	For 5G ProSe Layer-2 Remote UE, indicates the PLMNs in which the UE is authorized to use a 5G ProSe Layer-2 UE-to-Network Relay.

The authorisation for a UE to act as a 5G ProSe Remote UE also authorizes the use of 5G ProSe UE-to-Network Relay discovery with Model A and Model B.

NOTE 3:	It is up to UE and application implementation to select a discovery model or whether to perform both models simultaneously.

2)	Policy/parameters for 5G ProSe UE-to-Network Relay Discovery:

-	Includes the parameters for 5G ProSe Relay Discovery and for enabling the UE to connect to the 5G ProSe UE-to-Network Relay after discovery when provided by PCF or provisioned in the ME or configured in the UICC:

-	5G ProSe UE-to-Network Relay Discovery parameters (User Info ID, Relay Service Code(s), UE-to-Network Relay Layer indicator per RSC, optional Control Plane Security Indicator per RSC). The UE-to-Network Relay Layer Indicator indicates whether the associated RSC is offering 5G ProSe Layer-2 or Layer-3 UE-to-Network Relay service. If the Control Plane Security Indicator is provided for a RSC, then the Control Plane based security procedures as described in clause 5.1.4.3.2 is performed for UE-to-Network Relay Communication for that RSC, otherwise if it is not provided for a RSC, then the User Plane based security procedures as described in clause 5.1.4.3.3 is performed for that RSC. RSC(s) dedicated for emergency service may be provisioned to enable the support of emergency services for UE-to-Network Relaying.

-	Default Destination Layer-2 ID(s) for sending Relay Discovery Solicitation messages and receiving Relay Discovery Announcement and Relay Discovery Additional Information messages;

-	For 5G ProSe Layer-3 UE-to-Network Relay, the PDU Session parameters (PDU Session type, DNN, SSC Mode, S-NSSAI, Access Type Preference) to be used for the relayed traffic without using N3IWF access, or an indication of N3IWF access, for each ProSe Relay Service Code;

-	For 5G ProSe Layer-3 UE-to-Network Relay, optionally the ProSe application Traffic Descriptor(s) (as defined in TS 23.503 [9]) to be used for the relayed traffic for each ProSe Relay Service Code;

-	Includes security related content for 5G ProSe UE-to-Network Relay, see TS 33.503 [29].

3)	Policy/parameters for N3IWF selection for 5G ProSe Layer-3 Remote UE:

-	N3IWF identifier configuration for 5G ProSe Layer-3 Remote UE (either FQDN or IP address) in the HPLMN.

-	5G ProSe Layer-3 UE-to-Network Relay access node selection information - a prioritized list of PLMNs for N3IWF selection. It also indicates if selection of an N3IWF in a PLMN should be based on Tracking Area Identity FQDN or on Operator Identifier FQDN.

NOTE 4:	5G ProSe Relay Discovery policy/parameters can be provided from ProSe Application Server to the 5G ProSe Remote UE, except for the Security parameters in bullet 2).

4)	Destination Layer-2 ID to receive warning messages.

5)	PC5 QoS parameters defined in clause 5.6.1 to receive warning messages.

6)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe UE-to-Network Relay discovery and communication.

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe UE-to-Network Relay as well as in the UE in support of the UE assuming the role of a 5G ProSe Remote UE and thereby enabling the use of a 5G ProSe UE-to-Network Relay:

1)	Radio parameters for 5G ProSe UE-to-Network Relay Discovery when the UE is not "served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform 5G ProSe Direct Discovery over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

-	Default PC5 DRX configuration (see TS 38.331 [16]).

2)	Radio parameters for 5G ProSe UE-to-Network Relay communication when the UE is not "served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform 5G ProSe Direct Communication over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

NOTE 5:	The validity time of these radio parameters is the same as the validity time of the Policy/Parameter listed above for 5G ProSe UE-to-Network Relay or 5G ProSe Remote UE.

3)	Policy/parameters related to privacy:

-	A privacy timer value indicating the duration after which the UE shall change each source Layer-2 ID self-assigned by the UE when privacy is required.

***** NEXT CHANGE *****

5.X	Support of Public Warning Notification Relaying

The 5G ProSe UE-to-Network Relay broadcasts the warning message received from the network to the 5G ProSe Remote UE(s) by using Broadcast mode of 5G ProSe direct communication as specified in clause 5.3.2. The 5G ProSe UE-to-Network Relay uses a configured Destination Layer-2 ID as specified in clause 5.1.4.1 when broadcasting the warning message and the 5G ProSe Remote UE receives warning messages broadcasted over PC5 reference point by using a configured Destination Layer-2 ID as specified in clause 5.1.4.1. The PC5 QoS parameters as specified in clause 5.1.4.1 are used to broadcast and receive the warning message for the 5G ProSe UE-to-Network Relay and the 5G ProSe Remote UE, respectively.

A 5G ProSe Remote UE can receive the broadcasted warning message independent if the 5G ProSe Remote UE is connected to the 5G ProSe UE-to-Network Relay or not.

The 5G ProSe UE-to-Network Relay suppresses received duplicated warning messages i.e., does not broadcast these over PC5 reference point.

5G ProSe Layer-2 UE-to-Network Relay can alternatively forward the PWS SIBs to a connected 5G ProSe Layer-2 Remote UE over the unicast link as specified in TS 38.300 [12].

The Public Warning System architecture for 5G System is specified in TS 23.041 [xx].

***** END OF CHANGES *****