3GPP TSG-SA WG2 Meeting #157 	S2-2306889

MAY 22 – 26, 2023, Berlin, Germany	(was )

Source:	vivo

Title:	TS 23.586: Procedures for UE based SL Positioning for UE without NAS connection

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract:  This paper tries to add Procedures for UE based SL Positioning for UE without NAS connection.

1. Introduction/Discussion

This paper adds the Procedures for UE based SL Positioning for UE without NAS connection based on the high level function descriptions in clause 5.5.5

### 5.5.5	UE based SL Positioning for UE without NAS connection

For UE based SL Positioning for UE without NAS connection, the procedure is similar to that of the Network based SL Positioning for UE without NAS connection (in clause 5.5.3), with the following exceptions:

-	The LMF may decide to choose a SL Positioning Server UE, e.g. Located UE, for result calculation, as described in clause 5.3.1.

-	The LMF may provide assistance information to the SL Positioning Server UE to assist the Sidelink Positioning result calculation.

Editor's note:	The contents of the assistance information will be defined based on RAN WG conclusions.

2. Text Proposal

It is proposed to capture the following changes to TS 23.586.

# * * * * First change * * * *

### 6.5.4	Procedures for UE based SL Positioning for UE without NAS connection

The Procedures for UE based SL Positioning for UE without NAS connection takes the Procedures for SL positioning for target UE without NAS connection using Located UE with network connection in clause 6.5.2 as baseline with following exceptions:

- In step6 for the Located UE performed SL-MO-LR procedure:

-	the LMF may decide a SL Positioning Server UE (e.g., one of the reported Located UE with SL Positioning Server UE capabilities or a standalone SL Positioning Server UE) for SL Positioning result calculation. The LMF provides the assistance data and/or the location calculation assistance data to the SL Positioning Server UE and requests for the SL Positioning result from the SL Positioning Server UE.

-	SL Positioning Server UE calculates SL Positioning results based on the SL measurements data and the assistance data or location calculation assistance data provided by the LMF. If the SL positioning Server UE is not the Located UE, the LMF needs to additionally provide the SL measurements data between the Located UE and Target UE to the SL positioning Server UE.

-	SL Positioning Server UE returns the SL Positioning result to the LMF.

# * * * * End of changes * * * *