SA WG2 Meeting #S2-157	S2-2307034

# Berlin,Germnay, 22  - 26 May 2023		(revision of S2-230XXXX)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1061 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RT latency requirements for different data flows |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The RT latency requirements of the UL and DL is only supported for a single data flow in current TS. In fact, in the XRM service, the Motion-to-photon latency requirements defined in TS22.261 is the total latency for two data flows, i.e. the UL user input (e.g. user position and or user control command) data flow and DL video stream data flow. |  |  |  |  |  |  |  |  |
| Summary of change: |  | As described in clause 5.37.6 of TS23.501 [2], the policy control of round-trip (RT) latency requirement can also be used on two different data flows. For the two different data flows, AF needs to provide delay requirement between the UE and the PSA UPF expressed as the QoS Reference parameter or individual QoS parameters (as defined in clause 6.1.3.22) for each data flow, and the two data flows can have different QoS requirements (e.g. different delay), . The PCF then identifies the PDB and assigns the 5QI of each PCC rule for the two data flows. In this case, the RT latency requirement of the two data flows is fulfilled by the sum of the PDB requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RT latency requirements cannot be supported as defined in TS22.261. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.27.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.1.3.27.2	UL/DL policy control based on round-trip latency requirement

The AF may provide the round-trip (RT) latency requirement for an XR or other interactive media services with an RT latency indication via the AF session with required QoS procedure described in clause 6.1.3.22. The RT latency indication indicates the service data flow needs to meet the RT latency requirement of the service, which is twice the single direction delay requirement between the UE and the PSA UPF described by the QoS Reference parameter or individual QoS parameter.

Based on the RT latency requirement received from the AF, the PCF can split the RT latency requirement into two PDBs of two PCC rules, used for the UL QoS Flow and DL QoS Flow to carry the UL and DL traffics of the service respectively. The two PDBs can be unequal, but their sum shall not exceed the RT latency requirement.NOTE 1:	RT latency requirement may also be locally configured in the PCF together with delay requirement.

To enable RT latency tracking, the PCF shall generate associated QoS monitoring policies for the two correlated QoS Flows. The uplink and downlink delay for the two QoS Flows shall be tracked by PCF independently with same reporting period.

When the QoS monitoring results are reported to PCF, the PCF can derive and track the RT latency by combining the QoS monitoring reports for the UL and the DL packet delay. The PCF may adjust the PDBs of the two PCC rules using SM Policy Association Modification procedure described in clause 4.16.5.2 of TS 23.502 [3] to better fit the new situation.

NOTE 2:	How the PCF derives the round-trip latency and takes policy decisions is up to implementation.

As described in clause 5.37.6 of TS23.501 [2], the policy control of round-trip (RT) latency requirement can also be used on two different data flows. For the two different data flows, AF needs to provide delay requirement between the UE and the PSA UPF expressed as the QoS Reference parameter or individual QoS parameters (as defined in clause 6.1.3.22) for each data flow, and  the two data flows can have different QoS requirements (e.g. different delay). The PCF then identifies the PDB and assigns the 5QI of each PCC rule for the two data flows. In this case, the RT latency requirement of the two data flows is fulfilled by the sum of the PDB requirements.

# * * * * End of changes * * * *