SA WG2 Meeting #S2-158	S2-2308350

21 - 25 August, 2023, Goteborg, Sweden

3GPP TSG-SA3 Meeting #112 	S3-233786

Goteborg, Sweden, 14 - 18 August 2023

Title:	LS-Reply on Introduction of the domain "ipxnetwork.org"

Response to:	LS S3-233532/5GMRR Doc 42a_04r2 on LS to 3GPP on the introduction of the domain “ipxnetwork.org” in addition to “3gppnetwork.org” from GSMA 5GMRR

Release:	Rel-18

Work Item:

Source:	3GPP SA WG3

To:	GSMA 5GMRR, 3GPP WG SA1

Cc:	3GPP WGs SA2, CT4

Contact person:	Anja Jerichow (Nokia)

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1	Overall description

SA3 thanks 5GMRR for their LS to 3GPP on the introduction of the domain "ipxnetwork.org" in addition to "3gppnetwork.org".

SA3 acknowledges that the domain "ipxnetwork.org" has been introduced by GSMA for specific pre-5G roaming use cases. The 5G architecture as currently specified, however, stipulates that for any given roaming relation, exactly two SEPPs are involved, and that they mutually authenticate each other using a combination of the PLMN ID of the operator whose network edge they represent, and the domain name "3gppnetwork.org".

While it is in principle possible to use a different domain suffix like "ipxnetwork.org" for SEPP discovery purposes in the context of intermediaries, such discovery and SEPP identification may have security implications. Given that an anticipated SA1 study will cover scenarios involving intermediaries, it seems prudent to await the results of this study before updating specifications with respect to introducing domain names for intermediary discovery.

SA3 believes that a SEPP discovery procedure that covers all deployment scenarios in a manner that preserves e2e security between roaming partners is required, and kindly asks GSMA to provide further details on the mechanisms already documented.

SA3 would also like to get confirmation on the following: If an operator decides to use a Hosted SEPP managed by a service provider instead of operating its own SEPP, then the Hosted SEPP’s service provider would become the edge of one PLMN and, hence, the Hosted SEPP should become part of the PLMN security domain. If this is the correct understanding, an operator should also allow the IPX provider to use the operator domain name to connect to the other PLMN’s SEPP to fulfil the SEPP behaviour as specified in clause 5.9.3 of 33.501.

# 2	Actions

To GSMA 5GMRR, 3GPP SA1, SA2, CT4

ACTION:

SA3 asks 5GMRR to confirm the above, provide further details on the GSMA discovery mechanisms and to confirm that, for any given roaming relation, exactly two SEPPs representing the edges of PLMNs are involved.

SA3 asks SA1, SA2 and CT4 to take the information into account.

# 3	Dates of next TSG SA WG 3 meetings

SA3#113	6 -10 November 2023	Chicago, US

SA3#114	22 -26 January 2024	EU (TBD)