3GPP SA WG2 Meeting #158	S2-2308404

21 - 25 Aug 2023, Gothenburg, Sweden	(revision of S2-23xxxxx)

Source:	Intel

Title:	On the use of Namf_Communication_N1N2MessageTransfer instead of Namf_MT_EnableUEReachability service operation

Document for:	Approval

Agenda Item:	7.4

Work Item / Release:	5G_CIoT / Rel-16

Abstract of the contribution: The contribution argues that the Namf_Communication_N1N2MessageTransfer service can be used in all cases where the Namf_MT_EnableUEReachability service is used.

# 1	Introduction

*************************************************************************************

The discussion in this contribution should be considered together with the discussion in S2-2309253. The discussion has been split in two parts for the reader’s convenience.

*************************************************************************************

This contribution points out that the use of Namf_MT_EnableUEReachability service partly overlaps with the use of the Namf_Communication_N1N2MessageTransfer service and can be used only in a subset of scenarios that are covered by the latter. Actually, the authors argue that the specification today is not very clear about the logic that the service consumer should follow to determine which of these two services to invoke. In certain cases the use of the Namf_MT_EnableUEReachability service seems redundant (e.g. MT SMS). The authors conclude that the Namf_Communication_N1N2MessageTransfer service can be used in all cases where the Namf_MT_EnableUEReachability service is used.

# 2	Discussion

Figure 1 illustrates the NW-initiated procedure for connection resumption for UE in RRC_INACTIVE state. It is noted that in step 1a.2 the SMF invokes the Namf_MT_EnableUEReachability service.

Figure 1 (Figure 4.8.2.2b-1): Network Triggered Connection Resume for UE in RRC_INACTIVE with CN based MT communication handling

The procedure in Figure 1 is for UE in RRC_INACTIVE state and was modelled based on the procedure for Data Transport in Control Plane CIoT 5GS Optimization, as indicated by the pointer to step 2 in TS 23.502 Figure 4.24.2 (reproduced below as Figure 2):

1a.	When downlink data is received and the SMF/UPF is requested to perform buffering as specified in clause 4.8.1.1a, the UPF/SMF checks with AMF for the possibility of data delivery, similar to step 2 of clause 4.24.2 with the following differences:

Figure 2 illustrates the NW-initiated procedure for connection resumption for UE in RRC_IDLE state (TS 23.502 Figure 4.24.2) and was defined as part of Control Plane CIoT 5GS Optimizations. It is worth noting that in step 2 the SMF invokes either the Namf_MT_EnableUEReachability service (step 2c) or the Namf_Communication_N1N2MessageTransfer service (step 2g), depending whether the MT data buffering is performed in the UPF or in the SMF.

Figure 2 (Figure 4.24.2-1): Mobile Terminated Data Transport in Control Plane CIoT 5GS Optimisation

It is not clear to the authors why the SMF invokes a different service operation in step 2, depending whether the buffering is in the SMF or in the UPF. One might think that the guiding principle for such a design was that the SMF should invoke the Namf_Communication_N1N2MessageTransfer service when it has a data container (i.e. either a NAS message or a data packet in case of CIoT optimizations with buffering in the SMF), whereas the Namf_MT_EnableUEReachability service is invoked when there is no data container to convey via the Control plane (e.g. when MT data is buffered in the UPF).

However, this logic would be in contradiction with the basic (Rel-15) network-triggered Service Request procedure (refer to Figure 3), which does rely on the Namf_Communication_N1N2MessageTransfer message in step 3a, even though there is no data container to convey via NAS.

Observation 1: The logic for choosing between the Namf_MT_EnableUEReachability service or the Namf_Communication_N1N2MessageTransfer service is not clearly defined today.

In fact, the introductory text in TS 23.502 clause 4.2.3.3 explains that the remainder of the NW-triggered Service Request procedure starting with the Paging message (step 4b) could also be considered when the procedure is triggered by the Namf_MT_EnableUEReachability service operation, which further blurs the difference between these two service operations.

-	NF (e.g. SMSF or SMF) triggers AMF, using the Namf_MT_EnableUEReachability service operation, to setup a NAS connection with the UE and the UE is in CM-IDLE state: The trigger is specific to the procedure and Step 4b (paging) occurs.

Figure 3 (Figure 4.2.3.3-1): Network Triggered Service Request

Another interesting point is that the Namf_MT_EnableUEReachability service can only be used to trigger paging, but it needs to be followed on with invocation of the Namf_Communication_N1N2MessageTransfer service in case there is a data container (e.g. a NAS message or a user plane packet in case of CIoT optimizations with buffering in the SMF) to be delivered via NAS. In contrast, the Namf_Communication_N1N2MessageTransfer service can in principle be invoked in all cases, regardless whether there is a data container to be delivered via NAS. This means that the Namf_Communication_N1N2MessageTransfer service could in principle be invoked in all cases where the Namf_MT_EnableUEReachability service is used, but not the other way round.

Observation 2: The Namf_Communication_N1N2MessageTransfer service could in principle be invoked in all cases where the Namf_MT_EnableUEReachability service is used, but not the other way round.

The authors further note here that in the case of MT SMS the stage-2 call flow (refer to Figure 4) indicates that the consumer function (SMSF in this case) invokes both services i.e. first the SMSF invokes the Namf_MT_EnableUEReachability service (step 4a) and only after UE responds to paging, the SMSF invokes the Namf_Communication_N1N2MessageTransfer service in step 5. In the authors’ view the step 4a is redundant and stage-3 procedures are defined in such a way that the SMSF can directly invoke the Namf_Communication_N1N2MessageTransfer service in step 4 of the call flow.

Observation 3: In case of MT SMS the use of Namf_MT_EnableUEReachability service is redundant.

Figure 4 (Figure 4.13.3.6-1): MT SMS over NAS in CM-IDLE state via 3GPP access

Based on the discussion so far, the authors propose that the invocation of the Namf_Communication_N1N2MessageTransfer service should be generalized for all cases of MT communication. Specifically, it should be possible to use the Namf_Communication_N1N2MessageTransfer service in all cases for MT communication where today’s stage-2 description uses the Namf_MT_EnableUEReachability service.

Proposal 1: Generalize the invocation of the Namf_Communication_N1N2MessageTransfer service for MT communication in conjunction with extended buffering or extended DRX cycle.

To minimize the specification text changes, it is proposed to add NOTEs in TS 23.502 stating that Namf_Communication_N1N2MessageTransfer service can also be used where the call flow indicates use of the Namf_MT_EnableUEReachability service.

A companion set of three 23.502 CRs (for Rel-16/17/18) implement the proposal outlined in this discussion paper, in addition to other corrections:

S2-2308405: “Correction to UPF anchored Mobile Terminated Data Transport in Control Plane CIoT 5GS Optimisation”, 23502CR4253 (Type F; Rel-16)

S2-2308406: “Correction to UPF anchored Mobile Terminated Data Transport in Control Plane CIoT 5GS Optimisation”, 23502CR4254 (Type A; Rel-17)

S2-2308407: “Correction to UPF anchored Mobile Terminated Data Transport in Control Plane CIoT 5GS Optimisation”, 23502CR4255 (Type A; Rel-18)

Proposal 2: Agree the companion set of three CRs (for Rel-16/17/18) implementing the proposal outlined in this discussion paper.

# 4	Proposal

Observation 1: The logic for choosing between the Namf_MT_EnableUEReachability service or the Namf_Communication_N1N2MessageTransfer service is not clearly defined today.

Observation 2: The Namf_Communication_N1N2MessageTransfer service could in principle be invoked in all cases where the Namf_MT_EnableUEReachability service is used, but not the other way round.

Observation 3: In case of MT SMS the use of Namf_MT_EnableUEReachability service is redundant.

Proposal 1: Generalize the invocation of the Namf_Communication_N1N2MessageTransfer service for MT communication in conjunction with extended buffering or extended DRX cycle.

Proposal 2: Agree the companion set of three CRs (for Rel-16/17/18) implementing the proposal outlined in this discussion paper.