3GPP SA WG2 Meeting #158	S2-2308803

21-25 Aug 2023, Goteborg, Sweden                  	(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0314 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on 5G ProSe Authorised Information for NG_RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the S2-2308317/R2-2306889 Reply LS on ProSe Authorization information related to UE-to-UE Relay operation from RAN2, the following answers are given: SA2 Question 1: Whether the "5G ProSe authorised" information needs to be enhanced to include the authorization information for UE-to-UE Relay operation? RAN2’s Feedback: RAN2 achieved the following agreements for UE-to-UE Relay operation: Authorization for L2 U2U relay operation is supported. Authorization for L2 U2U relay operation includes: 1) whether the UE is authorized to act as a 5G ProSe Layer-2 U2U Relay UE; 2) whether the UE is authorized to act as a 5G ProSe Layer-2 U2U Remote UE. Authorization for L3 U2U relay operation is not supported. The legacy authorization for “5G ProSe Direct discovery” and “5G ProSe Direct communication” can be reused for L3 U2U remote/relay UE.. So the answer is yes for L2 UE-to-UE Relay operation. The "5G ProSe authorised" information needs to be enhanced to include the authorization information for L2 UE-to-UE Relay operation from RAN2 perspective. SA2 Question 2: If the answer to Q1 is yes, which bullet(s) need to be included? RAN2’s Feedback The following two bullets should be included: - whether the UE is authorized to act as a 5G ProSe Layer-2 UE-to-UE Relay; - whether the UE is authorized to act as a 5G ProSe Layer-2 U2U UE. Based on the feedback from RAN2, it is proposed to include 5G ProSe authorised Information for 5G ProSe Layer-2 UE-to-UE Relay operations in the NGAP message sent to NG-RAN. |
| Summary of change: |  | Add the following items in 5G ProSe authorised Information for NG_RAN: - whether the UE is authorized to act as a 5G ProSe Layer-2 UE-to-UE Relay - whether the UE is authorized to act as a 5G ProSe Layer-2 End UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 5G ProSe authorised Information for Layer-2 UE-to-UE Relay operation sent to NG_RAN is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

6.6.2	Registration procedure

The Registration procedure for UE is performed as defined in TS 23.502 [5] clause 4.2.2.2 with the following additions:

-	The UE includes the 5G ProSe Capability as part of the "5GMM capability" in the Registration Request message. The AMF stores the 5G ProSe Capability for 5G ProSe operation.

-	The 5G ProSe Capability indicates whether the UE supports one or more of the following ProSe capabilities:

-	5G ProSe Direct Discovery;

-	5G ProSe Direct Communication;

-	5G ProSe Layer-2 UE-to-Network Relay;

-	5G ProSe Layer-3 UE-to-Network Relay;

-	5G ProSe Layer-2 Remote UE;

-	5G ProSe Layer-3 Remote UE;

-	5G ProSe Layer-2 UE-to-UE Relay;

-	5G ProSe Layer-3 UE-to-UE Relay;

-	5G ProSe Layer-2 End UE; and

-	5G ProSe Layer-3 End UE.

-	The AMF obtains the 5G ProSe subscription data as part of the user subscription data from UDM during UE Registration procedure using Nudm_SDM service as defined in clause 4.2.2.2.2 of TS 23.502 [5].

-	The AMF determines whether the UE is authorised to use 5G ProSe services based on UE's 5G ProSe Capability and the ProSe Service Authorisation included in the subscription data received from UDM as specified in clause 5.7. ProSe NR UE-PC5-AMBR is also provided to the AMF as part of the subscription data for 5G ProSe services. The AMF stores the authorized 5G ProSe Capability.

-	The AMF sends the authorized 5G ProSe Capability for 5G ProSe operation to PCF. Based on the received 5G ProSe Capability from the AMF, the PCF provides the PC5 QoS parameters for 5G ProSe to AMF. The AMF stores such information as part of the UE context.

-	If the UE is authorised to use 5G ProSe services, then the AMF shall include in a NGAP message sent to NG-RAN:

-	"5G ProSe authorised" information, including one or more of the following:

-	whether the UE is authorized to use 5G ProSe Direct Discovery;

-	whether the UE is authorized to use 5G ProSe Direct Communication;

-	whether the UE is authorized to act as a 5G ProSe Layer-2 UE-to-Network Relay;

-	whether the UE is authorized to act as a 5G ProSe Layer-3 UE-to-Network Relay;

-	whether the UE is authorized to act as a 5G ProSe Layer-2 Remote UE;

-	whether the UE is authorized to act as a 5G ProSe Layer-2 UE-to-UE Relay;

-	whether the UE is authorized to act as a 5G ProSe Layer-2 End UE;

-	whether the UE is authorized to use multi-path communication via direct Uu path and via 5G ProSe Layer-2 UE-to-Network Relay as a 5G ProSe Layer-2 Remote UE.

-	ProSe NR UE-PC5-AMBR, used by NG-RAN for the resource management of UE's PC5 transmission for 5G ProSe services in network scheduled mode.

-	the PC5 QoS parameters for 5G ProSe used by the NG-RAN for the resource management of UE's PC5 transmission for ProSe services in network scheduled mode.

-	If the UE is authorised to use 5G ProSe services, then the AMF should not initiate the release of the signalling connection after the completion of the Registration procedure. The release of the signalling connection relies on the decision of NG-RAN, as specified in TS 23.502 [5].

END OF CHANGES