3GPP TSG-SA2 Meeting #158	S2-2308807

# 21 - 25 August, 2023, Goteborg, Sweden

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0878 | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification for roaming exchange capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In last meeting, it is approved to change roaming entry capability to roaming exchange capability. But there are still some misalignments of that in TS 23.288. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Align the terminology for RE-NWDAF from Roaming Entry NWDAF to Roaming Exchange NWDAF - Clarify the NWDAF in some specific cases is the H-RE-NWDAF or V-RE-NWDAF for roamig exchange. - Refine the figure in 6.1.5.2 and 6.1.5.3 to do terminology alignment. - Some other miscellaneous changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment of RE-NWDAF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3, 6.1.1.1, 6.1.1.2, 6.1.2.1, 6.1.2.2, 6.1.5.1, 6.1.5.2, 6.1.5.3. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

## 4.3	Roaming architecture

Based on operator's policy and local regulations (e.g. privacy), data or analytics may be exchanged between PLMNs (i.e. HPLMN and VPLMN). In a PLMN, an NWDAF is used as entry point to exchange analytics and to collect input data for analytics with other PLMNs. The NWDAF with roaming exchange capability is called Roaming Exchange NWDAF (RE-NWDAF).

Editor's note:	Data and analytics exchange between HPLMN and VPLMN require updates based on feedback from GSMA.

Figure 4.3-1: Roaming Architecture to exchange Input Data or Data Analytics between V-PLMN and H-PLMN

Using the architecture shown in Figure 4.3-1:

-	For outbound roaming users, the NF consumer in the HPLMN can retrieve analytics fromthe VPLMN via the H-RE-NWDAF in HPLMN and V-RE-NWDAF in VPLMN.

NOTE 1:	The analytics from the VPLMN may be generated by the V-RE-NWDAF in the VPLMN or by other NWDAFs in the VPLMN.

-	For outbound roaming users, the H-RE-NWDAF in HPLMN can collect data from the VPLMN via V-RE-NWDAF in VPLMN.

-	For inbound roaming users, the NF consumer in the VPLMN can retrieve analytics from the HPLMN via V-RE-NWDAF in VPLMN and H-RE-NWDAF in HPLMN.

NOTE 2:	The analytics from the HPLMN may be generated by H-RE-NWDAF in the HPLMN or other NWDAFs in the HPLMN.

-	For inbound roaming users, the V-RE-NWDAF can collect data from the HPLMN via the H-RE-NWDAF.

NOTE 3:	Both local breakout and home routed roaming architecture support the data or analytics exchanging between PLMNs.

NOTE 4:	Interactions between RE-NWDAFs of different PLMNs may be via SEPPs, which are not depicted in the architecture for the sake of clarify.

# * * * * Next changes * * * *

#### 6.1.1.1	Analytics subscribe/unsubscribe by NWDAF service consumer

This procedure is used by any NWDAF service consumer (e.g. including NFs/OAM) to subscribe/unsubscribe at NWDAF to be notified on analytics information, using Nnwdaf_AnalyticsSubscription service defined in clause 7.2. This service is also used by an NWDAF service consumer to modify existing analytics subscription(s). Any entity can consume this service as defined in clause 7.2.

Figure 6.1.1.1-1: Network data analytics Subscribe/unsubscribe

1.	The NWDAF service consumer subscribes to or cancels subscription to analytics information by invoking the Nnwdaf_AnalyticsSubscription_Subscribe/ Nnwdaf_AnalyticsSubscription_Unsubscribe service operation. The parameters that can be provided by the NWDAF service consumer are listed in clause 6.1.3.

When a subscription to analytics information is received, the NWDAF determines whether triggering new data collection is needed.

If the service invocation is for a subscription modification, the NF service consumer includes an identifier (Subscription Correlation ID) to be modified in the invocation of Nnwdaf_AnalyticsSubscription_Subscribe. In addition, if the NWDAF service consumer has taken an action(s) influenced by the previously received analytics information at step 2, which may or may not affect the ground truth data corresponding to Analytics ID requested at the time which the prediction refers to, the NWDAF service consumer may include Analytics feedback information in the invocation of Nnwdaf_AnalyticsSubscription_Subscribe.

If the subscription relates to outbound roaming users, the NWDAF in the HPLMN may decide to retrieve or to subscribe to input data or analytics from the VPLMN, the NWDAF in the HPLMN is H-RE-NWDAF as described in clause 4.3, and the detailed procedure is described in clause 6.1.5.3.

If the subscription relates to inbound roaming users, the NWDAF in the VPLMN may decide to retrieve or to subscribe to input data or analytics from the HPLMN, the NWDAF in the VPLMN is V-RE-NWDAF as described in clause 4.3, and the detailed procedure is described in clause 6.1.5.2.

2.	If NWDAF service consumer is subscribed to analytics information, the NWDAF notifies the NWDAF service consumer with the analytics information by invoking Nnwdaf_AnalyticsSubscription_Notify service operation, based on the request from the NWDAF service consumer, e.g. Analytics Reporting Parameters. If the NWDAF provides a Termination Request, then the consumer cancels subscription to analytics information by invoking the Nnwdaf_AnalyticsSubscription_Unsubscribe service operation.

When calculating the analytics/ML model accuracy information with the retrieved Analytics feedback information, in addition to comparing predictions of ML model and its corresponding ground truth data, the NWDAF may additionally determine/take into account whether the action(s) taken by the NWDAF service consumer affects the ground truth data corresponding to Analytics ID requested at the time which the prediction refers to as described in clauses 6.2D and 6.2E, which may affect the ML Model Accuracy monitoring.

# * * * * Next changes * * * *

#### 6.1.1.2	Analytics subscribe/unsubscribe by AFs via NEF

The analytics exposure to AFs may be performed via NEF by using analytics subscription to NWDAF.

Figure 6.1.1.2-1 illustrates the interaction between AF and NWDAF performed via the NEF.

Figure 6.1.1.2-1: Procedure for analytics subscribe/unsubscribe by AFs via NEF

0.	NEF controls the analytics exposure mapping among the AF identifier with allowed Analytics ID and associated inbound restrictions (i.e, applied to subscription of the Analytics ID for an AF) and/or outbound restrictions (i.e. applied to notification of Analytics ID to an AF).

In this Release, AF is configured with the appropriated NEF to subscribe to analytics information, the allowed Analytics ID(s) and with allowed inbound restrictions (i.e. parameters and/or parameter values) for subscription to each Analytics ID.

1.	The AF subscribes to or cancels subscription to analytics information via NEF by invoking the Nnef_AnalyticsExposure_Subscribe/ Nnef_AnalyticsExposure_Unsubscribe service operation defined in TS 23.502 [3]. If the AF wants to modify an existing analytics subscription at NEF, it includes an identifier (Subscription Correlation ID) to be modified in the invocation of Nnef_AnalyticsExposure_Subscribe, in addition, if the AF has taken an action(s) influenced by the previously received analytics information at step 4, which may or may not affect the ground truth data corresponding to Analytics ID requested at the time which the prediction refers to, the AF may include Analytics feedback information in the invocation of Nnef_AnalyticsExposure_Subscribe. If the analytics information subscription is authorized by the NEF, the NEF proceeds with th  e steps below.

2.	Based on th e request from the AF, the NEF subscribes to or cancels subscription to analytics information by invoking the Nnwdaf_AnalyticsSubscription_Subscribe/ Nnwdaf_AnalyticsSubscription_Unsubscribe service operation.

If the parameters and/or parameters values of the AF request comply with the inbound restriction in the analytics exposure mapping, NEF forwards in the subscription to NWDAF service the Analytics ID, parameters and/or parameters values from the AF request.

If the request from AF does not comply with the restrictions in the analytics exposure mapping, NEF may apply restrictions to the subscription request to NWDAF (e.g. restrictions to parameters or parameter values of the Nnwdaf_AnalyticsSubscription_Subscribe service operations), based on operator configuration and/or may apply parameter mapping (e.g. geo coordinate mapping to TA(s)/Cell-id(s)).

The NEF records the association of the analytics request from the AF and the analytics request sent to the NWDAF.

The NEF selects an NWDAF that supports analytics information requested by the AF using the NWDAF discovery procedure defined in TS 23.501 [2].

If the subscription relates to outbound roaming users, the NWDAF in the HPLMN may decide to retrieve or to subscribe to input data or analytics from the VPLMN, the NWDAF in the HPLMN is H-RE-NWDAF as described in clause 4.3, and the detailed procedure is described in clause 6.1.5.3.

If the AF request is for a modification of the existing analytics subscription(s), the NEF invokes Nnwdaf_AnalyticsSubscription_Subscribe to modify the analytics subscription identified by an identifier (Subscription Correlation ID) associated with the AF.

3.	If the NEF has subscribed to analytics information, the NWDAF notifies the NEF with the analytics information or Termination Request by invoking Nnwdaf_AnalyticsSubscription_Notify service operation.

4.	If the NEF receives the notification from the NWDAF, the NEF notifies the AF with the analytics information or Termination Request by invoking Nnef_AnalyticsExposure_Notify service operation defined in TS 23.502 [3]. NEF may apply outbound restrictions to the notifications to AFs (e.g. restrictions to parameters or parameter values of the Nnef_AnalyticsExposure_Notify service operation) based on analytics exposure mapping and may apply parameter mapping for external usage (e.g. TA(s), Cell-id(s) to geo coordinate). The AF checks if a Termination Request is present in the Nnef_AnalyticsExposure_Notify as defined in step 2 in clause 6.1.1.1.

When calculating the analytics/ML model accuracy information with the retrieved Analytics feedback information, in addition to comparing predictions of ML model and its corresponding ground truth data, the NWDAF may determine/take into account whether the action(s) taken by the AF affects the ground truth data corresponding to Analytics ID requested at the time which the prediction refers to or start rating AF(s) used as data sources, as described in clauses 6.2D and 6.2E, which may affect the ML Model Accuracy monitoring.

# * * * * Next changes * * * *

#### 6.1.2.1	Analytics request by NWDAF service consumer

This procedure is used by the NWDAF service consumer (e.g. including NFs/OAM) to request and get from NWDAF analytics information, using Nnwdaf_AnalyticsInfo service defined in clause 7.3.

Figure 6.1.2.1-1: Network data analytics Request

1.	The NWDAF service consumer requests analytics information by invoking Nnwdaf_AnalyticsInfo_Request service operation. The parameters that can be provided by the NWDAF service consumer are listed in clause 6.1.3.

When a request for analytics information is received, the NWDAF determines whether triggering new data collection is needed.

If the analytics request relates to outbound roaming users, the NWDAF in the H-PLMN may decide to retrieve input data or analytics from the related V-PLMN, the NWDAF in the HPLMN is H-RE-NWDAF as described in clause 4.3, and the detailed procedure is described in clause 6.1.5.3.

If the analytics request relates to inbound roaming users, the NWDAF in the V-PLMN may decide to retrieve input data or analytics, from the related H-PLMN, the NWDAF in the VPLMN is V-RE-NWDAF as described in clause 4.3, and the detailed procedure is described in clause 6.1.5.2.

2.	The NWDAF responds with analytics information to the NWDAF service consumer. The NWDAF checks if a Termination Request is indicated as defined in step 2 in clause 6.1.1.1.

# * * * * Next changes * * * *

#### 6.1.2.2	Analytics request by AFs via NEF

The analytics exposure to AFs may be performed via NEF by using analytics request to NWDAF.

Figure 6.1.2.2-1 illustrates the interaction between AF and NWDAF performed via the NEF.

Figure 6.1.2.2-1: Procedure for analytics request by AFs via NEF

0.	NEF controls the analytics exposure mapping among the AF identifier with allowed Analytics ID(s) and associated inbound restrictions (i.e. applied to the Analytics ID requested by AF and/or outbound restrictions (i.e. applied to the response of Analytics ID to AF).

In this Release, AF is configured, e.g. via static OAM configuration, with the appropriated NEF to subscribe to analytics information, the allowed Analytics ID(s) and with allowed inbound restrictions (i.e. parameters and/or parameter values) for requesting each Analytics ID.

1.	The AF requests to receive analytics information via NEF by invoking the Nnef_AnalyticsExposure_Fetch service operation defined in TS 23.502 [3]. If the analytics information request is authorized by the NEF, the NEF proceeds with the steps below.

2.	Based on the request from the AF, the NEF requests analytics information by invoking the Nnwdaf_AnalyticsInfo_Request service operation.

If the parameters and/or parameters values of the AF request comply with the restriction in the analytics exposure mapping, NEF forwards in the subscription to NWDAF service the Analytics ID, parameters and/or parameters values from AF in the request to NWDAF.

If the request from AF does not comply with the restrictions in the analytics exposure mapping, NEF may apply restrictions to the request to NWDAF (e.g. restrictions to parameters or parameter values of the Nnwdaf_AnalyticsInfo_Request service operations) based on operator configuration and/or may apply parameter mapping (e.g. geo coordinate mapping to TA(s), Cell-id(s)).

The NEF records the association of the analytics request from the AF and the analytics request sent to the NWDAF.

The NEF selects an NWDAF that supports analytics information requested by the AF using the NWDAF discovery procedure defined in TS 23.501 [2].

If the analytics request relates to outbound roaming users, the NWDAF in the H-PLMN may decide to retrieve input data or analytics from the related V-PLMN, the NWDAF in the HPLMN is H-RE-NWDAF as described in clause 4.3, and the detailed procedure is described in clause 6.1.5.3.

3.	The NWDAF responds with the analytics information to the NEF.

4.	The NEF responds with the analytics information to the AF. NEF may apply restrictions to the response to AFs (e.g. restrictions to parameters or parameter values of the Nnef_AnalyticsExposure_Fetch response service operation) based on operator configuration. The AF checks if a Termination Request is present and then follows as defined in step 2 in clause 6.1.1.1.

# * * * * Next changes * * * *

#### 6.1.5.1	General

Based on roaming agreements (e.g. SLA), analytics may be exchanged between PLMNs (i.e. HPLMN and VPLMN of a UE served by the NWDAF analytics consumer). In this case, an NWDAF with roaming exchange capability (RE-NWDAF) is used as entry point in a PLMN to exchange analytics in roaming scenario with other PLMNs. It authorizes the analytics request according to roaming agreements, and filters the information exposed to other PLMNs. The roaming architecture is defined in clause 4.3.

The H-RE-NWDAF may provide analytics to the V-RE-NWDAF. The V-RE-NWDAF may provide analytics to the H-NWDAF.

NOTE 1:	The roaming agreements (e.g. SLA) between HPLMN and VPLMN on network analytics and data exchange takes into consideration all relevant regulatory requirements.

NOTE 2:	It depends on the deployment whether analytics exchange between PLMNs is supported, and thus whether H-RE-NWDAF and V-RE-NWDAF are provided in a PLMN.

The H-RE-NWDAF or V-RE-NWDAF provides the Nnwdaf_RoamingAnalytics for that purpose. An RE-NWDAF is the only consumer of these services, i.e. both H-NWDAF and V-NWDAF need to have roaming exchange capability when used as entry point to exchange analytics in roaming scenario.

NOTE 3:	The access to the Nnwdaf_AnalyticsSubscription service and the Nnwdaf_AnalyticsInfo service is expected to be restricted by the NRF to NF service consumers within the same PLMN to prevent bypassing checks based on user consent and operator policy

Editor's note:	The user consent check procedure and security aspects will align with SA3 WG's conclusion.

V-RE-NWDAF may request or subscribe to HPLMN analytics from the H-RE-NWDAF as described in clause 6.1.5.2, and then the analytics can be leveraged by the 5GC NF in the VPLMN, for example:

-	In home routed roaming scenarios, HPLMN analytics (i.e. slice load level analytics, NF load analytics, etc.) can be leveraged by the AMF in the VPLMN for network slice selection and SMF selection for PDU Session management.

-	UE-related analytics (e.g.. service experience analytics, etc.) can include statistics or predictions for outbound roaming UEs from the HPLMN.

NOTE 4:	Analytics that rely on input data from the VPLMN are preferably not provided from H-RE-NWDAF to V-RE-NWDAF, but generated by a NWDAF in the VPLMN.

H-RE-NWDAF may request or subscribe to VPLMN analytics from the V-RE-NWDAF as described in clause 6.1.5.3, and then the analytics can be leveraged by the 5GC NF in the HPLMN, for example:

-	In home routed roaming scenarios, analytics information with statistics or predictions for outbound roaming UEs can be leveraged by the H-PCF for QoS control of the PDU Session.

-	Analytics (i.e. service experience analytics, slice load level analytics, etc.) can be leveraged by the H-PCF for decision of NSSP in URSP rules provisioned to the UE roaming in the VPLMN.

NOTE 5:	Analytics that rely on input data from the HPLMN are preferably not provided from V-RE-NWDAF to H-RE-NWDAF, but generated by a NWDAF in the HPLMN.

# * * * * Next changes * * * *

#### 6.1.5.2	Analytics Exposure from HPLMN to VPLMN

Figure 6.1.5.2-1 shows the procedure that the V-RE-NWDAF requests network analytics (i.e. slice load level analytics, NF load analytics, service experience analytics, etc.) in HPLMN from a H-RE-NWDAF, upon receiving an analytics information request/subscription from the NF (e.g. AMF) in VPLMN.

Figure 6.1.5.2-1: Procedure for analytics exposure from HPLMN to VPLMN

1.	Consumer NF in VPLMN (e.g. AMF) discovers a V-RE-NWDAF as described in clause 5.2 and sends an Analytics request/subscribe (Analytics ID, Analytics Filter Information, Target of Analytics Reporting) to V-RE-NWDAF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe. For the inbound roaming user(s) indicated in the Target of Analytics Reporting, V-RE-NWDAF determines based on operator configuration and the requested analytics whether analytics or input data from the HPLMN are required, or the analytics can be derived locally. The subsequent steps only apply if analytics from the HPLMN are required. If input data from the HPLMN are required, the procedures in clause 6.2.11 apply.

NOTE 1:	It is possible that the Target of Analytics Reporting sent by the Consumer NF to the V-RE-NWDAF includes both inbound roaming user(s) and non-roaming user(s).

2.	V-RE-NWDAF checks the roaming agreements related to analytics from HPLMN to determine if the roaming analytics request/subscribe can be accepted or must be rejected. If the request is rejected, the following steps are skipped.

V-RE-NWDAF discovers a H-RE-NWDAF as described in clause 5.2. V-RE-NWDAF sends a roaming analytics request/subscribe (Analytics ID, Analytics Filter Information, Target of Analytics Reporting) to H-RE-NWDAF by invoking a Nnwdaf_RoamingAnalytics_Request or a Nnwdaf_RoamingAnalytics_Subscribe, based on the Analytics request/subscribe received from the Consumer NF in VPLMN. The Target of Analytics Reporting sent by the V-RE-NWDAF to the H-RE-NWDAF only contains the inbound roaming user(s).

NOTE 2:	The inbound roaming user(s) are distinguished by the V-RE-NWDAF according to UE ID(s) (i.e. SUPI(s)).

3a.	H-RE-NWDAF checks the roaming agreements between the HPLMN and the VPLMN, and user consent for analytics as defined in clause 6.2.9 if needed, to determine if the roaming analytics request/subscribe can be accepted or must be rejected. If the roaming analytics request/subscribe is rejected, the following steps are skipped.

If H-RE-NWDAF supports to generate the requested analytics, it collects data from the NF(s) and/or OAM in HPLMN and derives the requested analytics; otherwise steps 3b and 3c are executed.

3b-3c.	[Optional] If the H-RE-NWDAF does not support to generate the requested analytics, it may request/subscribe to other NWDAF(s) in the HPLMN (if available) for the analytics and get corresponding response/notification.

Editor's note:	The user consent check procedure and security aspects will align with SA WG3's conclusion.

4.	H-RE-NWDAF sends the HPLMN analytics information to the V-RE-NWDAF using either Nnwdaf_RoamingAnalytics_Request response or Nnwdaf_RoamingAnalytics_Notify, depending on the service used in step 2. The H-RE-NWDAF may restrict the exposed analytics information based on HPLMN operator polices.

5.	[Optional] If the Consumer NF also indicates request or subscription of analytics information available in the VPLMN (e.g. via Target of Analytics Reporting) in step 1, V-RE-NWDAF collects data from the NF(s) and/or OAM in VPLMN and derives the requested analytics. The V-RE-NWDAF may perform analytics aggregation with the analytics information received from the H-RE-NWDAF and analytics information generated by itself, based on the analytics request or subscription.

6.	V-RE-NWDAF sends the HPLMN analytics information received in step 4, or the aggregated analytics information if step 5 are performed, to the Consumer NF in VPLMN using either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1.

NOTE 3:	The present document describes that the RE-NWDAF may perform analytics aggregation for roaming scenario, but whether and how the RE-NWDAF performs analytics aggregation for roaming scenario are up to implementation.

# * * * * Next changes * * * *

#### 6.1.5.3	Analytics Exposure from VPLMN to HPLMN

Figure 6.1.5.3-1 shows the procedure that the H-RE-NWDAF requests network analytics (i.e. service experience analytics, slice load level analytics, etc.) in VPLMN from the V-RE-NWDAF, upon receiving an analytics information request/subscription from the NF (e.g. PCF) in HPLMN.

Figure 6.1.5.3-1: Procedure for analytics exposure from VPLMN to HPLMN

1.	If the Consumer NF is aware that the UE(s) indicated in Target of Analytics Reporting is/are roaming, Consumer NF in HPLMN (e.g. H-PCF) discovers a H-RE-NWDAF as described in clause 5.2 and sends an Analytics request/subscribe (Analytics ID, Target of Analytics Reporting, Analytics Filter Information) to H-RE-NWDAF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe. For the outbound roaming user(s) indicated in the Target of Analytics Reporting, H-RE-NWDAF determines based on operator configuration and the requested analytics whether analytics or input data from the VPLMN are required, or the analytics can be derived locally. The subsequent steps only apply if analytics from the VPLMN are required. If input data from the VPLMN are required, the procedures in clause 6.2.10 apply.

If the Consumer NF is unaware that the UE(s) indicated in Target of Analytics Reporting is/are roaming and sends the Analytics request/subscribe to a H-NWDAF which does not support roaming exchange capability, the H-NWDAF may either:

-	after determining that the UE(s) indicated in Target of Analytics Reporting is/are roaming, e.g. by inquiring the AMF(s) serving the UE(s) at the UDM with the Nudm_UECM service, forward the Analytics request/subscribe to a H-RE-NWDAF after discovering a H-RE-NWDAF as described in clause 5.2. The H-NWDAF may include the VPLMN ID in the Analytics request/subscribe; or

-	reject the Analytics request/subscribe with a proper cause value. And the following steps will not be performed.

NOTE 1:	It is possible that the Target of Analytics Reporting sent by the Consumer NF to the H-RE-NWDAF includes both outbound roaming user(s) and non-roaming user(s).

NOTE 2:	The H-NWDAF is not depicted in the flow.

2.	If PLMN ID of the VPLMN is not included in the Analytics request/subscribe, the H-NWDAF inquires it at the UDM for the UEs indicated as Target of Analytics Reporting. The H-RE-NWDAF checks user consent for analytics as defined in clause 6.2.9. H-RE-NWDAF checks the roaming agreements between the HPLMN and the VPLMN to determine if the roaming analytics request/subscription can be accepted or must be rejected. If the request is rejected, the following steps are skipped.

Editor's note:	The user consent check procedure and security aspects will align with SA WG3's conclusion.

The H-RE-NWDAF discovers the V-RE-NWDAF as described in clause 5.2.

H-RE-NWDAF sends a roaming analytics request/subscribe (Analytics ID, Analytics Filter Information, Target of Analytics Reporting, [NF ID(s)]) to V-RE-NWDAF by invoking a Nnwdaf_RoamingAnalytics_Request or a Nnwdaf_RoamingAnalytics_Subscribe, based on the Analytics request/subscribe received from the Consumer NF in HPLMN. The Target of Analytics Reporting sent by the H-RE-NWDAF to the V-RE-NWDAF only contains the outbound roaming user(s). The H-RE-NWDAF may obtain NF ID(s) of the NF(s) serving the roaming UE(s) in the VPLMN, e.g. AMF ID(s), SMF ID(s), from the UDM and include the NF ID(s) in the analytics request/subscribe.

3a.	V-RE-NWDAF checks the roaming agreements between the HPLMN and the VPLMN, to determine if the roaming analytics request/subscribe can be accepted or must be rejected. If the roaming analytics request/subscribe is rejected, the following steps are skipped.

If V-RE-NWDAF supports to generate the requested analytics, it collects data from the NF(s) serving the roaming UE(s) and/or OAM in VPLMN and derives the analytics; otherwise steps 3b and 3c are executed. The NF(s) serving the roaming UE(s), e.g. AMF(s) or SMF(s), if indicated in step 2, can be used as data source.

3b-3c.	[Optional] If the V-RE-NWDAF does not support to generate the requested analytics, it may request/subscribe to other NWDAF(s) in the VPLMN (if available) for the analytics and get corresponding response/notification(s).

4.	V-RE-NWDAF sends the VPLMN analytics information to the H-RE-NWDAF using either Nnwdaf_RoamingAnalytics_Request response or Nnwdaf_RoamingAnalytics_Notify, depending on the service used in step 2. The V-RE-NWDAF may restrict the exposed analytics information based on VPLMN operator polices.

5.	[Optional] If the Consumer NF also indicates request or subscription of analytics information available in the HPLMN (e.g. via Target of Analytics Reporting) in step 1, H-RE-NWDAF collects data from the NF(s) and/or OAM in HPLMN and derives the analytics. These steps can be executed in parallel with steps 3-6. The H-RE-NWDAF may perform analytics aggregation with the analytics information received from the V-RE-NWDAF and analytics information generated by itself, based on the analytics request or subscription.

6.	H-RE-NWDAF sends the VPLMN analytics information received in step 4, or the aggregated analytics information if step 5 are performed, to the Consumer NF in HPLMN using either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1.

If the Analytics request/subscribe is forwarded to the H-RE-NWDAF by a H-NWDAF as described in step 1, the H-NWDAF forwards the analytics information received from the H-RE-NWDAF to the Consumer NF using either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1.

NOTE 3:	The present document describes that the RE-NWDAF may perform analytics aggregation for roaming scenario, but whether and how the RE-NWDAF performs analytics aggregation for roaming scenario are up to implementation.

* * * End of Changes * * *