3GPP TSG-SA WG2 Meeting #158	S2-2308916

# Goteborg, Sweden, 21  - 25 August 2023		 (revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4808 | rev | - | Current version: | 18.2.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove the EN on the Non-homogenous support of PDU Set based handling in NG-RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-08-08 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is EN in clause 5.37.5.3 as below: Editor's note: In the case of handover from a source NG-RAN node not supporting PDU Set handling to a target NG-RAN node supporting PDU Set handling, whether there is a reason for the target NG-RAN node to temporarily handle both unmarked PDUs and marked PDUs within the same QoS flow is FFS. Whether and how the NG-RAN temporarily handles both unmarked PDUs and marked PDUs within the same QoS flow is to be defined. If the QoS Flow is configured with the legacy QoS parameter, the NG-RAN hands the PDU with the PDU Set marking with the legacy QoS. If the QoS Flow is configured with PDU Set QoS but the packet without PDU Set marking, how the NG-RAN handles these packets is decided by the RAN WG. But from SA2 point of view, after the QoS Flow is handover to the target NG-RAN supporting PDU Set handling, the SMF can initiate the PDU Session Modification to configure the PDU Set QoS to the QoS Flow. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following new text is added and the EN is removed. After the handover from a source NG-RAN node not supporting PDU Set handling to a target NG-RAN node supporting PDU Set handling, the SMF can initiate the PDU Session modification procedure to activate the PDU Set handling for the same QoS flow. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear how to handle the QoS Flow from the 5G core network after handover from a source NG-RAN node not supporting PDU Set handling to a target NG-RAN node supporting PDU Set handling. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.37.5.3	Non-homogenous support of PDU set based handling in NG-RAN

By sending at least one PDU Set QoS parameter to the NG-RAN, the SMF requests the NG-RAN to activate PDU Set QoS handling for a given QoS flow and the NG-RAN provides the SMF with an indication of whether the PDU Set QoS parameter is accepted or not. At NG-RAN Xn handover and N2 handover, the target NG-RAN provides to the SMF with an indication of whether the target NG-RAN node supports PDU Set based handling, as specified in TS 38.413 [34]. Based on the NG-RAN indications, the SMF configures the PSA UPF to activate/deactivate the PDU Set identification and marking.

In the case where the PSA UPF identifies and marks PDUs with PDU Set information in GTP-U header it shall start doing so from a complete PDU Set.

After the handover from a source NG-RAN node not supporting PDU Set handling to a target NG-RAN node supporting PDU Set handling, the SMF can initiate the PDU Session modification procedure to activate UE, the NG-RAN and PSA UPF of the PDU Set handling for the QoS flow(s).

# * * * * End of changes * * * *