SA WG2 Meeting #158	S2-2308971

# 21-25 August 2023, Gothenburg, Sweden	(was S2-230xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4371 | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New UPF service for UE public IP address request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-08-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It is approved to support NAT in UPEAS for Rel-18. Several procedures were updated to support NAT in UPEAS and AIMLsys. This CR proposed to support NAT for UE data collection procedure. In order to support this, a new UPF service is proposed in this contribution, if the public UE IP address is received and the NAT is deployed, the SMF will further request UPF to provide the pbulic UE IP address and port number via Nupf_GetPublicUEIPaddr Service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - add Nupf_GetPublicUEIPaddr Service in table 5.2.26.1-1. - define Nupf_GetPublicUEIPaddr Servicein clause 5.2.26.x. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE data collection is not supported for the NAT deployed use case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.26.1, new clause 5.2.26.x, new clause 5.2.26.x.1, new clause 5.2.26.x.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

#### 5.2.26.1	General

The following table shows the UPF Services and UPF Service Operations.

Table 5.2.26.1-1: NF services provided by the UPF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nupf_EventExposure | Notify | Subscribe/Notify | NEF, AF, NWDAF, TSNAF/DCCF/MFAF/TSCTSF |
|  | Subscribe | Subscribe/Notify | NWDAF/DCCF, SMF |
|  | Unsubscribe | Subscribe/Notify | NWDAF/DCCF, SMF |
| Nupf_GetPrivateUEIPaddr | Get | Request/Response | NEF |
| Nupf_GetPublicUEIPaddr | Get | Request/Response | SMF |

# >>>> NEXT CHANGES<<<<

#### 5.2.26.x	Nupf_GetPublicUEIPaddr Service

##### 5.2.26.x.1	General

This service provides the public UE IP address and port number to the service consumer.

##### 5.2.26.x.2	Nupf_GetPublicUEIPaddr Service operation

Service operation name: Nupf_GetPublicUEIPaddr_Get

Description: NF service consumer gets the UE Public IP address and port number that used between UE and the Data Network if NAT is used in UPF.

Inputs, Required: PDU Session ID.

Inputs, Optional: DNN, S-NSSAI.

Outputs, Required: UE Public IP address, port number.

Outputs, Optional: None.

# >>>>END OF CHANGES<<<<