3GPP TSG-SA WG2 Meeting #158	S2-2309145

Gothenburg, Sweden, August 21 – 25, 2023	(revision of S2-230xxxx)

Title:	DRAFT Reply LS on Issues on QoS monitoring for satellite backhaul

Response to:	S2-2308298 / C3-232469

Release:	Rel-17, Rel-18

Work Item:	SMS_SBI

Source:	Nokia, to be SA2

To:	CT3

Cc:

Contact Person:

#### Name:	 Hannu Hietalahti

E-mail Address:	 hannu.hietalahti@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.501 CR 4861

1. Overall Description:

SA2 thanks CT3 for their LS and provides the following comments and answers.

The possibility to re-use the already specified QoS monitoring mechanism to determine the actual packet delay on the user plane between the UE and the UPF has been specified as a UPF option to obtain measured QoS information to assist the UPF policy decisions. This option is not mandatory for the PCF as in some cases the backhaul delay can be considered negligible or be determined by other means. If the PCF chooses to trigger the user plane QoS measurement between the UE and the UPF, it is a PCF option whether and how to use the results in its policy decisions.

SA2 answers to the questions from CT3 as follows:

Question 1:	Please clarify whether the PCF may determine to request QoS monitoring to get the report(s) for the packet delay based on the request received from the AF or based on local policy/configuration at the PCF (e.g., on the satellite backhaul category reported from the SMF)?

SA2 answer: The PCF can decide whether to request QoS monitoring based on its own policy or configuration. SA2 has clarified the TS 23.501 text in the attached CR to say that the QoS measurement procedure that was initially designed for the AF triggered PCF behaviour is re-used in Rel-18 also as a PCF option if it receives dynamic satellite backhaul indication.

Question 2:	If the PCF triggers packet delay reporting for dynamic satellite backhaul categories based on local policy/configuration, does the report influence any policy decisions at the PCF?

SA2 answer: The PCF may use the QoS measurement results as appropriate, e.g. to ensure that QoS with excessively strict latency requirement is not assigned for UE whose connections uses dynamic satellite backhaul.

2. Actions:

To CT3:

ACTION: 	SA2 kindly asks CT3 to take notice of this information and to align their specifications with it, as necessary.

3. Date of Next TSG-SA WG2 Meetings:

|  | DATES | LOCATION | CTRY |
| --- | --- | --- | --- |
| SA2#159 | Oct 9 – 13, 2023 | Xiamen | China |
| SA2 #160 | Nov 13 – 17, 2023 | Chicago | US |

See https://portal.3gpp.org/Meetings?tbid=375&SubTB=385#/