3GPP TSG-SA2 Meeting #158	S2-2309591

# 21 - 25 August, 2023, Goteborg, Sweden

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0880 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Refinement for ML Model information for KI#4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 6.2A.2, apart from ML model file address and ADRF (set) ID, some other extra information was also added to the ML Model Information, that was not correct and not aligned with descriptions for ML Model Information in other clauses, e.g. 6.2B.7 and 6.2F.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Refine the wording for ML Model Information to only include ML model file address and ADRF (set) ID, excluding other extra information. - Refine the overlapped wording for data sources. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misunderstanding of ML Model Information and data sources. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

### 6.2A.2	Contents of ML Model Provisioning

The consumers of the ML model provisioning services (i.e. an NWDAF containing AnLF) as described in clause 7.5 and clause 7.6 may provide the input parameters as listed below:

-	Information of the analytics for which the requested ML model is to be used, including:

-	A list of Analytics ID(s): identifies the analytics for which the ML model is used.

-	NF consumer information: identifies the vendor of NWDAF containing AnLF.

NOTE 1:	NF consumer information such as Vendor ID is defined in Stage 3.

-	[OPTIONAL] Use case context: indicates the context of use of the analytics to select the most relevant ML model ML model.

NOTE 2:	The NWDAF containing MTLF can use the parameter "Use case context" to select the most relevant ML model, when several ML models are available for the requested Analytics ID(s). The values of this parameter are not standardized.

-	[OPTIONAL] ML Model Interoperability Information. This is vendor-specific information that conveys, e.g., requested model file format, model execution environment, etc. The encoding, format, and value of ML Model Interoperable Information is not specified since it is vendor specific information, and is agreed between vendors, if necessary for sharing purposes.

-	[OPTIONAL] ML Model Filter Information: enables to select which ML model for the analytics is requested, e.g. S-NSSAI, Area of Interest. Parameter types in the ML Model Filter Information are the same as parameter types in the Analytics Filter Information which are defined in procedures.

-	[OPTIONAL] Target of ML Model Reporting: indicates the object(s) for which ML model is requested, e.g. specific UEs, a group of UE(s) or any UE (i.e. all UEs).

-	[OPTIONAL] Requested representative ratio: a minimum percentage of UEs in the group whose data is a non-empty set and can be used in the model training when the Target of ML Model Reporting is a group of UEs.

-	ML Model Reporting Information with the following parameters:

-	(Only for Nnwdaf_MLModelProvision_Subscribe) ML Model Reporting Information Parameters as per Event Reporting Information Parameter defined in Table 4.15.1-1, TS 23.502 [3].

-	[OPTIONAL] ML Model Target Period: indicates time interval [start, end] for which ML model for the Analytics is requested. The time interval is expressed with actual start time and actual end time (e.g. via UTC time).

-	[OPTIONAL] Inference Input Data information: contains information about various settings that are expected to be used by AnLF during inferences such as:

-	the "Input Data" that are expected be used, each of them optionally accompanied by metrics that show the granularity with which this data will be used (i.e., a sampling ratio, the maximum number of input values, and/or a maximum time interval between the samples of this input data).

NOTE 3:	This can be a subset of the possible Input Data specified for a certain analytics type.

-	the data sources that are expected to be used as a list of NF instance (or NF set) identifiers.

-	A Notification Target Address (+ Notification Correlation ID) as defined in clause 4.15.1 of TS 23.502 [3], allowing to correlate notifications received from the NWDAF containing MTLF with this subscription.

-	[OPTIONAL] Indication of supporting multiple ML models.

-	[OPTIONAL] Accuracy level(s) of Interest.

-	[OPTIONAL] Time when model is needed: indicates the latest time when the consumer expects to receive the ML model(s).

-	[OPTIONAL] Number of ML model(s), indicating the maximum number of Multiple ML models the ML Model provider e.g. NWDAF(MTLF) could provide to the consumers of the ML model(s).

NOTE 4:	Multiple ML models Filter Information are composed by Indication of supporting multiple ML models, Accuracy level(s) of Interest, Number of ML model(s).

-	[OPTIONAL] ML Model Monitoring Information: This is information provided to the NWDAF containing MTLF which may include ML Model metric (i.e. ML Model Accuracy.), ML model monitoring reporting mode (Accuracy reporting interval or pre-determined status (ML Model Accuracy threshold(s)). Depending on the reporting mode, the NWDAF containing MTLF reports the model accuracy to NWDAF containing AnLF either periodically or when the ML model accuracy is crossing an ML Model Accuracy threshold, i.e. the accuracy either becomes higher or lower than the ML Model Accuracy threshold.

-	[OPTIONAL] ML Model Accuracy Monitoring Information with the following parameters:

-	[OPTIONAL] Analytics Accuracy Threshold: indicating the accuracy threshold of the ML Model requested by the consumer. It also can be used as an indication that the MTLF is triggered to execute the accuracy monitoring operations for the ML Model provisioned to AnLF.

-	[OPTIONAL] DataSetTag and ADRF ID if available: indicates the inference data (including input data, prediction and the ground truth data at the time which the prediction refers to) stored in ADRF which can be used by MTLF to retrain or reprovision of the ML model.

The NWDAF containing MTLF provides to the consumer of the ML model provisioning service operations as described in clause 7.5 and 7.6, the output information as listed below:

-	(Only for Nnwdaf_MLModelProvision_Notify) The Notification Correlation Information.

-	For each Analytics ID requested by the service consumer, a set of pair (s) of unique ML Model identifier and the following information.

-	ML Model Information, which includes:

-	the ML model file address (e.g. URL or FQDN); or

-	ADRF (Set) ID.

When ADRF (Set) ID is provisioned, a Storage Transaction ID may also be provisioned.

-	[OPTIONAL] ML model degradation indicator: indicates whether the provided ML model is degraded.

-	[OPTIONAL] Validity period: indicates time period when the provided ML Model Information applies.

-	[OPTIONAL] Spatial validity: indicates Area where the provided ML Model Information applies.

-	[OPTIONAL] ML model representative ratio: indicating the percentage of UEs in the group whose data is used in the ML model training when the Target of ML Model Reporting is a group of UEs.

-	[OPTIONAL] Training Input Data Information: contains information about various settings that have been used by MTLF during training, such as:

-	the "Input Data" that have been used, each of them optionally accompanied by metrics that show the data characteristics and granularity with which this data has been used (i.e. a sampling ratio, the maximum number of input values, and/or a maximum time interval between the samples of this input data, data range including maximum and minimum values, mean and standard deviation and data distribution when applicable) and the time, i.e. timestamp and duration, when this data was obtained.

-	the data sources related to the "Input Data" that were used for ML model training.

-	ADRF (Set) ID.

When ADRF (Set) ID is provisioned, a Storage Transaction ID may also be provisioned.

NOTE 5:	This can be a subset of the possible Input Data specified for a certain analytics type.

-	the data sources that have been used as a list of NF instance (or NF set) identifiers.

NOTE 6:	Spatial validity and Validity period are determined by MTLF internal logic and it is a subset of AoI if provided in ML Model Filter Information and of ML Model Target Period, respectively.

NOTE 7:	Data source information enables ML Model selection when different models are available for an Analytics ID, or it enables a consumer to avoid selecting a ML model that used data from a specific data source at a particular time or used data characterized by specific data characteristics.

-	[OPTIONAL]ML Model Accuracy Information: indicates the accuracy of the ML model if analytics accuracy threshold is requested, which includes:

-	the accuracy information of the ML model.

-	[OPTIONAL] ML model metric (i.e. ML Model Accuracy).

* * * End of Changes * * *