3GPP TSG-WG SA2 Meeting #158	S2-2309992

# Gothenburg Aug 21 – Aug 25, 2023

# Title:	(DRAFT) Reply LS on periodic attempts for re-selection to a higher priority SNPN when access for localized services in SNPN is enabled

# Response to:	(S2-2308277/C1-232888) LS on periodic attempts for re-selection to a higher priority SNPN when access for localized services in SNPN is enabled

# Release:	Rel-18

# Work Item:	eNPN_Ph2

Source:	SA2

To:	CT1

Cc:	SA1

Contact Person:

Name:	Peter Hedman

E-mail Address:	peter dot hedman at ericsson dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: None

1. Overall Description:

SA1 replied to CT1’s question in S1-231752, that there is no service requirement on periodic attempts to reselect higher priority SNPN.

SA2 would like to clarify that the periodic reselection mechanism is part of the automatic hosting network SNPN selection function, to address the follow SA1 requirements for automatic selecting hosting network for localized services.

The 5G system shall enable the home network to allow a UE to automatically select a hosting network for accessing localized services when specified conditions (e.g., predefined time, location) are fulfilled.

The 5G system shall enable the home network to instruct a UE to select a hosting network with certain conditions (e.g., predefined time, location) based on the request from a service provider.

The 5G system shall enable the home network to allow a UE to select a hosting network or change to another hosting network, without any additional user intervention as long as the delivered services, both localized services and home routed services, are unchanged.

Without periodic reselection mechanism, the above requirements are not fully addressed, as it will sometime either require end user interaction (e.g. manually toggle access to localized service) or depend on how hosting network and localized service is deployed (e.g. only single hosting network SNPN exists at a location).

SA2 assumes the automatic selection applies in any type of deployment, e.g. single hosting network SNPN deployed at one location for a localized service, multiple SNPNs coexists at the same area and one or more of them are providing access to the same or different localized services, etc.

A scenario discussed in SA2 leading to the need for periodic reselection is described below:

-	In Figure-1 below, when the UE is in location 1, the UE remains in subscribed SNPN even after the end user has enabled the access to localized service and validity condition changes from not met to met (as the hosting network is not in reach for the UE).

-	When the UE later moves to hosting network area, the UE will automatically select to the hosting network SNPN during next periodic reselection.

Figure 1 Periodic reselection

SA2 would like SA1 be aware of that the lack of periodic re-attempt to select higher priority SNPN providing localized services could lead to the automatic selection not enabling the UE to use localized services in above scenario. Hence, SA2 proposes that periodic re-attempt to select higher priority SNPN (at least those providing localized services) is added to address the above scenario. The periodic re-attempt to select higher priority SNPN for localized service should be done such that it does not negatively impact the UE battery e.g. with additional timers for subsequent scan or also considering location information. Note that SA2 assumes the automatic selection is only done if enabled by the user as described in TS 23.501 clause 5.30.2.4.2 and that the user is able to disable it once an SNPN been selected for localized services.

2. Actions:

To CT1:

ACTION	SA2 would like CT1 to take the above into consideration and specify the periodic attempts for higher priority SNPN for localized service to make the automatic selection procedure complete.

3. Date of Next SA2 Meetings:

SA2#159	9th – 13th October 2023 		Xiamen, China

SA2#160	13th – 17th November 2023 		Chicago, USA