3GPP TSG-SA WG2 Meeting #159	S2-2311445

# Xiamen, China, October 9 - 13, 2023                                         (revision of S2-2311044)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4532 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving unclear description of UPF event exposure service related with AOI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - There are two different cases in this procedure, the first one is subscription with only AOI, and the second one is subscription with both AOI and its own target UE list. The final UE list for the first case is not described in the previous specific version. - The subscription via the SMF should be the SMF serving the UE, which is not described. - “The UE list in the AOI” is not clear to describe the UE(s) which are located in the AOI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Complete the text description for two use cases and some editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not clear procedure to be understood in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.4.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** First Change **********

4.15.4.5.4	Information flow for subscription via SMF to UPF event exposure service related with AOI

Figure 4.15.4.5.4-1: Subscription to UPF event exposure service for AOI via SMF

If the subscription is related to AOI, the UPF event consumer (e.g. NWDAF) firstly get the UE list which includes the UE(s) located in the AOI from the AMF(s), then subscribe to UPF via SMF as described in clause 4.15.4.5.2. If the UPF event consumer (e.g. NWDAF) further needs to filter certain UE(s) out from the UE list get from the AMF(s), the UPF event consumer (e.g. NWDAF) locally decide the final UE list.

1.	The UPF event consumer (e.g. NWDAF) determines the AMF(s) based on the AOI, i.e. TAIs and possibly on the target S-NSSAI and obtains the UE list which includes the UE(s) located in the AoI from AMF(s) by invoking Namf_EventExposure_Subscribe service operation to get the presence of UE(s) and moving in or out status in Area of Interest as described in clause 5.2.2.3 and in clause 5.3.4.4 of TS 23.501 [2]. The target of Namf_EventExposure_Subscribe depends on whether the UPF event consumer (e.g. NWDAF) targets one UE a group of UE or Any UE.

2.	The UPF event consumer (e.g. NWDAF) locally computes the final UE list by comparing the UE list from AMF(s) and its own target UE list if it exists. If the UPF event consumer (e.g. NWDAF) subscription related to only AOI, the final UE list equals the UE list in step 1.

3.	For each UE in the final UE list, the UPF event consumer (e.g. NWDAF) issues a subscription to UPF event exposure service via the SMF serving the UE (Nsmf_EventExposure Subscription) to get UPF data as described in clause 4.15.4.5.2. When an AMF reports a change of the list of UE(s) in the AoI (Namf_EventExposure_Notify), the UPF event consumer (e.g. NWDAF) may need to cancel the Nsmf_EventExposure Subscription or to issue a new Nsmf_EventExposure Subscription.

********** End of Changes **********