3GPP TSG-SA2 Meeting #160AH-e	S2-2401124

22 - 29 January 2024, Electronic Meeting	(revision of S2-24xxxxx)

Source:	CATT

Title:	23.700-59: New KI for FS_UAS_Ph3 WT#3

Document for:	Approval

Agenda Item:	19.10

Work Item / Release:	FS_UAS_Ph3

Abstract of the contribution: This paper is to propose new key issue for FS_UAS_Ph3.

1. Introduction

It is proposed to agree the new key issue into TR 23.700-59 based on the following WT#3 of FS_UAS_Ph3.

-	WT#3: Study how to support no-transmit zones for UAVs.

2. ECC Requirements Analyses

The WT#3 aims to support the following requirements on no-transmit zones described in ECC Decision (22)07[1]:

703-733 MHz: Protection of DTT receivers and RAS sites

- Aerial UE operating in 703-733 MHz should not transmit when less than 30 m above ground level to avoid interference to DTT receivers1;

- Nationally determined no-transmit zones are required around RAS sites operating in 1400-1427 MHz for aerial UE operating in the 703-718 MHz frequency band, as appropriate.

832-837 MHz: Protection of RAS sites

- Nationally determined no-transmit zones are required around RAS sites operating in 1660-1670 MHz for aerial UE operating in the 832-837 MHz frequency band, as appropriate.

2500-2570 MHz/2570-2620 MHz: Protection of RAS sites and radars

- Nationally determined no-transmit zones are required around RAS sites operating in 2690-2700 MHz for aerial UE operating in the 2500-2570 MHz or 2570-2620 MHz frequency band, as appropriate;

- Nationally determined no-transmit zones might be required around radars operating in 2700-2900 MHz for aerial UE operating in the 2500-2570 MHz or 2570-2620 MHz frequency band.

Based on the requirements above, we can know the following points:

-	The no-transmit zones is nationally determined areas based on the description highlighted in red.

-	The no-transmit zone is likely to be a small geographical area (e.g. smaller than a cell) based on the description highlighted in green.

-	The UAV cannot send UL data and signal when in no-transmit zones based on the description highlighted in blue.

Based on the requirements above, the following point is unclear:

-	Whether the UAV can receive DL data when in no-transmit zones.

3. Proposal

Based on the analyses in clause 2, it is proposed to study the following questions for WT#3:

-	Which NF is configured with the no-transmit zones related information, i.e. geographical area and the restricted frequency band of the no-transmit zones.

-	Which NF is responsible to convert the geographical area of the no-transmit zones into 3GPP identifiers (e.g. cell(s), TA(s)) and how, especially when the no-transmit zone does not match the 3GPP identifiers completely, e.g. the no-transmit zone is much smaller than a cell.

-	Whether and how the UAV obtains the no-transmit zone related information.

-	What is UAV behaviour when the event that flying into, existing in and flying out of the no-transmit zone is detected, e.g. whether the UAV can receive DL data and signal when the event that flying into or existing in the no-transmit zone happens.

NOTE 1:	Clarification on the scenario that the UAV is in the no-transmit zone is that when a new no-transmit zone is added, there may be some UAV already exists in the new no-transmit zone.

-	Which NF is responsible to detect the event that UAV flying into, exist in and flying out of the no-transmit zone. If the event is detected, how does the network ensure that the UAV obey the specified UAV behaviour for no-transmit zone. If the UAV does not obey the specified behaviour is detected, how to control UAV behaviour.

-	Whether it is needed or not to expose no-transmit zone related UAV event to UTM/USS.

4. Reference

[1] ECC Decision (22)07: “Harmonised technical conditions for the usage of aerial UE for communications based on LTE and 5G NR in the bands 703-733 MHz, 832-862 MHz, 880-915 MHz, 1710- 1785 MHz, 1920-1980 MHz, 2500-2570 MHz and 2570- 2620 MHz harmonised for MFCN”, approved 18 November 2022.

* First change *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

[y] 	ECC Decision (22)07: “Harmonised technical conditions for the usage of aerial UE for communications based on LTE and 5G NR in the bands 703-733 MHz, 832-862 MHz, 880-915 MHz, 1710- 1785 MHz, 1920-1980 MHz, 2500-2570 MHz and 2570- 2620 MHz harmonised for MFCN”, approved 18 November 2022.

* Second change *

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

No-Transmit Zone: a geographical area where UAV is not allowed to transmit user data and signal for spectrum compatibility purposes.

* Third change *

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

NTZ	No-Transmit Zone

* Fourth change *

# 5	Key Issues

## 5.x	Key Issue #x: No-Transmit Zones Support

### 5.X.1	Description

No-Transmit Zone (NTZ) is a geographical area where UAV is not allowed to transmit user data and signal for spectrum compatibility purposes. The requirements related to NTZ are specified in [y].

To support the NTZ, the following aspects need to be addressed for this key issue:

-	Which NF is configured with the no-transmit zones related information, i.e. geographical area and the restricted frequency band of the no-transmit zones.

-	Which NF is responsible to convert the geographical area of the no-transmit zones into 3GPP identifiers (e.g. cell(s), TA(s)) and how, especially when the no-transmit zone does not match the 3GPP identifiers completely, e.g. the no-transmit zone is much smaller than a cell.

-	Whether and how the UAV obtains the no-transmit zone related information.

-	What is UAV behaviour when the event that flying into, existing in and flying out of the no-transmit zone is detected, e.g. whether the UAV can receive DL data and signal when the event that flying into or existing in the no-transmit zone happens.

NOTE x:	Clarification on the scenario that the UAV is in the no-transmit zone is that when a new no-transmit zone is added, there may be some UAV already exists in the new no-transmit zone.

-	Which NF is responsible to detect the event that UAV flying into, exist in and flying out of the no-transmit zone. If the event is detected, how does the network ensure that the UAV obey the specified UAV behaviour for no-transmit zone. If the UAV does not obey the specified behaviour is detected, how to control UAV behaviour.

-	Whether it is needed to expose no-transmit zone related UAV event to UTM/USS.

* End of changes *