SA WG2 Meeting #160	S2-2313364

# November 13th – 17th, 2023; Chicago, US               	       (revision of S2-2312988)

Source: 	CATT, Thales, Novamint, China Telecom, Spreadtrum, vivo, TNO, Eutelsat, Nokia, Nokia Shanghai Bell, NEC

Title: 	FS_5GSAT_Ph3_ARCH_Key issues for WT#1

Document for: 	Approval

Agenda Item: 	19.1

Work Item / Release:	FS_5GSAT_ARCH_Ph3 / Rel-19

Abstract of the contribution: this paper proposes a key issue for the WT#1 of SID FS_5GSAT_ARCH_Ph3.

- Discussion

The Work Task#1 of the SID is:

WT1: Regenerative payload generic architecture study

WT-1.1: Study and identify any impacts on 5GS and EPS for the scenario with gNB/eNB embedded on the satellite.

In fact, a key issue for this WT was already identified in Rel-16 TR 23.737, but was not studied:

“5.6	Key Issue #6: RAN mobility with NGSO regenerative-based satellite access

5.6.1	General description

Embarking RANs on-board moving NGSO satellites implies frequent handovers of these RANs for any given 5CN, as well as their interconnection through an Xn interface.

As NGSO satellites coverage spans a large coverage, and can include a significant number of UEs, a large number of UE’s could be simultaneously handed over from one RAN to another one, leading to a group handover of the anchor point be the RAN and the CN.

Under these circumstances, this key issue shall address the following points:

-	What are the conditions for a 5CN to handover from a RAN to another RAN?

-	What are the requirements on the Xn and on the NG interfaces to ensure the mobility of the RAN with respect to the 5CN?

-	What are the functional requirements on the 5CN and RAN to ensure the mobility of the RAN with respect to the CN?

Existing / proposed procedures from other topics should also be reviewed.

”

Therefore, it is proposed to use this key issue as the baseline in this study.

Apart from existing issues, we have following observations:

Observation 1: RAN mobility will cause much frequent disconnection and re-connection of TNL over N2/S1-AP interface.

As RAN node mobility, in case RAN nodes are boarded on satellite, will cause much frequent disconnection and re-connection of TNL over N2/S1-AP interface of an AMF/MME, it is also proposed to study how to efficient manage N2/S1-AP connections. For example, in the following figure, when the satellite disconnects the feeder link towards the AMF in the PLMN, all DL signaling from the AMF to the gNB on the satellite will not be delivered successfully, but the AMF doesn’t know it, so the AMF needs to rely on low layer time out mechanism to know the failure of TNL (normally, the period of heart beat on SCTP layer is 30 sec, the retransmission interval could be 1 to 60 sec), and would still send DL signalling/message to the RAN node which has gone, which causes unnecessary signaling cost and delay.

In addition, when the SCTP endpoint detect the path failure based on heart beat time out, it will close the SCTP association and delete the transmission control block, which means that, when the satellite flies back to the PLMN A, it has to re-establish the SCTP association via four-way handshake. If all satellites need to re-establish the TNL toward the AMF, there will be lots for signaling towards the AMF on the ground.

Observation 2: RAN mobility will cause the change of RAN nodes serving a TA(s)

RAN mobility will also cause the change of RAN nodes serving a TA(s), no matter whether the RAN node has earth fixed cells or earth moving cells, thus for paging a UE in a TA(s), the AMF need to know which RAN node should page the UE over the air interface. For example, in the following figure, the TA2, 5 and 7 served by satellite 1 will be served by satellite 2 at some time later, so the AMF needs to contact gNB on the satellite 2 to page UE in the TA 2, 5 and 7 at that time.

Observation 3: RAN node on the satellite will be subject to dynamic backhaul.

At last, due to ISL, the RAN node on the satellite will be subject to dynamic backhaul as specified in Rel-18 5GSATB, so such RAN node will be subject to dynamic backhaul delay, especially when the backhaul is over cross-seam ISLs, but it would not require upper layer control plane protocol adaption as it is assumed that the maximum delay between UE and 5GC/EPC would not exceed 100 ms even for NGSO satellites with ISL.

2 Proposal

It is proposed to update TR 23.700-29 for FS_5GSAT_ARCH_Ph3 as followed.

# * * * * First Change * * * *

5	Key Issues

5.X	Key Issue #X: Support of Regenerative-based satellite access

5.X.1	Description

The deployment of an eNB or an gNB on a satellite for regenerative based satellite access for LEO/MEO deployment is to be studied. A number of example network scenarios RAN nodes on-board moving satellites are illustrated in Annex A. The aspects to be studied includes:

-	Identify and study whether there is any impact on 5GS and EPS to support an gNB and eNB embedded on a satellite.

# -	any impact of RAN nodes changing for any given 5GC/EPC and for a given area in the case of RAN nodes moving.

# * * * * Next Change * * * *

Annex A:
Example network scenarios for regenerative payload generic architecture (e/gNB on LEO satellite)

The following diagrams are examples of scenarios and situations that the 3GPP standards should handle in order to offer good service to UEs when an eNB/gNB is deployed on a LEO satellite.

These examples are not intended to be an exhaustive set.

Figure A-1: Basic concept of eNB/gNB on LEO satellite

Figure A-2: Handover between satellites with common earth station location

In figure A-2, the UE is basically stationary; the satellite with eNB X is going down in elevation angle, but before X becomes unusable, the satellite with eNB Y rises above the horizon and becomes usable. Again, using methods already discussed in RAN WGs during Release 17/18, the UE can be handed over (e.g. from cell B-a to cell Y-c) during the time that the UE's area is in coverage of both satellites. This handover could be either an X2 or an S1 handover, but note that NB-IoT devices do not support handover.

Figure A-3: Feeder link switch with 2 eNBs/gNBs on LEO satellite

In figure A-3, the UE is basically stationary, but as the satellite flies over, the satellite is moving out of range of one earth station (ES1) and into range of another earth station (ES2). At some time before time T, the satellite system's O&M activates a second eNB (eNB-Z) on the satellite and the S1 interfaces from eNB-Z through ES2 are established at time T. Using methods already discussed in RAN WGs during Release 17/18, the UE can be handed over (e.g. from cell X1-a to cell Z2-b) in the period between time T and time T+k. This handover could be an S1 handover, or, if the eNBs know that they are connected to the same MME, it could be an X2 handover. Note that NB-IoT devices do not support handover.

Figure A-4: Handover between satellites using different earth station location

Figure A-4 is a combination of the preceding scenarios but needs to be supported.

Figure A-5: eNB migration between satellites

In figure A-5, the UE is basically stationary; the satellite X with eNB X is going down in elevation angle, but before X becomes unusable, the satellite Y rises above the horizon and becomes usable. Relying on virtualized satellite computing platform, the eNB X embarking on the satellite X will be migrated to the satellite Y at this time. After the context of eNB X is completely transferred to the satellite Y, the service link for Uu of a cell of eNB X and feeder link for S1-C/S1-U of eNB X will be changed from satellite X to satellite Y. With this approach, the UE will always receives cell X from eNB X, as it the eNB X doesn’t move. How to migrate eNB from satellite X to satellite Y is not within 3GPP scope.

# * * * * End of Change * * * *