3GPP TSG-SA WG2#161	S2-2403064

Athens, Greece, 26 February-1 March 2024	(was S2-2402117)

Source:	Qualcomm Incorporated, Huawei

Title:	23.700-84: KI#1: Solution about LMF based ML model training and Inference

Document for:	Discussion/Approval

Agenda Item:	19.15

Work Item / Release:	FS_AIML_CN / Rel-19

Abstract of the contribution: This contribution proposed a LMF based ML model training and Inference solution for KI#1.

# 1.	Discussion

This contribution proposed a LMF based ML model training and Inference solution for KI#1.

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-84:

# >>>>BEGINNING OF CHANGES<<<<

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[6]	3GPP TR 38.843: "Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface".

[x]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

# >>>>NEXT CHANGE (new text) <<<<

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues and Use Cases

|  | Key Issues |  | Use cases (optional) |  |
| --- | --- | --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> | <use case #x> | <use case #y> |
| x | x |  |  |  |

## 6.X	Solution #X: LMF based ML model training and Inference

### 6.X.1	Description

This is a solution proposed for KI#1: Enhancements to LCS to support Direct AI/ML based Positioning.

In order to support the Direct AI/ML based positioning for case 2b, 3b, the LMF is required to be enhanced to support the AI/ML- based  positioning. How LMF performs ML model training and inference, is based on implementation (out of scope of 3GPP).

LMF may collect data from UE, NG-RAN or NWDAF for model training and inference. LMF collects data from UE via either CP or UP solution with LPP protocol as described in TS 23.273 [x], LMF collects data from NG-RAN via NRPPa protocol as described in 23.273. LMF may also request NWDAF to expose some information that supported in TS 23.288 [5] to assist the AIML based positioning. How LMF uses the received parameters to perform ML model training and inference is based on implementation (out of scope of 3GPP).

Editor’s NOTE:	Whether and which existing analytics or new analytics result is required to provide from NWDAF to LMF to support Direct AI/ML based Positioning  in Rel-19 is FFS.

### 6.X.2	Procedures

This is the data collection procedure to support ML model training and inference for AI/ML based positioning in LMF.

Figure 6.X.2-1	Data collection for LMF based ML model training and inference

- LMF requests from the UE to report the positioning measurement parameters as described in TS 23.273 [x]. UE reports positioning measurement parameters by using LPP protocol via either the CP or UP.

- LMF requests NG-RAN to report the positioning measurement parameters as described in TS 23.273 [x]. NG-RAN reports positioning measurement parameters by using NRPPa protocol.

- LMF requests NWDAF to expose the analytics result, for example, location Accuracy Analytics as described in clause 6.17 in TS 23.288 [5], UE mobility analytics in clause 6.7 in TS 23.288 [5].

Editor’s NOTE:	Whether and which existing analytics or new analytics result is required to provide from NWDAF to LMF to support Direct AI/ML based Positioning  in Rel-19 is FFS.

NOTE 1:	There is no sequence of these 3 steps, LCS may trigger these steps in parallel.

NOTE 2:	The collected measurement parameters from UE in step 1 and NG-RAN in step 2 is determined by RAN group. If there are new measurement parameters that provided by UE or NG-RAN in Rel-19 depends on RAN’s progress in Rel-19.

LMF will use the received parameters from UE, NG-RAN and NWDAF to perform ML model training or inference, and it will generate the target UE positioning as inference output.  How LMF uses the received parameters to perform ML model training and inference is based on implementation (out of scope of 3GPP).

### 6.X.3	Impacts on services, entities and interfaces

NWDAF

Editor’s NOTE:	Whether and which existing analytics or new analytics result is required to provide from NWDAF to LMF to support Direct AI/ML based Positioning  in Rel-19 is FFS.

LMF

NOTE 3:	The collected measurement parameters from UE in step 1 and NG-RAN in step 2 is determined by RAN group. If there are new measurement parameters that provided by UE or NG-RAN to LMF in Rel-19 depends on RAN’s progress in Rel-19.

# >>>>END OF CHANGES<<<<