SA WG2 Meeting #162	 S2-2404136

# 15 April – 19 April 2024, Changsha, China

Source:	Lenovo

Title:	Update to Solution 24 to address ENs.

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM_Ph2 / Rel-19

Abstract of the contribution:

# 1		Discussion

Answers are provided for the following ENs are added in this solution:

Editor's Note 1:

Editor's note:	It is FFS whether UPF establishes a tunnelled connection on per UE basis or whether the tunnelled connection is shared for traffic for multiple UEs that have established a session with the same application server.

Answer:	This would be up to the UPF (HTTP/3 client) implementation. If the UPF determines that there is already a tunnel open towards the XR application server (e.g. from another UE XR session), the UPF may forward the UE packet via the existing connection. There is no issue on how the UPF routes received packets over the core network as the UPF would route the received packet based on its inner header (rather than the outer IP header information).

Editor's Note 2:

Editor's note:	It is FFS if there is user experience degradation if step 6 (application sending a packet towards the content server address) is carried out before the SMF configures N4 rules to the UPF indicating to establish a connect-UDP connection with an AS (due to control plane signalling latency).

Answer:	This race condition could also occur currently and can be considered as a corner case. For example, when the AF requests establishment of an AF session with specific PDU-set requirements, initial XR packets between UE and network may be routed via the default PDU session until the UPF receives the N4 routes to route packets via a PDU-Set QoS flow.

Editor's Note 3:

Editor's note:	Whether and how IETF standardization is required for the HTTP Datagram is FFS.

Answer:	The proposal is to have SA4 to define the HTTP Datagram that contains PDU Set Information. The EN is rephrased.

Editor's Note 4:

Editor's note:	It is FFS whether and how the UPF can determine to establish a UDP tunnel to the AS by sending an HTTP request with the "connect-udp" upgrade token especially when the same EAS IP address would host XRM and non XRM based applications or when there two XRM service instances on the same UE and they use the same UE IP address for the same service.

Answer: For both scenario 1 and scenario 2 the nature of the Connect-UDP protocols can resolve the underlined issues. The Connect-UDP request includes the following info:

-	Method: CONNECT

-	Protocol: connect-UDP

-	Path= /target.example.com/443 (i.e. content server address)

The HTTP proxy can resolve the target server address by processing the "path" info and identifying how to route the packet to the correct server address. In case both XR and non-XR services then the traffic of non-XR services can be routed via the QUIC tunnel. In the latter scenario no HTTP Datagrams are added in the QUIC connection.

# 2		Proposal

The following updates are proposed.

********************************************** First change ****************************************

## 6.24	Solution #24: PDU set identification when an end-to-end XR session is fully encrypted using a tunnelled connection over N6

### 6.24.1	Key Issue mapping

This solution addresses key issue #2.

### 6.24.2	Description

The transport of multimedia application data (e.g. AR/VR/XR, interactive and streaming services, web applications) is often conveyed over RTP protocol.

For supporting XR media over 3GPP, 3GPP defined extensions to the RTP headers allowing an XR media application server to provide additional information to assist the 3GPP network to identify a group of packets (PDU set identification) and delivering such packets with specific QoS requirements (e.g. PDU Set Delay Budget) to a UE. Such approach requires the RTP headers to be sent unencrypted so as the UPF within the 5G network to identify PDU set information. However, the industry is now focusing on fully encrypting the media packet including the protocol headers either by using solutions relying on QUIC (RFC 9000 [11]) or by enhancing the RTP protocol with fully encrypted headers as described in RFC 9335 [7].

Consequently, there is a need to allow a solution to support PDU-set identification when the end-to-end connection between the UE and the Application server is fully encrypted.

The solution proposes to have a framework where fully encrypted packets from an application server are tunnelled via an encapsulation protocol between the UPF and the Application Server over the N6 reference point as shown in the Figure below.

Figure 6.24.2-1: General framework for supporting PDU set identification for fully encrypted media packets

The main steps of the solution comprise of the following elements:

1.	An AF when requesting an AF session towards the 3GPP network (NEF) includes additional information indicating to the 3GPP network that the UPF can identify PDU-set information by inspecting the headers of an encapsulation protocol. The AF also including the address of the XR Video application server where the UPF will need to establish a connection using the encapsulation protocol.

NOTE 1:	It is assumed that the 3rd party provider of the AF and the 3GPP network have SLA agreement on the type of encapsulation protocol to use between the UPF and AS over N6 reference point.

2.	The PCF in the 3GPP network providing PCC rules where the PCC rules includes the information provided by the AF in step 1.

3.	The SMF constructing N4 rules based on the PCC rules where the N4 rules include uplink and downlink Packet Detection Rules indicating to the UPF that:

a.	for uplink packets sent from a specific UE (or any UE) to a specific application server the UPF would need to establish a connection using an encapsulation protocol and route the packet via the encapsulation protocol to the Application Server address.

b.	For downlink packets received by the UPF over N6 configuration information indicating the UPF to:

i.	Extract the encapsulated UDP packet from the received IP packet.

ii.	Enable PDU set identification and retrieve PDU set information from information contained within the encapsulation protocol.

iii.	Routing the de-encapsulated UDP packet over a QoS flow with PSDB requirements within GTP signalling towards the RAN.

4.	UPF that supports an encapsulation protocol client establishing a protocol session with a server once the UPF detects that an uplink packet is sent towards a specific destination address.

NOTE  X:	If the UPF determines that there is already a tunnel open towards the XR application server (e.g. from another UE XR session), the UPF can forward the UE packet via the existing connection. The decision is based on UPF implementations or operator configuration.

5.	The Application Server sending downlink packets via the encapsulation protocol and includes additional PDU set information provided in-band within the encapsulation protocol to assist the UPF to identify PDU set information.

6.	The UPF identifying PDU set information of PDUs of PDU sets based on the information provided within the encapsulation protocol and routing the packet via a QoS flow with PSDB requirements based on the N4 rules including within the GTP-U header PDU set information for each PDU.

NOTE 2:	It is up to UPF implementation when the tunnelled connection is released, e.g. when the UPF determines no traffic is routed via the tunnelled connection for a specific period of time then the connection is released. Other solutions are FFS.

### 6.24.3	Procedures

#### 6.24.3.1	General procedure

Details steps of the procedure are shown in Figure below:

Figure 6.24.3.1-1: General procedure for supporting PDU set identification for fully encrypted media packets

1.	An AF requests to establish an AF session with QoS by invoking the Nnef_AFSessionWithQoS Create service operation as described in clause 4.15.6.6 of TS 23.502 [3] including PDU Set QoS parameters for the XR service. The AF additionally includes information to enable an encapsulation protocol connection between the UPF and AS and the address of the server where the UPF can establish the encapsulation protocol session.

Not shown in the Figure the NEF authorizes the request and forwards the request to the PCF by invoking an Npcf_PolicyAuthorization_Create request including the information provided by the AF.

2.	The PCF creates PCC rules taking into account the PDU Set QoS parameters as described in clause 6.1.3.22 of TS 23.503 [4]. The PCC rule also include the encapsulation protocol details and server address.

3.	The SMF creates Packet Detection Rules (N4 rules) for the UPF. The Uplink Packet Detection Rule include an indication to the UPF to establish an encapsulation protocol session with the server address (provided by the AF) when the UPF detects that a packet is sent towards the server address. The downlink Packet Detection rule include configuration information to enable PDU set inspection and extract PDU-set information from information contained within the encapsulation protocol.

4.	The N4 rules are sent to the UPF

5.	An application in the UE is triggered to connect to the AS

6.	The application sent an application packet via uplink over the 3GPP network

7.	The UPF inspect the packet and determines that there is a matching PDR rule indicating to establish an encapsulation protocol session to a server address and route the uplink packet via the encapsulation protocol

8.	The UPF sends a session request to establish a connection using the encapsulation protocol procedure

9.	The server acknowledges

10.	The UPF encapsulates the uplink packet within the encapsulation protocol header.

11.	The session packet is sent to the AS via N6

12.	Further downlink and uplink packets may be routed to fully encrypt the connection between the UE and AS

13.	The Application Server determines PDUs belonging to PDU set and adds PDU set information within the encapsulation protocol

14.	The UDP packet is sent via the encapsulation protocol to the UPF

15.	The UPF extracts the UDP packet and determines PDU set information using the PDU set information provided within the encapsulation protocol.

#### 6.24.3.2	Using Connect-UDP

The architecture when using Connect-UDP protocol between the UPF and AS is shown in Figure 6.24.3.2-1.

Figure 6.24.3.2-1: Using Connect-UDP or HTTP/3 proxy for proxying fully encrypted media packets via N6

The main procedure using QUIC/Connect-UDP is as follows:

1.	An AF when requesting an AF session towards the 3GPP network (NEF) includes additional information indicating to the 3GPP network that the UPF need to establish a QUIC/Connect-UDP session with a HTTP/3 proxy and indicate that the UPF can identify PDU-set information by inspecting PDU-set information contained within HTTP datagrams. The AF also including the address of a HTTP/3 proxy server where the UPF will need to establish a QUIC session.

2.	The PCF in the 3GPP network providing PCC rules where the PCC rules includes the information provided by the AF in step 1.

3.	The SMF constructing N4 rules based on the PCC rules where the N4 rules include uplink and downlink Packet Detection Rules indicating to the UPF that:

a.	for uplink packets sent from a specific UE (or any UE) to a specific application server the UPF would need to establish a QUIC/Connect UDP session and route the packet within a HTTP Datagram to the Application Server address.

b.	For downlink packets received from the QUIC connection by the UPF over N6 include configuration information indicating the UPF to:

i.	Extract the UDP packet from the received HTTP Datagram.

ii.	Enable PDU set identification and retrieve PDU set information from HTTP Datagram. The HTTP Datagram includes a new Context ID indicating PDU-set information.

iii.	Routing the extracted UDP packet over a QoS flow with PSDB requirements towards the RAN.

4.	The UPF that supports an HTTP/3 client establishing a QUIC connection with an HTTP/3 proxy when an uplink packet is received that matches the uplink PDR provided by the SMF.

5.	The AS/HTTP/3 Proxy includes downlink packet within a HTTP Datagram using a transport mode which is an extension of the mode defined in RFC 9298 [38]. The AS/QUIC includes the encrypted UDP packets within HTTP Datagram frames and provides unreliable transport and includes PDU set information of the encrypted packet within the HTTP Datagram. A new Datagram mode needs to be defined by using the Context ID (defined in RFC 9298 [38]) indicating PDU set information.

6.	The UPF extracting the UDP packet from received HTTP Datagram frames, determining to enable PDU set inspection based on configuration rules provided by the SMF and identifying PDU set information of the extracted PDU by checking the headers of the HTTP Datagram. The UPF then routing the extracted packet via a QoS flow with PSDB requirements based on configuration rules from the SMF and sending the extracted packet towards the RAN within a GTP packet including within the GTP-U header the retrieved PDU set information.

NOTE 1:	It is up to UPF implementation when the QUIC connection is released, e.g. when the UPF determines no traffic is routed via the QUIC connection for a specific period of time then the connection is released. Other solutions are FFS

Details steps of the procedure are shown in Figure 6.24.3.2-2 below:

Figure 6.24.3.2-2: Using Connect-IDP for supporting PDU set identification for fully encrypted media packets

1.	An AF requests to establish an AF session with QoS by invoking the Nnef_AFSessionWithQoS Create service operation as described in clause 4.15.6.6 of TS 23.502 [3] including PDU Set QoS parameters for the XR service. The AF additionally includes information to enable HTTP/3 proxy connect-UDP session between the UPF and AS and the address (IP address and port) of the HTTP/3 server where the UPF can establish the HTTP/3 session. Not shown in the Figure the NEF authorizes the request and forwards the request to the PCF by invoking an Npcf_PolicyAuthorization_Create request including the information provided by the AF. The AF is aware of the address of the UE via previous interaction between the UE and AF/AS.

2.	The PCF creates PCC rules taking into account the PDU Set QoS parameters as described in clause 6.1.3.22 of TS 23.503 [4]. The PCC rule also include the HTTP/3 proxy connect-UDP protocol information and server address.

3.	The SMF creates Packet Detection Rules (N4 rules) for the UPF. The Uplink Packet Detection Rule include an indication to the UPF to establish an HTTP/3 connect-UDP protocol session with an HTTP/3 proxy server address (provided by the AF) when the UPF detects that a packet is sent towards a content server address and the origin address is the address of the UE (the origin address of the UE may be omitted). The downlink Packet Detection rule include configuration information to enable PDU set inspection and extract PDU-set information from information from the received HTTP Datagram frame.

4.	The N4 rules are sent to the UPF.

5.	An application in the UE is triggered to connect to the AS.

6.	The application sent an application packet towards a content server address via uplink over the 3GPP network.

7.	The UPF inspect the packet and determines that there is a matching PDR rule indicating to establish an HTTP/3 session to a specific HTTP/3 proxy server address and route the uplink packet via the HTTP/3 session.

8.	The UPF (HTTP/3 client in the UPF) sends a CONNECT request using CONNECT-UDP protocol to the proxy with target address the address (IP address/port) of the content server.

9.	The server acknowledges the request sending a STATUS-200 message.

10.	The UPF includes the UDP packet received from the UE within a HTTP Datagrams and forwards to the HTTP/3 proxy.

11.	The HTTP Datagram is sent to the HTTP/3 proxy via N6. The proxy extracts the UDP packets and forwards to the content server address according to the target address included in step 8 within the Connect-UDP protocol. The HTTP proxy resolves the target address based on the Path information included in the Session request using Connect-UDP protocol.

NOTE 2:	Content server address may be collocated with the HTTP/3 proxy.

NOTE 3:	In case the content server supports both XR and non-XR related services then the traffic of non-XR services is routed via the QUIC connection over N6.

12.	Further downlink and uplink packets may be routed to fully encrypt the connection between the UE and content server.

13.	The content server/HTTP/3 proxy determines PDUs belonging to PDU set and determines PDU set information. The PDU set information is added within HTTP datagram. A new Context ID is defined that indicates a new datagram mode for PDU set information.

Editor's note:	The HTTP Datagram containing PDU Set information will be defined in 3GPP SA4.

14.	The HTTP datagram is sent to the UPF (HTTP/3 client) via N6. A new Transport Mode is defined for the HTTP Datagram that includes a Context ID denoting additional PDU set information included within the HTTP Datagram.

15.	The UPF extracts the UDP packet from the HTTP datagram and determines PDU set information using the PDU set information provided within the HTTP datagram. The UPF then routes the packets towards the RAN within a GTP packet including within GTP-U header the identified PDU set information.

### 6.24.4	Impacts on services, entities and interfaces

UPF:

-	UPF supporting an encapsulation protocol, e.g. Connect-UDP.

******************************** End of change *******************************