SA WG2 Meeting #162	S2-2404651

# Changsha, China, 15th – 19th April, 2024                   (revision of S2-2402167)

Source: 	CATT

Title: 	KI#2, New Sol: Authorization of UE acting as an intermediate node

Document for: 	Approval

Agenda Item: 	19.14

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper is to propose a new solution to support Authorization of UE acting as an intermediate node.

1. Proposal

It is proposed to agree the following changes into TR 23.700-13.

1st Change

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | Key Issue #1 | Key Issue #2 | Key Issue #3 |
| #1 |  | X |  |
| #2 |  | X |  |
| #3 | X | X | X |
| #4 | X | X | X |
| #5 |  | X | X |
| #6 | X | X | X |
| #7 | X | X |  |
| #8 | X | X |  |
| #9 | X | X | X |
| #10 | X | X |  |
| #11 |  |  | X |
| #12 | X | X | X |
| #X |  | x |  |

2nd Change

6.X	Solution #X:  Authorization of UE acting as an intermediate node

6.X.1	Description

The solution applies to the Key Issue #2 "Identification, Subscription, Registration and Connection management".

The UE acting as an intermediate node needs to be authorized by network to communicate with the AIoT Devices. The UE provides AIoT Communication Capability to the AMF, and if it is authorized the authorized information is provided by UDM or PCF via the AMF to the UE.

6.X.2	Procedures

The following clauses 6.X.2.1 and 6.X.2.2 describe the procedures for UDM based authorization of UE acting as an intermediate node and PCF based authorization of UE acting as an intermediate node respectively.

6.X.2.1	Procedure for UDM based authorization of UE acting as an intermediate node

Depicted in Figure 6.X.2-1 is the procedure for UDM based authorization of UE acting as an intermediate node.

Figure 6.X.2.1-1: Procedure for UDM based authorization of UE acting as intermediate node

The UE acting as an intermediate node performs Registration procedure as described in clause 4.2.2.2.2 of TS 23.502 [5], and the following modifications are made:

1.	The UE acting as an intermediate node includes AIoT Communication Indication in the Registration Request message. AIoT Communication Indication indicates whether UE supports communication with AIoT Devices.

The UE may also indicate an AIoT Communication Indication to the AMF via RRC message and N2 message.

2.	The AMF retrieves subscription data of the UE from UDM and checks whether the UE is authorized to communicate with the AIoT Devices.

3.	If the UE is authorized to communicate with the AIoT Devices, the AMF includes authorized information in the Registration Accept message to the UE.

The AIoT authorized information may also be provided to the NG-RAN.

6.X.2.2	Procedure for PCF based authorization of UE acting as an intermediate node

Depicted in Figure 6.X.2.2-1 is the procedure for PCF based authorization of UE acting as an intermediate node.

Figure 6.X.2.2-1: Procedure for PCF based authorization of UE acting as intermediate node

The UE acting as an intermediate node performs Registration procedure as described in clause 4.2.2.2.2 of TS 23.502 [5], and the following modifications are made:

1.	The UE acting as an intermediate node includes AIoT Communication Indication in the Registration Request message. AIoT Communication Indication indicates whether UE supports communication with AIoT Devices.

The UE may also indicate an AIoT Communication Indication to the AMF via RRC message and N2 message.

2.	The AMF sends Registration Accept message to the UE.

3.	If the UE indicates AIoT Communication Indication in the Registration Request message and if the UE is authorized to communicate with AIoT Devices based on the subscription data, the AMF reports the authorized AIoT Communication Indication to the PCF.

4.	The PCF responds to the AMF.

5.	The PCF invokes Namf_Communication_N1N2MessageTransfer service operation to the AMF, and the message includes authorized information for the UE.

6.	The AMF transfers the authorized information to the UE.

The AIoT authorized information may also be provided to the NG-RAN.

6.X.3	Impacts on services, entities and interfaces

UE

-	The UE indicates AIoT Communication Indication to the AMF and receives AIoT authorized information from the AMF.

AMF

-	The AMF receives AIoT Communication Indication from the UE and provides AIoT authorized information to the UE.

UDM

-	The UDM stores subscription data for UE communicating with AIoT Devices.

PCF

-	The PCF provides authorized information to the UE via the AMF.

End of Changes