3GPP TSG-SA WG2 Meeting #162	S2-2404044

Changsha, China, April 15 – April 19, 2024

Title:	Reply LS on LCS user plane connection binding to the UE

Response to:	LS on LCS user plane connection binding to the UE (C1-241722)

Release:	Rel-18

Work Item:	5G_eLCS_Ph3

Source:	SA2

To:	CT1

Cc:	CT4

Contact Person:

Name:	Amy Zhang

Tel. Number:

E-mail Address:	amy.zhang@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.273 CR#0515

1. Overall Description:

SA2 would like to thank CT1 for the LS on LCS-UP binding issue between the LMF and the UE.

Regarding the issues mentioned in the LS, SA2 believes:

- the LCS-UP binding over TLS needs to be supported so that the LMF can link one secured user plane connection with one UE.

- the LMF also needs to be enabled to reuse the same secured user plane connection when receiving the subsequent location service for the same UE.

In order to address the above requirements, SA2 has approved the attached CR for TS 23.273 to provide a resolution for the LCS-UP binding issue:

- When creating a new user plane connection between the LMF and the UE, the LMF allocates and provides a unique LCS-UP connection ID in the user plane information to UE. The UE uses LCS-UP connection ID to enable LMF to bind the secured user plane connection with UE during step 7 of clause 6.18.1 of TS 23.273 or step 6 of clause 6.18.2 of TS 23.273. How to achieve the LCS-UP binding over TLS is left for CT1 design. After the secured user plane connection is successfully established, the LCS-UP connection ID is stored as part of LCS-UP connection context on the AMF side.

- When the subsequent location service for the same UE comes to the AMF, the AMF selects the LMF that maintains the secured user plane connection with the UE. And the AMF delivers the LCS-UP connection ID to the LMF in Nlmf_location_determinelocation service, to enable the LMF to reuse the secured user plane connection.

2. Actions:

To CT1

ACTION: 	SA2 kindly asks CT1 to take the above information into consideration.

3. Date of Next SA2 Meetings:

SA2#163	27th   – 31th May 2023 		Korea

SA2#164	19th   – 23th August 2023 		Maastricht , NL