SA WG2 Meeting #162	S2-2404967

# April 15 – April 19, 2024, Changsha, China

Source: 	CableLabs, Comcast

Title: 	New Solution for KI#1.3: session management procedures for switching between the ATSSS and DualSteer Features

Document for: 	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This pCR proposes a new solution for KI#1.3 on Session Management procedures for switching between the ATSSS and DualSteer Feature based on extensions of the ATSSS MA PDU Session

# 1	Discussion

This paper proposes a new solution for session management procedures for switching between the ATSSS and DualSteer Feature for KI#1.3 based on extensions of the ATSSS MA PDU Session. It addresses a category of UEs supporting both ATSSS and DualSteer features. It leverages the existing multi-access (MA) PDU session procedures defined for ATSSS and defines similar multi-access (MA) PDU session procedures for DualSteer. For more details on the DualSteer related updates please refer to our companion contribution (S2-2404966).

This solution minimizes the impact on the existing Registration procedures. It only uses the indication of the DualSteer support by the network within the Registration accept message. The 5GS network feature support informational element is used by the network to indicate whether it supports ATSSS, DualSteer or both. If network supports both, it should also indicate the preferred functionality – ATSSS or DualSteer. The individual registrations across both the 3GPP access and non-3GPP access network are independent and the ATSSS or DualSteer policies and rules are applied considering the network availability.

This defines procedures for the ATSSS UE to replace the non-3GPP access lwith another 3GPP access (transitioning from ATSSS to DualSteer) and the DualSteer UE to replace one of the 3GPP access with a non-3GPP access (transitioning from DualSteer to ATSSS), assuming both the UE and the network support both ATSSS and DualSteer features, and the UE has access to the respective networks.

The solution is discussed in the context of the following scenarios:

- Two NR/5GC accesses in two different PLMNs (HPLMN/VPLMN) with each access being NR TN and a non-3GPP access in the HPLMN

- Two NR/5GC accesses in two different PLMNs (HPLMN/VPLMN) with each access being NR TN and a non-3GPP access in the VPLMN

- Two NR/5GC accesses in two different PLMNs (HPLMN/VPLMN) with each access being NR TN and a non-3GPP access in a different VPLMN

The solution is also applicable for the following scenarios:

- Two NR/5GC accesses with each access being NR TN and a non-3GPP access in same PLMN

- Two NR/5GC accesses with each access being NR TN in same PLMN and a non-3GPP access in different PLMN

# 2. Proposal

It is proposed to adopt the following text in TR 23.700-54.

*** Start of 1st change (all new text) ***

### 6.1.X	Solution #X: Switching between ATSSS and DualSteer Feature

#### 6.1.X.1	Introduction

This solution applies to UE devices supporting both ATSSS and DualSteer features. The proposal attempts to leverage the existing multi-access (MA) PDU session procedures defined for ATSSS and define similar multi-access (MA) PDU session procedures for DualSteer.

The proposal has no impacts to the existing Registration procedures except for indicating the DualSteer support by the network within the Registration accept message indicating support for DualSteer feature using the 5GS network feature support informational element. Within this IE, the network should indicate whether it supports ATSSS, DualSteer or both. If network supports both, it should also indicate its preference either ATSSS or DualSteer. The individual registrations across the 3GPP access networks and non-3GPP access network are independent and the ATSSS or DualSteer policies and rules are applied considering the network availability.

The proposed solution defines procedures for the ATSSS UE to replace the non-3GPP access with another 3GPP access (transitioning from ATSSS to DualSteer) and the DualSteer UE to replace one of the 3GPP access with a non-3GPP access (transitioning from DualSteer to ATSSS). It assumes that both the UE and the network support both ATSSS and DualSteer features, and the UE has access to the respective networks.

##### 6.1.X.1.1 Multi-access PDU connectivity service activation for UE that supports both ATSSS and DualSteer feature

The UE initiates MA PDU activation across two 3GPP access networks or a 3GPP and a non-3GPP access network or a single network (3GPP or non-3GPP) based on the feature support for ATSSS and DualSteer by the UE and the network, network availability and the network preference as indicated by the network in Registration Accept.

If the two 3GPP networks (in case of DualSteer) or the 3GPP and non-3GPP network (in case of ATSSS) are in same PLMN, the UE initiates the MA PDU session establishment procedures on the selected access to establish user plane resources across the access networks. If the two 3GPP networks (in case of DualSteer) or the 3GPP and non-3GPP network (in case of ATSSS) are in different PLMN, the UE initiates the MA PDU session establishment procedures on the available accesses (sequentially) to establish user plane resources across the access networks.

- If the UE is registered across all the three networks, two 3GPP networks and a non-3GPP network, the UE initiates the MA PDU session establishment procedures, either for DualSteer or ATSSS based on the network preference, on the selected access (based on UE implementation).

- If the UE is registered across two networks, either two 3GPP networks or one 3GPP and a non-3GPP network, regardless of the network preference, the UE initiates the MA PDU session establishment procedures using both accesses, either for DualSteer or ATSSS.(based on UE implementation). If the UE at a later point in time, registers to another access then the UE decides either to transition to DualSteer if the existing MA PDU Session was established across a 3GPP and a non-3GPP network or transition to ATSSS if the existing MA PDU Session was established across two 3GPP networks, based on feature support indication by network and the network preference.

- If the UE is registered across a single access, either 3GPP or non-3GPP, then:

  - If the registered access is a non-3GPP access, then the UE initiates the MA PDU session establishment procedure based on feature support indication by the network. When the UE registers to a 3GPP access at a later point in time, it initiates the MA PDU session establishment procedures over the 3GPP access using the same PDU Session ID. If the UE registers further to another 3GPP access at a later point in time, the UE will decide whether to switch from ATSSS to DualSteer based on the network preference.

  - If the registered access is a 3GPP access, the UE initiates a single access PDU session establishment.

    - If the UE, at a later point in time registers to a non-3GPP access, the UE upgrades the established PDU Session to MA PDU session on the registered 3GPP access and initiate the MA PDU session establishment procedures on the non-3GPP access. If the UE at a later point in time, registers to another 3GPP access, the UE will decide to transition to ATSSS, based on feature support indication by the network and the network preference.

#### If the UE, at a later point in time registers to another 3GPP access, the UE will upgrade the established PDU Session to MA PDU session on the registered 3GPP access and initiate the MA PDU session establishment procedures on the other 3GPP access. If the UE registers further to a non-3GPP access, the UE will decide to transition to ATSSS, based on feature support indication by the network and the network preference.

#### 6.1.X.2	Architectural Description

This solution provides the architectural details and the session management procédures for transitioning from using two 3GPP access networks for traffic steering and switching to using a 3GPP access network and a non-3GPP access network for traffic steering, switching and splitting and vice versa. It leverages the multi-access (MA) PDU session, currently defined for ATSSS, as a PDU session that provides a PDU connectivity service, which can use one access network at a time, or simultaneously one 3GPP access network and one non-3GPP access network. The MA PDU session is extended to a PDU session that provides a PDU connectivity service, which can use either one access network at a time, or simultaneously one 3GPP access network and one non-3GPP access network or simultaneously two 3GPP access networks.

Figures 6.1.X.2-1 – 6.1.X.2.4 provide the high-level architecture description of this solution.

AMF

H-SMF

PCF

UDM

H-UPF

Data Network

3GPP Access

N11

N7

N3

N2

N1

N8

N4

N10

N6

V-SMF

V-UPF

AMF

N11

3GPP Access

3GPP

ATSSS and DualSteer UE

N4

N3

N2

Uu

Uu

N1

VPLMN

HPLMN

N8

N16

N9

Non-3GPP Access

N3IWF / TNGF / W-AGF

N3

N2

N1

The H-UPF can be a PSA UPF or the H-UPF may route to a PSA UPF

Figure 6.1.X.2-1: Non-3GPP access in same PLMN as the HPLMN

N2

AMF

H-SMF

PCF

UDM

H-UPF

The H-UPF can be a PSA UPF or the H-UPF may route to a PSA UPF

Data Network

3GPP Access

N11

N7

N3

N2

N1

N8

N4

N10

N6

V-SMF

V-UPF

AMF

N11

3GPP Access

N4

N3

N2

Uu

Uu

N1

VPLMN

HPLMN

N8

N16

N9

Non-3GPP Access

N3IWF / TNGF / W-AGF

N3

N1

3GPP

ATSSS and DualSteer UE

Figure 6.1.X.2-2: Non-3GPP access in same PLMN as the VPLMN

AMF

H-SMF

PCF

UDM

H-UPF

The H-UPF can be a PSA UPF or the H-UPF may route to a PSA UPF

Data Network

3GPP Access

N11

N7

N3

N2

N1

N8

N4

N10

N6

V-SMF

V-UPF

AMF

N11

3GPP Access

N4

N3

N2

Uu

Uu

N1

VPLMN 1

HPLMN

N8

N16

N9

Non-3GPP Access

N3IWF / TNGF / W-AGF

V-SMF

AMF

N11

V-UPF

N9

N16

N8

N3

N4

VPLMN 2

3GPP

ATSSS and DualSteer UE

N2

N1

Figure 6.1.X.2-3: Non-3GPP access in different VPLMN

The H-UPF can be a PSA UPF or the H-UPF may route to a PSA UPF

V-SMF

V-UPF

AMF

N11

3GPP Access

N4

N3

N2

Uu

N1

VPLMN 1

HPLMN

3GPP

ATSSS and DualSteer UE

Non-3GPP Access

N3IWF / TNGF / W-AGF

H-SMF

AMF

N11

H-UPF

N3

N4

N2

N1

PCF

N7

UDM

N8

N10

N6

Data Network

V-SMF

V-UPF

AMF

N11

3GPP Access

N4

N3

N2

Uu

N1

VPLMN 2

N16

N16

N8

N8

N9

N9

Figure 6.1.X.2-4: Only Non-3GPP access in HPLMN

#### 6.1.X.3	Procedures

6.1.X.3.1 Switching from ATSSS to DualSteer

The procedure, detailed in Figure 6.1.X.3.1-1, applies to the transition from ATSSS (traffic steering, switching, and splitting) to DualSteer (traffic steering and traffic switching). The assumption is the UE and network support both ATSSS and DualSteer features.

The procedure is defined as an example using Untrusted non-3GPP access with N3IWF in non-roaming or roaming with Local Breakout architecture as shown in Figure 6.1.X.2-1. When using trusted non-3GPP access with TNGF the procedures will be as defined in TS 23.502 clause 4.12.a7. When using Wireline non-3GPP access with 5G-RG and W-AGF the procedures will be as defined in TS 23.316 clause 7.3.3 and when using Wireline non-3GPP access with FN-RG, the procedures will be as defined in TS 23.316 clause 7.3.7.

0a. The UE is assumed to have registered with a 3GPP access and a non-3GPP access (in the HPLMN) with one or more MA PDU Sessions (based on the ATSSS feature) established for the UE using the procedure described in TS 23.502 clause 4.22.2.1. During the registration procedures over the 3GPP and non-3GPP access, the network will indicate support for DualSteer and/or ATSSS feature in the 5GS network feature support IE within the Registration Accept message along with the preferred feature if both features are supported by the network. The assumption is that the network indicates support for both DualSteer and ATSSS feature with DualSteer being the preferred feature.

0b. The UE registers with another 3GPP access (in different PLMN). During the registration over the 3GPP access in VPLMN, the network indicates the support for DualSteer and/or ATSSS feature in the 5GS network feature support IE within the Registration Accept message. The assumption here is that the network indicates support for DualSteer.

Steps 1-11 are for UE initiated the MA PDU Session Release to release the user plane resources on the non-3GPP access following the procedures defined in TS 23.502 clause 4.12.7 (step 2 to step 12) for PDU Session Release via untrusted non-3GPP access with differences and clarifications specified in TS 23.502 clause 4.22.10.2 for MA PDU Session Release.

NOTE: If any traffic is being split across both 3GPP and non-3GPP access, after the user plane resources on the non-3GPP access are released, the UE will steer the SDFs (previously switched across 3GPP and non-3GPP access) only on the available 3GPP access.

Steps 12-22 are for UE-requested PDU session modification procedure on the HPLMN to update the existing MA PDU Session from ATSSS to DualSteer based on the procedures defined in TS 23.502 clause 4.3.3.2 with following modifications:

- The UE includes the PDU Session ID and MA PDU session request within the PDU Session Modification Request message indicating the support for DualSteer feature using the DualSteer Indication and DualSteer supported steering functionality/modes using the DualSteer Capability IEs to the AMF in step 12.

- The SM context is updated with DualSteer indication for MA PDU Session in Step 14.

- The N4 session is modified with DualSteer rules in step 17.

- The DualSteer rules from the SMF are sent to the UE via AMF using the Namf_Communication_N1N2MessageTransfer and AN-specific resource modification messages.

Step 23-48 are for UE-requested PDU session establishment procedure on the VPLMN and are as defined in TS 23.502 clause 4.3.2.2.2 (steps 22 to 45) with the following modifications:

- The UE includes the PDU Session ID and MA PDU session request within the PDU Session Establishment Request message indicating the support for DualSteer feature using the DualSteer Indication and DualSteer supported steering functionality/modes using the DualSteer Capability IEs to the AMF in step 23.

- Based on the indication in Step 7, the AMF in the VPLMN selects the same DualSteer capable H-SMF and forwards the MA PDU Session Request with the DualSteer Indication within the CreateSMcontext Request message to the H-SMF in step 28 (via V-SMF in step 25a).

- The H-SMF performs an SM Policy Association Establishment procedure to establish an SM Policy Association with the PCF in the home network and gets the default PCC Rules for the PDU Session. This association considers the DualSteer feature support and any associated polices in Steps 31b.

- The H-SMF includes the PDU Session ID and the DualSteer response (indicating the support for DualSteer feature and the configured DualSteer rules) to the AMF in step 38.

- The N2 PDU Session Request in step 39, and the RRC Reconfiguration message in step 40 contain DualSteer Container IE for the DualSteer UE from the SMF containing information elements associated with DualSteer.

- In step 37a/37b, UL N9 tunnel CN info is allocated by the H-SMF or H-UPF. After this, N9 tunnels between the H-UPF and V-UPF are established.

- The H-SMF selects the same DualSteer capable H-UPF sends the DualSteer rules to the H-UPF for establishing the user plane resources in N4 session Establishment message in steps 34a.

Figure 6.1.X.3.1-1: Session Management Procedures for Switching from ATSSS to DualSteer

6.1.X.3.2 Switching from DualSteer to ATSSS

The procedure shown in Figure 6.1.X.3.2-1 applies to the transition from DualSteer (traffic steering and traffic switching) to ATSSS (traffic steering, switching, and splitting). The assumption is that both the UE and the network support both ATSSS and DualSteer features.

The procedure is defined as an example using Untrusted non-3GPP access with N3IWF.When using trusted non-3GPP access, the procedure follows the procedures defined in TS 23.502 clause 4.12.5-1 with modifications captured in TS 23.502 clause 4.12a.5-1 and differences and clarifications captured in TS 23.502 clause 4.22.2 for UE requested MA PDU Session Establishment. When using Wireline non-3GPP access with 5G-RG and W-AGF, step 12 executes after the completion of the procedures defined in TS 23.502 clause 4.3.2.2.1 with changes captured in TS 23.316 clause 7.3.1 and when using Wireline non-3GPP access with FN-RG, step 12 will follow procedures defined in TS 23.502 clause 4.3.2.2.1 with changes captured in TS 23.316 clause 7.3.4. 
Similar procedures are followed for Home-routed architecture when the UE is registered to the same or different VPLMN on the non-3GPP access.

0a. The UE is assumed to have registered to two 3GPP accesses (in the different PLMNs) with one or more MA PDU Sessions established for the UE based on the procedures specified in TS 23.502 clause 4.3.2.2.1 (for HPLMN) and 4.3.2.2.2 (for VPLMN) with modification captured in Figure 6.1.X.3.1-1. During the registration procedures over the 3GPP accesses, the network indicates support for DualSteer and/or ATSSS feature in the 5GS network feature support IE within the Registration Accept message along with the preferred feature if both features are supported by the network. The assumption here is that the network indicates support for both DualSteer and ATSSS feature with ATSSS being the preferred feature.

0b. The UE registers with a non-3GPP access (in HPLMN). During the registration over the non-3GPP access, the network indicates the support ATSSS feature in the 5GS network feature support IE within the Registration Accept message. The assumption here is that the network indicates support for ATSSS.

Step 1 – The UE sends a MA PDU Session Release Request on the VPLMN with the DualSteer Indication and SUPI.

Step 2 to step 17 will follow the steps 1a to 16b as defined in TS 23.502 clause 4.3.4.3 for UE or network requested PDU Session Release for home-routed roaming.

After the MA PDU Session Release on VPLMN, the existing MA PDU Session on the HPLMN used for DualSteer needs to be modified for ATSSS. Step 18 to step 28 will follow the PDU Session Modification Procedures to update the existing MA PDU Session from DualSteer to ATSSS based on the procedures defined in TS 23.502 clause 4.3.3.2 with following modifications:

- The UE includes the PDU Session ID and MA PDU session request within the PDU Session Modification Request message indicating the support for ATSSS feature using the ATSSS Indication and ATSSS supported steering functionality/modes using the ATSSS Capability IEs to the AMF in step 18.

- The SM context is updated with ATSSS indication for MA PDU Session in Step 20.

- The N4 session is modified with ATSSS rules in step 23.

- The ATSSS rules from the SMF will be sent to the UE via AMF using the Namf_Communication_N1N2MessageTransfer and AN-specific resource modification messages.

Step 29 to step 36 will follow step 1 to step 8b as defined in TS 23.502 clause 4.12.5 for PDU session establishment via untrusted non-3GPP access with differences and clarifications captured in TS 23.502 clause 4.22.2 for UE requested MA PDU Session Establishment. The UE initiates the PDU Session Establishment procedures using the same PDU Session ID for the established PDU Session on HPLMN. After the user plane resources are successfully established on the non-3GPP access, the UE assumes to have successfully transitioned from DualSteer to ATSSS.

Figure 6.1.X.3.2-1: Session Management Procedures for Switching from DualSteer to ATSSS

#### 6.1.X.4	Impacts on services, entities and interfaces

AMF:

-	Supports DualSteer capable SMF selection that can support session management across both 3GPP networks.

SMF:

-	Supports DualSteer capable PSA UPF selection that can support session management across both 3GPP accesses.

-	Indicates DualSteer feature support and sends the DualSteer container IE to the UE via the AMF including the DualSteer parameters.

- 	Maps the PCC rules into DualSteer rules which are sent further to the UE via the AMF.

PCF:

- Creates PCC rules based on the specified DualSteer steering functionality.

UE:

-	Indicates DualSteer feature support as part of the MA PDU request to the SMF via the AMF.

*** End of 1st change ***