3GPP TSG-WG SA2 Meeting #162	S2-2405590

# Changsha, China, April 15- April 19, 2024

Source: 	China Mobile

Title: 	Conclusion on KI #2: Enhancement of EAS and local UPF (re)selection

Document for: 	Approval

Agenda Item: 	19.9

Work Item / Release:	FS_eEDGE_5GC_ph3 / Rel-19

Abstract of the contribution: Propose an evaluation on KI #2: Enhancement of EAS and local UPF (re)selection.

1	Discussion

This paper provides evaluation of KI #2: Enhancement of EAS and local UPF (re)selection.

2	Proposal

It is proposed to include the below changes into TR 23.700-49 v0.2.0.

FIRST CHANGE (all new text)

8.x	Conclusion for KI #2: Enhancement of EAS and local UPF (re)selection

The following principles are recommended in normative work for KI #2: Enhancement of EAS and local UPF (re)selection

-	SMF selects local PSA UPF considering N6 delay, when available.

Editor’s NOTE: Whether SMF collects N6 delay measurements from the L-PSA UPF or from the AF is FFS

Editor’s NOTE: Whether EAS load can also be used by the SMF/EASDF is FFS.

-	N6 delay between L-PSA UPF and EAS is measured by leveraging existing mechanisms (e.g., defined by IETF, PING, TWAMP, OWAMP, etc.)

-	Interaction between AF and 5GC may be needed to enable the measurement

Editor’s NOTE: Details of such interaction are FFS.

Editor’s NOTE: Whether the NWDAF should be involved is FFS.

End of CHANGES