3GPP TSG-SA2 Meeting #162	S2-2405797

# 15 – 19 April, 2024, Changsha, China     				 (revision of S2-2405533)

Title:	LS reply on LCS user plane connection binding to the UE

Response to:	S2-2403871 (C1-241722) LS on LCS user plane connection binding to the UE

Release:	Rel-18

Work Item:	5G_eLCS_Ph3

Source:	SA2

To:	CT1

Cc:	CT4, RAN2

Contact Person:

Name:	Yunjing Hou

E-mail Address:	houyunjing@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.273 CR 0521.

1. Overall Description:

SA2 thanks CT1 for the liaison in S2-2403871/C1-241722 concerning the establishment and usage of a secure user plane connection between a UE and LMF for the transfer of LPP and certain LCS supplementary service messages.

SA2 notes the following issues cited by CT1.

1.	After the TLS connection is established, the UE and the LMF initiate LPP or LCS-SS procedure. For the LMF-initiated case, given that multiple UEs are served by the LMF and multiple TLS connections are active, the LMF needs to determine a TLS connection for the target UE. However, it has not been specified how to link the association between the TLS connection and the UE, e.g., how the LMF knows which TLS connection is for the UE who has received the UPP-CM command message. If the TLS connection can be linked to the ‘wrong UE’ then there could be a risk of location related data from one UE which is sent to another UE.

2.	Once the binding of the TLS connection to the UE is done, if there is a new LCS service request (e.g., MT-LR) for the same UE, the AMF invokes Nlmf_location_determinelocation service operation to the LMF. However, it has not been specified how the LMF associates the LCS service request to the TLS connection of the UE. In this case, the LMF cannot determine which TLS connection can be used for the LCS service request. Therefore, the LMF cannot reuse the TLS connection of this UE for subsequent LCS service request(s).

SA2 has discussed above issues and agreed resolutions as bellow:

Resolution of Issue 1:	The LMF allocates an ID (e.g. LCS UP connection ID), to be used to associate with the LCS UP connection (and configuration for the connection to be established) for the UE.

Resolution of Issue 2:	If UE supports user plane positioning capability, the AMF shall include the same UE ID (i.e. SUPI or GPSI, existing parameters) in the subsequent Nlmf_Location_DetermineLocation request, LMF does the correlations of <SUPI or GPSI, LCS UP connection ID >.

SA2 has agreed the attached CR to TS 23.273 to resolve both issues.

2. Actions:

To CT1 group:

ACTION: 	SA2 asks CT1 to take into account the above resolutions and attached CR in defining the location service over user plane in the TS 24.572.

3. Date of Next SA2 Meetings:

TSG-SA2 Meeting #163		27 - 31 May 2024			Korea, KR

TSG-SA2 Meeting #164		19 - 23 August 2024			Maastricht, NL