3GPP TSG-WG SA2 Meeting #162	S2-2404101

Changsha, China, April 15th – 19th, 2024

Source:	Huawei, HiSilicon

Title:	KI#2: Evaluation

Document for:	Approval

Agenda Item:	19.6

Work Item / Release:	FS_VMR_Ph2/ Rel-19

Abstract: Evaluation and Conclusion for KI#2: Authorization of a MWAB and configuration of MWAB.

1. Discussion

This contribution proposes the evaluation for KI#2.

| Key issue | Solutions in TR 23.700-06 V0.2.0 |
| --- | --- |
| #2: Authorization of a MWAB and configuration of MWAB | Solution #4: MWAB authorization handling Solution #5: Authorization and Change of Authorization of a MWAB and configuration of a MWAB |

The basic concept for authorization for MWAB can be categorized into 3 phases:

- MWAB-UE registration and authorization phase

- MWAB-gNB operation handing

- Registered MWAB-UE authorization status change

  - MWAB-UE authorization status change from allowed to not-allowed

  - MWAB-UE authorization status change from not-allowed to allowed

2. Text Proposal

It is proposed to capture the following changes into TR 23.700-06.

# * * * * First change * * * *

7	Evaluation

Editor's note:	This clause provides the evaluations of the solutions of clause 6.

7.X  Evaluation for KI#2

From all 2 solutions proposed for KI#1, three categories are identified for authorization and configuration of MWAB as described below.

- The details of MWAB-UE registration and authorization phase are described in the Table 7.X-1.

Figure 7.X-1:  Different message types during Registration

Table 7.X-1:  MWAB-UE registration and authorization phase

| Solutions | Up Link message |  |  | Down Link message |  | Authorization checking | Authorization result | AMF behaviour |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RRC message (MWAB UE-> BH gNB) | N2 message (BH gNB-> BH AMF) | NAS message (MWAB UE->BH AMF) | NAS message (BH AMF->MWAB UE) | N2 message (BH AMF-> BH gNB) |  |  |  |
| Solution #4 | MWAB Indication | MWAB Indication | None | MWAB authorization status | MWAB authorization status | Based on subscription data | Registration accept if it is authorized | The AMF is selected by BH gNB based on MWAB Indication. The selected AMF checks MWAB’s subscription data and authorizes the MWAB |
| Solution #5 | None | None | MWAB Indication in UE MM Core network capability | MWAB operation information, which includes a) MWAB-Authorized. Additionally, the area and time the UE is authorized optionally for per PLMN; or b) MWAB-Unauthorized, additionally the area and time the UE is not authorized optionally per PLMN for. | None | Based on subscription data | Registration accept if it is authorized Registration reject if it is not authorized | If the AMF selected by the BG BH gNB doesn’t support MWAB feature, the AMF relocation is triggered. The new AMF checks MWAB’s subscription data and authorizes the MWAB |

- The details of MWAB-gNB operation handling are described in the Table 7.X-2.

Table 7.X-2:  MWAB-gNB operation handling

| Solutions | High-level Descriptions |
| --- | --- |
| Solution #4 | The MWAB-gNB connects to the AMF via the backhaul IP connectivity provided by the MWAB-UE. The MWAB-gNB initiates the service towards UE |
| Solution #5 | The MWAB-UE provides to the MWAB gNB the IP address it can use to contact the OAM server. The MWAB-gNB contact the OAM server and obtains configuration |

- Registered MWAB-UE authorization status change

  - MWAB-UE authorization status change from allowed to not-allowed

| Solutions | How to notify the MWAB-UE of authorization status change? | MWAB-UE authorization status change from allowed to not-allowed |
| --- | --- | --- |
| Solution #4 | UE Configuration Update | MWAB-gNB triggers the move of connected UEs to other cells MWAB-gNB may remove the TNLA and N GAP connection towards the AMF MWAB-UE may release the BH PDU Session(s) MWAB-UE may deregister from the AMF |
| Solution #5 | UE Configuration Update | MWAB-gNB hands over to other gNBs the UEs it serves. MWAB-gNB stops serving any UE and informs OAM it is out of service. The MWAB-gNB also releases all the NG connections to the AMFs if is connected to over the B/H PDU sessions MWAB-UE releases all the b/h PDU sessions |

  - MWAB-UE authorization status change from not-allowed to allowed

| Solutions | How to notify the MWAB-UE of authorization status change? | MWAB-UE authorization status change from NOT allowed to allowed |
| --- | --- | --- |
| Solution #4 | UE Configuration Update | Same as the MWAB-gNB operation initiation |
| Solution #5 | UE Configuration Update | Same as the MWAB-gNB operation initiation |

# * * * * End of changes * * * *