3GPP SA WG2 Meeting #162	S2-2404918

Changsha, China, April 15th- 19th, 2024

Source:	Huawei, HiSilicon

Title:	KI#1 New Sol: Multiple USS support during the UAV flight path

Document for:	Approval

Agenda Item:	19.10

Work Item / Release:	FS_ UAS_Ph3 / Rel-19

Abstract of the contribution: This paper proposes a solution to support multiple USS during the UAV flight path for KI#1 of FS_UAS_Ph3.

Discussion

The aim of the key issue #1 is to study whether and how to enhance NEF services to support service exposure and interactions between MNOs and UTM functions for supporting: pre-mission flight planning and in-mission flight monitoring for UAVs, C2 communication reliability and the scenario of multiple USS serving the geographical areas corresponding to the UAV flight path. This paper focuses on the scenario of multiple USS serving the geographical areas corresponding to the UAV flight path. and proposes a solution for the referred part.

The serving USS of UAV may change when the UAV flight path cross the geographical areas served by different USS.

The target USS will take control of the mobile UAV in replacement with the source USS. With the process of USS relocation, here come several problems to be solved.

Q1: How to determine the USS change/relocation?

The USS change is the application server change so the problem can reuse or refer to the EAS relocation solution as described in TS 23.558. Each EAS server has a EAS Geographical Service Area and can requests UE location information to EES. The EES is authorized to use Nnef Event Exposure API for UE location based on service level agreement with MNO. When the S-EAS detects the UE location out of the S-EAS Geographical Service Area, it discovers the T-EAS and determines the T-EAS to use.

Back to the mentioned USS change problem, we can propose that when the serving USS changes due to the UAV mobility, the S-USS can detect the UAV out of the serving geographical areas and determines the T-USS.

Proposal 1: The S-USS can detect the UAV out of the serving geographical areas and determine the T-USS.

Q2: What will be the impacts due to USS change?

When the UAV moves from the S-USS to the T-USS, the S-USS may initiate UUAA revocation and transfer the UAV context to T-USS. Then T-USS use UUAA Re-authorization procedures. Most of the mentioned function can reuse the existing procedure.

As defined in TS 23.256, a UAV is assigned a CAA-level UAV Identity by functions in the aviation domain (e.g. USS) and this assigned identity is used for Remote Identification and Tracking and to identify the UAV. When T-USS execute UUAA Re-authorization procedures, the T-USS can not use the old CAA-level UAV Identity in the Naf_Authentication_Notification request to UAS NF. It should indicate the USS change in the request and trigger UVA to report the new CAA-level UAV Identity. To support the current CAA-level UAV Identity report corresponding to the serving USS (e.g., first report in the UUAA registration procedure), the UAV should be configured with the mapping information between the CAA-level UAV ID and the USS information.

Proposal 2: The T-USS should indicate the USS change and trigger UAV to report the new CAA-level UAV Identity for UUAA re-authorization.

Proposal 3: The UAV should be configured with the mapping information between the CAA-level UAV ID and the USS information for the current CAA-level UAV Identity report.

Proposal

A solution is proposed for KI#1 in the UAS Ph3 TR 23.700-59.

*** Start of first changes ***

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  |  |
| --- | --- | --- | --- |
|  | <Key Issue #1> | <Key Issue #2> | <Key Issue #3> |
| #1 | X |  |  |
| #2 | X |  |  |
| #3 | X |  |  |
| #4 |  | X |  |
| #5 |  | X |  |
| #6 |  | X |  |
| #7 |  |  | X |
| #8 |  |  | X |
| #9 |  |  | X |
| #x | X |  |  |

*** Start of Second changes ***

6.X	Solution #X: Multiple USS support during the UAV flight path

6.X.1	Key Issue mapping

This solution addresses Key Issues #1 which focuses on the scenario of multiple USS serving the geographical areas corresponding to the UAV flight path.

The USS change is to consider the two potential problems: how to detect the USS change and what will be the impacts due to USS change.

6.X.2	Description

The serving USS of UAV may change when the UAV flight path cross the geographical areas served by different USS.

The target USS will take control of the mobile UAV in replacement with the source USS. With the process of USS relocation, how to detect the USS change and what will be the impacts due to USS change need to be discussed in the solution.

The USS change is the application server change so the problem can reuse or refer to the EAS relocation solution as described in TS 23.558. Each EAS server has a EAS Geographical Service Area and can requests UE location information to EES. The EES is authorized to use Nnef Event Exposure API for UE location based on service level agreement with MNO. When the S-EAS detects the UE location out of the S-EAS Geographical Service Area, it discovers the T-EAS and determines the T-EAS to use.

Back to the mentioned USS change problem, we can propose that when the serving USS changes due to the UAV mobility, the S-USS can detect the UAV out of the serving geographical areas and determines the T-USS. The part is not defined in SA2.

Proposal 1: The S-USS can detect the UAV out of the serving geographical areas and determine the T-USS.

When the UAV moves from the S-USS to the T-USS, the S-USS may initiate UUAA revocation and transfer the UAV context to T-USS. Then T-USS use UUAA Re-authorization procedures. Most of the mentioned function can reuse the existing defined procedure.

As described in TS 23.256, a UAV is assigned a CAA-level UAV Identity by functions in the aviation domain (e.g. USS) and this assigned identity is used for Remote Identification and Tracking and to identify the UAV. When T-USS executes UUAA Re-authorization procedures, the T-USS cannot use the old CAA-level UAV Identity in the Naf_Authentication_Notification request to UAS NF because the old CAA-level UAV Identity is assigned by the S-USS. The T-USS should indicate the USS change in the request and trigger UVA to report the new CAA-level UAV Identity for UUAA re-authorization. To support the current CAA-level UAV Identity report corresponding to the serving USS (e.g., first report in the UUAA registration procedure), the UAV should be configured with the mapping information between the CAA-level UAV ID and the USS information and send the corresponding CAA-level UAV Identity to the target USS for UUAA re-authorization.

Proposal 2: The T-USS should indicate the USS change and trigger UAV to report the new CAA-level UAV Identity for UUAA re-authorization.

Proposal 3: The UAV should be configured with the mapping information between the CAA-level UAV ID and the USS information for the current CAA-level UAV Identity report.

6.X.3	Procedures

Figure 6.X.3-1: Procedure for USS change

- The UAV is configured with the mapping information between the CAA-level UAV ID and the USS information for the current CAA-level UAV Identity report.

- The UAV performs Authentication and Authorization procedures. Details can be seen in TS 23.256 clause 5.2.2/5.2.3.

2.	The S-USS detects the UAV out of the serving geographical areas and determines the T-USS.

3.	The S-USS may execute the UUAA revocation procedure by USS/UTM in TS 23.256 clause 5.2.7.

4.	The S-USS transfers the UAV context to the T-USS.

5.	The T-USS sends a Naf_Authentication_Notification request to UAS NF for re-authentication of the UAV. The USS includes GPSI, CAA-Level UAV ID report, if available in the request.

6.  UAS NF retrieves the stored UUAA context for the UE. From the stored UUAA context the UAS NF determines the target AMF or SMF for sending the notification.

7.  The UAS NF sends Nnef_Authentication_Notification request to notify the target NF, i.e. either the AMF or the SMF, to initiate CAA-Level UAV ID report of the UAV.

8.  The UAS NF responds back to the T-USS indicating that re-authentication request has been successfully initiated.

9.  Either the AMF or the SMF triggers the UAV to report the current CAA-level UAV Identity.

10. The AMF or the SMF receives the current CAA-level UAV Identity and determines to invoke UAS NF/NEF service operation for UUAA Authentication/Authorization.

6.X.4	Impacts on services, entities and interfaces

AF (USS/UTM):

-	Indicate the USS change and trigger UAV to report the new CAA-level UAV Identity for UUAA re-authorization.

UE(UAV):

-	configured with the mapping information between the CAA-level UAV ID and the USS information

-	support to report the current CAA-level UAV Identity

*** END of changes ***