SA WG2 Meeting #163	S2-2407287r01

Jeju, Korea, May 27 – May 31, 2024

# Source:	Apple (Primary Rapporteur)

# Title:	New WID on Multi-Access (DualSteer and ATSSS_Ph4)

# Document for:	Approval

Agenda Item:	30.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: 	Multi-Access (DualSteer and ATSSS_Ph4)

Acronym:	MASSS

Unique identifier:	TBD

Potential target Release:	Rel-19

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  | X |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
|  |  |  | N/A |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 940070 | Access Traffic Steering, Switch and Splitting support in the 5G system architecture; Phase 3 | Rel-18 Work Item |
| 960018 | Study on upper layer traffic steering, switching and split over dual 3GPP access | Stage 1 study for DualSteer in Rel-19 |
| 1020031 | Upper layer traffic steering and switching over dual 3GPP access | Stage 1 work item for DualSteer in Rel-19 |
| 1020070 | Study on Multi-Access (DualSteer and ATSSS_Ph4) | Stage 2 study for MASSS in Rel-19 |

3	Justification

1)	In Rel-18, a high-layer steering functionality "MPQUIC steering functionality using UDP proxying over HTTP" was defined that enables steering, switching, and splitting UDP traffic based on IETF protocols. For TCP traffic, ATSSS has been relying on the use of the "MPTCP steering functionality" that was specified in Rel-16. The associated proxy functionalities (MPQUIC and MPTCP) add complexity for the operator deployment. In order to ease this deployment burden, it would be beneficial to specify how to enable the MPQUIC steering functionality to also steer, switch, and split non-UDP traffic (TCP, IP, Ethernet traffic) and at the same time make the MPTCP steering functionality optional for TCP traffic in ATSSS.

4	Objective

The objective of this work items is to specify the architecture enhancements, functionalities and procedures to support MASSS features based on conclusions of TR 23.700-54 (clause 8).

Specifically, the detailed objectives include:

1)	New functionalities and procedures to support MPQUIC steering functionalities in order to steer, switch, and split non-UDP traffic.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.501 | System architecture to support MASSS features. | TSG#106 (Dec. 2024) |  |
| 23.502 | Procedures to support MASSS features. | TSG#106 (Dec. 2024) |  |
| 23.503 | Policy control to support MASSS features. | TSG#106 (Dec. 2024) |  |

6	Work item Rapporteur(s)

Primary Rapporteur: Krisztian Kiss (Apple), krisztian@apple.com

Secondary Rapporteur: Zhuoyi Chen (China Telecom), chenzy34@chinatelecom.cn

(in charge of producing reports to the WG plenary on progress)

7	Work item leadership

SA2

8	Aspects that involve other WGs

The following aspects involving other WGs may arise related to this SID:

9	Supporting Individual Members

Supporting IM name

Airbus?

Apple

Avanti?

BT?

CableLabs

CATT?

CeWIT?

Charter Communications, Inc

China Telecom?

Cisco?

Comcast?

Dish Network?

Ericsson?

ESA?

ETRI?

Eutelsat Group?

Futurewei?

Gatehouse?

Intel?

KDDI?

Interdigital?

IRT Saint Exupery?

Lenovo?

LG Electronics

LG Uplus?

Ligado?

MATRIXX Software?

MITRE?

NEC?

Nokia?

Nokia Shanghai Bell?

Novamint?

Oppo?

Qualcomm?

Samsung?

Sateliot?

SES?

SHARP

SKY Perfect JSAT Corporation?

SyncTechno Inc.?

Thales?

US Cellular?

vivo?

ZTE?