SA WG2 Meeting #163	S2-2406348

May 20 - 31, 2024 Jeju, South Korea	(Revision of S2-240xxxx)

Source: 	China Mobile, vivo

Title: 	KI #6, Conclusion update for KI#6

Document for: 	Approval

Agenda Item: 	19.2

Work Item / Release:	FS_NG_RTC_Ph2 / Rel-19

Abstract of the contribution: This contribution provides update of conclusion for key issue #6.

1	Discussion

This contribution updates the interim conclusions for KI#6.

2 	Proposal

It is proposed to capture the following contents into TR 23.700-77.

FIRST CHANGE

8	Conclusions

8.X	Interim conclusion of KI#6

The following interim conclusions for KI#6 "Support of Standalone IMS Data Channel Sessions" are agreed:

-	Scenarios including establishing IMS session with standalone bootstrap DC, standalone application DC, or a combination of standalone bootstrap DC and application DC shall be supported in this release;

-	An IMS session with only standalone bootstrap DC is only used for downloading application list from DCSF. There is no IMS session with only standalone bootstrap DC to a terminating UE.

-	An IMS session with only application DC is used for a UE to use an already downloaded application towards application server or peer UE who have the same application. If the peer UE has not downloaded the application, the peer UE rejects the session.

-	On top of the above bullets, an IMS session with a combination of standalone bootstrap DC and application DC also allows the peer UE to download the application if not yet downloaded and update the session to use the application in the same session.

-	Changing an IMS session with audio/video/DC media components to a standalone IMS DC sessionand adding audio/video media components to an established standalone IMS DC session using ReINVITE request shall be supported in this release.

NOTE 1:	How the charging system is notified about the changing of the media components is in scope of SA5.

-	The handling of SIP preconditions for standalone DC IMS session will be addressed during normative phase.

-	The binding information is included in the SDP offer as specified in Rel-18.

NOTE 2：SA4 is responsible to provide version compatibility information. The alignment with SA4 can be done in normative phase if required.

-	When the UE initiates a IMS session with standalone bootstrap DC or standalone application DC:

-	The originating UE generally follows existing procedures to establish bootstrap DC or application DC as specified in 23.228 for standalone DC session establishment with the addition that the UE may only include DC media components when generating SDP offer in initial INVITE request;

-	For IMS session with standalone bootstrap DC only for downloading the application list, the DCSF serving the UE instructs IMS AS to terminates the session and reserve only the originating side media resource based on the received DC media components;

-	For IMS session with only standalone application DC, if the terminating UE has not yet downloaded the application, or if the terminating UE does not have a compatible version of application with the originating UE, the terminating UE rejects the session;

-	When the UE initiates an IMS session with combination of standalone bootstrap DC and standalone application DC

-	The originating UE only include DC media components for both bootstrap DC and application DC when generating SDP offer in initial INVITE request;

-	If the terminating UE has downloaded the application, the terminating UE responds with the SDP answers for both bootstrap DC and application DC;

-	If the terminating UE has not downloaded the application, the terminating UE responds with the SDP answer for bootstrap DC and set ports of application DC to zero and downloads the application after the bootstrap DC is established. The terminating UE via the SDP answer informs the originating UE that the terminating UE is downloading the application and will update the session further after downloading the application. After the application is downloaded, the terminating UE updates the session by adding the application DC for the application towards the originating UE and then alerts the user.

NOTE 3:	How the terminating UE informs originating UE of downloading application is determined by SA4.

NOTE4 :	Whether and how the DC media component can be held is determined by SA4.

-	The UE and IMS network negotiate the capability of supporting standalone DC during registration procedure, where the UE indicates its capability with a media feature tag in initial and subsequent REGISTER request and the S-CSCF indicates the capability with a Feature-Caps header field in the 200OK to initial and subsequent REGISTER request;

NOTE 5:	The capability indication of standalone DC can be used by the network and UE to differentiate with normal IMS DC session. The IMS network will only sends the standalone DC request to the UE supporting standalone DC. This can be revisited during normative phase.

-	Additional subscription for standalone data channel is optional. When additional subscription for standalone data channel is required, the subscription information is stored in service data of IMS data channel, based on which the IMS AS determines whether to accept the standalone data channel IMS session.

NOTE 6:	Whether and how to handle the IMS AS which is only used for standalone DC sessions is decided during normative phase.

End of CHANGES