3GPP TSG-WG SA2 Meeting #163	S2-2406403

Jeju, Korea, May 27 – 31, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	KI#9: Initial conclusion of KI#9 on enhancement for XR related network information exposure

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM_Ph2 / Rel-19

Abstract: Initial conclusion of KI#9 on enhancement for XR related network information exposure is proposed in this paper.

# 1. Introduction

In draft TR 23.700-70, there are 3 candidate solutions agreed on enhancement for XR related network information exposure as following:

-	In Sol#19, the Alternative QoS Profiles are extended to include the PDU Set QoS parameters. In case of QNC enabled, the QNC together with the reference to the matching Alternative QoS Profile can be exposed by the AF via the control plane or the user plane.

-	In Sol#33, the available data rate for (non-)GBR QoS Flow can be exposed by the NG-RAN to the AF via the control plane or the user plane.

-	In Sol#34, the PDU Set delay and PDU Set Loss Rate are proposed to be measured and reported to the AF.

Based on the summary above, this paper intends to propose the initial conclusion for KI#9 in this paper.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-70.

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policies and Charging control framework for the 5G System; Stage 2".

[5]	IETF RFC 3711: "The Secure Real-time Transport Protocol (SRTP)", March 2004.

[6]	IETF RFC 6904: "Encryption of Header Extensions in the Secure Real-time Transport Protocol (SRTP)".

[7]	IETF RFC 9335: "Completely Encrypting RTP Header Extensions and Contributing Sources".

[8]	IETF draft-ietf-avtcore-rtp-over-quic: "RTP over QUIC (RoQ)".

[9]	IETF draft-ietf-moq-transport: "Media over QUIC Transport".

[10]	IETF experimental draft-ietf-avtext-framemarking: "Frame Marking RTP Header Extension".

[11]	IETF RFC 9000: "QUIC: A UDP-Based Multiplexed and Secure Transport".

[12]	TR 26.926 v18.1.0: "Traffic Models and Quality Evaluation Methods for Media and XR Services in 5G Systems".

[13]	TR 26.925 v18.1.0: "Typical traffic characteristics of media services on 3GPP networks".

[14]	IETF RFC 9330: "Low Latency, Low Loss, and Scalable Throughput (L4S) Internet Service: Architecture.

[15]	IETF RFC 9331: "The Explicit Congestion Notification (ECN) Protocol for Low Latency, Low Loss, and Scalable Throughput (L4S)".

[16]	IETF draft-ietf-tsvwg-ecn-encap-guidelines-22: "Guidelines for Adding Congestion Notification to Protocols that Encapsulate IP".

[17]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[18]	CableLabs DOCSIS MULPI: "Data-Over-Cable Service Interface Specifications DOCSIS 3.1, MAC and Upper Layer Protocols Interface Specification".

[19]	IETF RFC 9332: " Dual-Queue Coupled Active Queue Management (AQM) for Low Latency, Low Loss, and Scalable Throughput (L4S)".

[20]	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations".

[21]	IETF draft-ietf-tsvwg-udp-options: "Transport options for UDP".

[22]	IETF RFC 6363: "Forward Error Correction (FEC) Framework".

[23]	IETF RFC 6364: "Session Description Protocol Elements for the Forward Error Correction (FEC) Framework".

[24]	IETF RFC 6681: "Raptor Forward Error Correction (FEC) Schemes for FECFRAME".

[25]	IETF RFC 6682: "RTP Payload Format for Raptor Forward Error Correction (FEC) ".

[26]	IETF RFC 6695: "Methods to Convey Forward Error Correction (FEC) Framework Configuration Information".

[27]	IETF RFC 6816: "Simple Low-Density Parity Check (LDPC) Staircase Forward Error Correction (FEC) Scheme for FECFRAME".

[28]	IETF RFC 6865: "Simple Reed-Solomon Forward Error Correction (FEC) Scheme for FECFRAME".

[29]	IETF RFC 8680: "Forward Error Correction (FEC) Framework Extension to Sliding Window Codes".

[30]	IETF RFC 8681: "Sliding Window Random Linear Code (RLC) Forward Erasure Correction (FEC) Schemes for FECFRAME".

[31]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[32]	IETF RFC 8627: "RTP Payload Format for Flexible Forward Error Correction (FEC)".

[33]	IETF RFC 7656: "A Taxonomy of Semantics and Mechanisms for Real-Time Transport Protocol (RTP) Sources".

[34]	S2-2210181: "Reply LS on further details on XR traffic".

[35]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes; Stage 3".

[36]	3GPP TS 38.415: "NG-RAN; PDU Session User Plane Protocol".

[37]	3GPP TS 33.210: "Network Domain Security (NDS); IP network layer security".

[38]	IETF RFC 9298: "Proxying UDP in HTTP".

[39]	IETF RFC 9297: "HTTP Datagrams and the Capsule Protocol".

[40]	IETF draft-ietf-masque-quic-proxy-01: "QUIC-Aware Proxying Using HTTP".

[41]	IETF RFC 5761: "Multiplexing RTP Data and Control Packets on a Single Port".

[42]	IETF RFC 5764:" Datagram Transport Layer Security (DTLS) Extension to Establish Keys for the Secure Real-time Transport Protocol (SRTP)".

[43]	IETF RFC 7983: "Multiplexing Scheme Updates for Secure Real-time Transport Protocol (SRTP) Extension for Datagram Transport Layer Security (DTLS)".

[44]	IETF RFC 8872: "Guidelines for Using the Multiplexing Features of RTP to Support Multiple Media Streams".

[45]	IETF RFC 9443: "Multiplexing Scheme Updates for QUIC".

[46]	IETF RFC 3550: "RTP: A Transport Protocol for Real-Time Applications".

Editor's note:	References [8], [9] and [10] cannot be formally referenced until published as RFC.

[47]	IETF RFC 6040: " Tunnelling of Explicit Congestion Notification".

[X]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

# * * * * Second change, all new texts * * * *

## 8.x	Conclusions for Key Issue #9

The following aspects are concluded as principles for the normative work:

-	For exposure of QNC together with “reference to the Alternative QoS Profile” via the user plane, the framework of User Plane Direct 5GS Information Exposure defined in clause 5.20d of TS 23.501 is reused:

-	Based on the AF input and/or local configuration, the PCF/SMF may indicate the NG-RAN to report the QoS notification together with the “reference to the Alternative QoS Profile” to the UPF via the GTP-U header of UL packets.

-	UPF further exposes the QoS notification together with “reference to the Alternative QoS Profile” to the AF directly or via the local NEF reusing .

-	The NEF/PCF/SMF reuses the “reference to the Alternative QoS Profile” provided by the AF, i.e. the altQoSParamSetRef [X].

-	For exposure of available data rate via control plane or user plane:

-	Based on the AF requests, the PCF instructs SMF to enable the available data rate report and exposure.

-	Following the instruction of SMF,

-	The NG-RAN reports available data rate to the SMF, and the SMF further sends the available data rate to the PCF then AF. or,

-	The NG-RAN reports the available data rate to the UPF via the GTP-U header of UL packets and UPF further exposes the available data rate to the AF directly or via the local NEF.

Editor’s Note: The conclusion may be refined based on the feedback from RAN WGs.

# * * * * End of changes * * * *